3GPP TSG-RAN WG4 Meeting #77	    R4-157331

Anaheim, CA, US, 16 – 20 Nov, 2015

Agenda Item:	7.13.4

Source: 	Samsung

Title: 	RSRP/RSRQ simulation results for LAA

Document for:	Discussion

Introduction

In RAN4 #76bis meeting, RAN4 has discussed the RRM part for WI “Licensed-Assisted Access to Unlicensed Spectrum”, and simulation assumptions for RSRP/RSRQ evaluation for LAA were agreed in [1].

In this contribution, we provide the link level simulation results for RSRP/RSRQ measurement according to the agreed simulation assumptions.

Simulation assumptions

The link level simulation assumptions for RSRP and RSRQ are shown in Table 1. In this contribution, only 1 Transmit antenna is evaluated.

Table 1: Simulation parameters for measurement in LAA

| Parameters | Value | Comments |
| --- | --- | --- |
| Measurement bandwidth | 6, 25,50RBs | RSRP measured over 6, 25,50 RBs |
| System bandwidth | 50 resource blocks |  |
| DMTC (Discovery signal Measurement Timing Configuration) | 160ms |  |
| DRS occasion duration | 12 symbols | The signals comprising the LAA DRS are the same as symbols 0-11 of the Rel-12 DRS for FDD (LAA assumes normal CP) - DRS occasion duration is 12 symbols [1] |
| DRS availability | - | DRS is present in all DRS occasions |
| L1 measurement | i*160 ms, where - i=1 (single shot) - i=2 and i=3 (multi-shot) |  |
| L3 filtering | Disabled |  |
| Transmit antenna | 1 | The number of transmit antennas in DRS and outside DRS is the same. |
| Receive antennas | 2 | The receive diversity rule as defined in TS 36.214. Both antennas with equal gain, no correlation between them. |
| DRX/DTX | OFF | DRX/DTX to be considered at later stage |
| Propagation conditions | Baseline: AWGN, EPA5, ETU30 |  |
| CP Length | Normal |  |
| Frequency band | 5GHz |  |
| Ês/Iot | -6,-5,-4, -3,-2,-1 dB |  |

Smulation results

The absolute RSRP and RSRQ measurement accuracy results are shown in Table 2 and Table 3 for {6RBs, 25RBs, 50RBs} and {i=1, i=2, i=3} combinations.

Table 2: RSRP absolute accuracy results

| Channel | SNR(dB) | 6RBs |  |  | 25RBs |  |  | 50RBs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | i=1 | i=2 | i=3 | i=1 | i=2 | i=3 | i=1 | i=2 | i=3 |
| AWGN | -6 | 3.49 | 2.91 | 2.59 | 1.98 | 1.58 | 1.44 | 1.51 | 1.18 | 1.06 |
|  | -5 | 3.06 | 2.54 | 2.25 | 1.72 | 1.36 | 1.24 | 1.30 | 1.01 | 0.91 |
|  | -4 | 2.71 | 2.22 | 1.96 | 1.49 | 1.18 | 1.07 | 1.12 | 0.87 | 0.78 |
|  | -3 | 2.39 | 1.96 | 1.72 | 1.31 | 1.02 | 0.93 | 0.97 | 0.76 | 0.68 |
|  | -2 | 2.11 | 1.71 | 1.50 | 1.15 | 0.89 | 0.80 | 0.85 | 0.66 | 0.59 |
|  | -1 | 1.87 | 1.50 | 1.33 | 1.00 | 0.78 | 0.70 | 0.74 | 0.58 | 0.51 |
| EPA5 | -6 | 4.29 | 3.24 | 2.74 | 1.98 | 1.45 | 1.23 | 1.31 | 0.95 | 0.82 |
|  | -5 | 3.59 | 2.74 | 2.33 | 1.64 | 1.19 | 1.00 | 1.09 | 0.79 | 0.68 |
|  | -4 | 3.05 | 2.30 | 1.95 | 1.38 | 1.01 | 0.84 | 0.93 | 0.67 | 0.57 |
|  | -3 | 2.56 | 1.92 | 1.63 | 1.15 | 0.88 | 0.71 | 0.79 | 0.57 | 0.49 |
|  | -2 | 2.20 | 1.65 | 1.38 | 0.98 | 0.74 | 0.60 | 0.67 | 0.49 | 0.41 |
|  | -1 | 1.91 | 1.40 | 1.18 | 0.84 | 0.64 | 0.52 | 0.58 | 0.42 | 0.35 |
| ETU30 | -6 | 3.26 | 2.44 | 2.08 | 2.01 | 1.50 | 1.25 | 1.68 | 1.31 | 1.13 |
|  | -5 | 2.79 | 2.00 | 1.69 | 1.84 | 1.41 | 1.19 | 1.56 | 1.21 | 1.07 |
|  | -4 | 2.68 | 1.88 | 1.55 | 1.71 | 1.31 | 1.13 | 1.45 | 1.14 | 1.02 |
|  | -3 | 2.54 | 1.82 | 1.55 | 1.60 | 1.22 | 1.06 | 1.37 | 1.08 | 0.96 |
|  | -2 | 2.40 | 1.74 | 1.49 | 1.50 | 1.15 | 1.00 | 1.30 | 1.03 | 0.92 |
|  | -1 | 2.24 | 1.64 | 1.42 | 1.41 | 1.08 | 0.94 | 1.24 | 1.00 | 0.89 |

Table 3: RSRQ absolute accuracy results

| Channel | SNR(dB) | 6RBs |  |  | 25RBs |  |  | 50RBs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | i=1 | i=2 | i=3 | i=1 | i=2 | i=3 | i=1 | i=2 | i=3 |
| AWGN | -6 | 3.53 | 2.92 | 2.63 | 2.02 | 1.63 | 1.48 | 1.56 | 1.24 | 1.11 |
|  | -5 | 3.12 | 2.56 | 2.30 | 1.76 | 1.42 | 1.30 | 1.36 | 1.09 | 0.97 |
|  | -4 | 2.77 | 2.25 | 2.03 | 1.55 | 1.25 | 1.14 | 1.21 | 0.96 | 0.87 |
|  | -3 | 2.47 | 2.00 | 1.80 | 1.38 | 1.12 | 1.02 | 1.08 | 0.87 | 0.78 |
|  | -2 | 2.22 | 1.78 | 1.60 | 1.24 | 1.01 | 0.91 | 0.97 | 0.79 | 0.71 |
|  | -1 | 2.00 | 1.61 | 1.44 | 1.12 | 0.92 | 0.83 | 0.88 | 0.72 | 0.66 |
| EPA5 | -6 | 4.30 | 3.34 | 2.87 | 2.04 | 1.54 | 1.33 | 1.36 | 1.04 | 0.89 |
|  | -5 | 3.67 | 2.85 | 2.46 | 1.71 | 1.28 | 1.12 | 1.17 | 0.90 | 0.78 |
|  | -4 | 3.05 | 2.44 | 2.10 | 1.46 | 1.12 | 0.97 | 1.01 | 0.78 | 0.69 |
|  | -3 | 2.57 | 2.07 | 1.81 | 1.27 | 0.99 | 0.84 | 0.90 | 0.70 | 0.62 |
|  | -2 | 2.26 | 1.79 | 1.56 | 1.11 | 0.88 | 0.76 | 0.82 | 0.64 | 0.57 |
|  | -1 | 1.95 | 1.57 | 1.36 | 1.00 | 0.80 | 0.69 | 0.74 | 1.18 | 0.53 |
| ETU30 | -6 | 3.33 | 2.54 | 2.21 | 1.83 | 1.35 | 1.11 | 1.55 | 1.07 | 1.01 |
|  | -5 | 2.78 | 2.10 | 1.83 | 1.68 | 1.25 | 1.03 | 1.41 | 0.99 | 0.93 |
|  | -4 | 2.34 | 1.76 | 1.52 | 1.52 | 1.14 | 0.94 | 1.29 | 0.90 | 0.85 |
|  | -3 | 2.23 | 1.52 | 1.26 | 1.37 | 1.03 | 0.85 | 1.20 | 0.82 | 0.78 |
|  | -2 | 2.10 | 1.42 | 1.19 | 1.26 | 0.94 | 0.78 | 1.11 | 0.76 | 0.72 |
|  | -1 | 1.89 | 1.30 | 1.10 | 1.16 | 0.86 | 0.70 | 1.02 | 0.86 | 0.65 |

The RSRP accuracy results are further summarized as below table, which can be easily to see whether the RSRP absolute accuracy can be met the legacy requirement or not.

Table 4: Summarized RSRP results

| SNR(dB) | 6RBs |  |  | 25RBs |  |  | 50RBs |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | i=1 | i=2 | i=3 | i=1 | i=2 | i=3 | i=1 | i=2 | i=3 |
| -6 | No | No | No | Yes | Yes | Yes | Yes | Yes | Yes |
| -5 | No | No | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| -4 | No | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| -3 | No | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| -2 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| -1 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

Following observations are made based on the simulation results:

- The RSRP and RSRQ accuracy can be improved by enlarging the measurement bandwidth and/or extending L1 measurement period, i.e. increasing shot number.

- For measurement bandwidth ≥ 25RBs, even when only one shot is used, the RSRP absolute accuracy can be met the legacy requirement for SNR = -6dB side condition, i.e. ±2.5dB without RF margin.

- For measurement bandwidth = 6RBs, even when 3 shots are used, the RSRP absolute accuracy can not be met the legacy requirement.

- For measurement bandwidth = 6RBs, when only one shot is used, the RSRP absolute accuracy under SINR ≥ -2dB can be met the legacy requirement.

The above observations are summarized as below:

Observation 1: RSRP and RSRQ accuracy can be improved by enlarging the measurement bandwidth and/or extending L1 measurement period, i.e. increasing shot number.

Observation 2: For measurement bandwidth ≥ 25RBs, the RSRP absolute accuracy can be met the legacy requirement for SNR ≥ -6dB and i ≥ 1.

Observation 3: For measurement bandwidth = 6RBs, the RSRP absolute accuracy can be met the legacy requirement for SNR ≥ -2dB and i ≥ 1.

Conclusion

In this contribution, we provide the link level simulation results for RSRP and RSRQ measurement accuracy for LAA. Based on the simulation results, below observations are summarized.

Observation 1: RSRP and RSRQ accuracy can be improved by enlarging the measurement bandwidth and/or extending L1 measurement period, i.e. increasing shot number.

Observation 2: For measurement bandwidth ≥ 25RBs, the RSRP absolute accuracy can be met the legacy requirement for SNR ≥ -6dB and i ≥ 1.

Observation 3: For measurement bandwidth = 6RBs, the RSRP absolute accuracy can be met the legacy requirement for SNR ≥ -2dB and i ≥ 1.

Reference

- R4-156645, Simulation assumptions for RSRP/RSRQ evaluation for LAA, Huawei, HiSilicon