3GPP TSG-RAN WG4 Meeting #77                                                                       R4-157372

# Anaheim, CA, US, 16 – 20 Nov., 2015

Source:	LG Electronics

Title:	Simulation results of CSI-RS based RSRP with LAA

Agenda item:	7.13.4

Document for:	Discussion

Introduction

In this paper, we present the simulation results for CSI-RS based RSRP measurement accuracy under LAA with agreed simulation assumptions[1]. Based on the simulation results, we provide our view on CSI-RS based RSRP measurement accuracy.

Simulation assumption

The performance evaluation of CSI-RS based RSRP on unlicensed carrier with LAA is to be based on the link-level simulation assumptions summarized in Table 1 below.

Table 1: Simulation parameters for CSI-RS RSRP measurements on unlicensed carrier

| Parameters | Value | Comments |
| --- | --- | --- |
| SNR | {-4, -3, -2, -1, 0} dB |  |
| Carrier frequency | 5 GHz |  |
| Measurement bandwidth | {6, 25, 50} RBs |  |
| DRS occasion periodicity (M) | 160 | A DRS occasion is transmitted every 160 ms |
| DRS occasion duration (N) | 1 | A DRS occasion for a cell comprises 1 subframe (N=1). |
| DRS availability | DRS is present in all DRS occasions |  |
| CSI-RS transmission | 1, 2, and 3 |  |
| CSI-RS transmit power | The transmit power is the same for all CSI-RS transmissions and it is the same as CRS transmit power. |  |
| CSI-RS reference signal configuration | Config. 2 with 2 REs/RB, Config. 2 with 8 REs/RB | Config. 2 with 4 REs/RB is optional for study |
| L1 measurement period | i*160 ms, where - i=1 (single shot) - i=2 and i=3 (multi-shot) |  |
| L3 filtering | Disabled |  |
| Number of transmit antennas | 1 and 2 |  |
| Number of receive antennas | 2 | The receive diversity rule as defined in TS 36.214. Both antennas with equal gain, no correlation between them. |
| Antenna correlation | Low |  |
| DRX | OFF |  |
| Propagation conditions | AWGN, EPA5, ETU30 |  |
| CP Length | Normal |  |

Note : CSI-RS transmission of 1 is considered for this simulation.

Performance Metrics

The CDF curves are to be provided for measurement errors for absolute CSI-RS based RSRP measurements:

- Delta CSI_RSRP   = (estimated CSI_RSRP – ideal CSI_RSRP) 	[dB]

No impairment margin should be included in the presented results.

Simulation results

Table 3-1, Table3-2 and Table 3-3 show delta CSIRS-RSRP in the measurement bandwidth of 6RB, 25RB  and 50RB  respectively.

# Table 3-1. CSI-RS based RSRP measurement accuracy with measurement bandwidth of 6RB

| Propagation condition | (i*160ms),i | Reference SNR[dB] | AWGN |  |  |  |  | EPA5 |  |  |  |  | ETU30 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  |
| 2RE | 1 | -4 | 2.77 | -0.78 | 5.49 | -3.55 | 2.72 | 1.85 | -2.12 | 7.11 | -3.97 | 5.26 | 1.81 | -2.21 | 5.96 | -4.02 | 4.15 |
|  |  | -3 | 2.22 | -1.11 | 4.85 | -3.33 | 2.62 | 1.49 | -2.26 | 6.32 | -3.75 | 4.83 | 1.37 | -2.37 | 5.23 | -3.73 | 3.87 |
|  |  | -2 | 1.84 | -1.46 | 4.23 | -3.30 | 2.39 | 1.05 | -2.30 | 5.51 | -3.35 | 4.46 | 1.09 | -2.57 | 4.79 | -3.66 | 3.70 |
|  |  | -1 | 1.55 | -1.41 | 3.80 | -2.96 | 2.25 | 0.86 | -2.53 | 4.72 | -3.39 | 3.87 | 0.87 | -2.64 | 4.11 | -3.52 | 3.24 |
|  | 2 | -4 | 2.54 | 0.14 | 4.64 | -2.40 | 2.10 | 1.72 | -1.03 | 4.85 | -2.75 | 3.13 | 1.74 | -1.01 | 4.49 | -2.75 | 2.75 |
|  |  | -3 | 2.02 | -0.33 | 4.04 | -2.36 | 2.02 | 1.42 | -1.19 | 4.43 | -2.61 | 3.01 | 1.34 | -1.30 | 3.98 | -2.64 | 2.64 |
|  |  | -2 | 1.61 | -0.60 | 3.41 | -2.21 | 1.80 | 0.98 | -1.30 | 3.66 | -2.27 | 2.69 | 1.03 | -1.42 | 3.60 | -2.46 | 2.56 |
|  |  | -1 | 1.32 | -0.71 | 3.05 | -2.03 | 1.73 | 0.82 | -1.41 | 3.19 | -2.23 | 2.37 | 0.79 | -1.52 | 3.03 | -2.31 | 2.24 |
|  | 3 | -4 | 2.45 | 0.49 | 4.23 | -1.96 | 1.78 | 1.70 | -0.55 | 4.23 | -2.25 | 2.53 | 1.74 | -0.53 | 3.98 | -2.27 | 2.24 |
|  |  | -3 | 1.91 | -0.03 | 3.64 | -1.94 | 1.73 | 1.36 | -0.78 | 3.80 | -2.14 | 2.43 | 1.37 | -0.80 | 3.47 | -2.17 | 2.09 |
|  |  | -2 | 1.52 | -0.30 | 3.02 | -1.82 | 1.50 | 0.96 | -0.84 | 3.18 | -1.80 | 2.22 | 1.06 | -1.00 | 3.05 | -2.06 | 1.98 |
|  |  | -1 | 1.23 | -0.44 | 2.67 | -1.67 | 1.44 | 0.84 | -1.03 | 2.73 | -1.86 | 1.89 | 0.79 | -1.09 | 2.64 | -1.88 | 1.85 |
| 4RE | 1 | -4 | 2.49 | 0.18 | 4.48 | -2.31 | 1.99 | 1.65 | -1.31 | 6.64 | -2.96 | 4.99 | 1.63 | -1.50 | 5.27 | -3.13 | 3.65 |
|  |  | -3 | 1.99 | -0.28 | 3.96 | -2.27 | 1.97 | 1.35 | -1.48 | 5.96 | -2.83 | 4.61 | 1.27 | -1.67 | 4.54 | -2.94 | 3.27 |
|  |  | -2 | 1.68 | -0.53 | 3.43 | -2.20 | 1.75 | 0.94 | -1.41 | 4.95 | -2.35 | 4.01 | 0.95 | -1.87 | 4.16 | -2.83 | 3.20 |
|  |  | -1 | 1.39 | -0.68 | 3.04 | -2.07 | 1.65 | 0.75 | -1.56 | 4.22 | -2.31 | 3.46 | 1.03 | -1.86 | 4.17 | -2.89 | 3.14 |
|  | 2 | -4 | 2.32 | 0.67 | 3.84 | -1.65 | 1.53 | 1.59 | -0.48 | 4.45 | -2.07 | 2.86 | 1.61 | -0.56 | 3.98 | -2.18 | 2.37 |
|  |  | -3 | 1.84 | 0.23 | 3.29 | -1.61 | 1.45 | 1.31 | -0.74 | 3.91 | -2.05 | 2.59 | 1.25 | -0.78 | 3.53 | -2.02 | 2.29 |
|  |  | -2 | 1.49 | -0.01 | 2.79 | -1.50 | 1.30 | 0.93 | -0.79 | 3.20 | -1.72 | 2.27 | 0.96 | -1.07 | 3.15 | -2.02 | 2.19 |
|  |  | -1 | 1.20 | -0.15 | 2.50 | -1.35 | 1.31 | 0.74 | -0.80 | 2.76 | -1.53 | 2.03 | 1.01 | -0.95 | 3.15 | -1.96 | 2.14 |
|  | 3 | -4 | 2.24 | 0.90 | 3.51 | -1.34 | 1.28 | 1.58 | -0.14 | 3.89 | -1.72 | 2.31 | 1.57 | -0.13 | 3.59 | -1.71 | 2.02 |
|  |  | -3 | 1.77 | 0.41 | 2.97 | -1.36 | 1.20 | 1.29 | -0.40 | 3.36 | -1.69 | 2.08 | 1.28 | -0.42 | 3.09 | -1.70 | 1.81 |
|  |  | -2 | 1.41 | 0.18 | 2.51 | -1.23 | 1.10 | 0.93 | -0.48 | 2.70 | -1.41 | 1.77 | 0.98 | -0.69 | 2.70 | -1.68 | 1.72 |
|  |  | -1 | 1.11 | 0.02 | 2.23 | -1.09 | 1.12 | 0.74 | -0.53 | 2.32 | -1.28 | 1.58 | 1.02 | -0.61 | 2.78 | -1.63 | 1.76 |
| 8RE | 1 | -4 | 2.34 | 0.73 | 3.81 | -1.62 | 1.47 | 1.59 | -0.79 | 6.31 | -2.38 | 4.72 | 1.55 | -1.04 | 4.89 | -2.58 | 3.34 |
|  |  | -3 | 1.83 | 0.32 | 3.27 | -1.51 | 1.44 | 1.23 | -0.87 | 5.60 | -2.10 | 4.37 | 1.21 | -1.17 | 4.19 | -2.37 | 2.99 |
|  |  | -2 | 1.47 | 0.00 | 2.82 | -1.48 | 1.35 | 0.89 | -0.85 | 4.66 | -1.75 | 3.77 | 0.94 | -1.34 | 3.74 | -2.28 | 2.80 |
|  |  | -1 | 1.18 | -0.20 | 2.44 | -1.38 | 1.26 | 0.71 | -0.99 | 3.91 | -1.69 | 3.21 | 0.72 | -1.45 | 3.19 | -2.17 | 2.47 |
|  | 2 | -4 | 2.21 | 1.05 | 3.29 | -1.15 | 1.08 | 1.57 | -0.18 | 4.09 | -1.74 | 2.52 | 1.53 | -0.27 | 3.67 | -1.80 | 2.14 |
|  |  | -3 | 1.69 | 0.61 | 2.78 | -1.08 | 1.09 | 1.24 | -0.36 | 3.51 | -1.60 | 2.27 | 1.17 | -0.48 | 3.16 | -1.65 | 1.99 |
|  |  | -2 | 1.36 | 0.31 | 2.34 | -1.05 | 0.98 | 0.89 | -0.36 | 2.88 | -1.25 | 1.99 | 0.91 | -0.70 | 2.81 | -1.62 | 1.90 |
|  |  | -1 | 1.06 | 0.09 | 1.98 | -0.97 | 0.92 | 0.69 | -0.47 | 2.41 | -1.16 | 1.72 | 0.68 | -0.82 | 2.31 | -1.50 | 1.64 |
|  | 3 | -4 | 2.14 | 1.21 | 3.05 | -0.93 | 0.91 | 1.57 | 0.11 | 3.61 | -1.47 | 2.04 | 1.54 | 0.04 | 3.26 | -1.50 | 1.72 |
|  |  | -3 | 1.63 | 0.73 | 2.52 | -0.91 | 0.89 | 1.25 | -0.08 | 3.02 | -1.32 | 1.78 | 1.18 | -0.16 | 2.78 | -1.34 | 1.60 |
|  |  | -2 | 1.30 | 0.45 | 2.09 | -0.85 | 0.79 | 0.91 | -0.15 | 2.49 | -1.06 | 1.58 | 0.93 | -0.40 | 2.43 | -1.34 | 1.50 |
|  |  | -1 | 0.99 | 0.24 | 1.78 | -0.75 | 0.79 | 0.70 | -0.26 | 2.09 | -0.96 | 1.39 | 0.69 | -0.54 | 2.03 | -1.23 | 1.34 |

# Table 3-2. CSI-RS based RSRP measurement accuracy with measurement bandwidth of 25RB

| Propagation condition | (i*160ms),i | Reference SNR[dB] | AWGN |  |  |  |  | EPA5 |  |  |  |  | ETU30 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  |
| 2RE | 1 | -4 | 1.16 | -1.37 | 3.01 | -2.53 | 1.85 | 0.48 | -2.18 | 3.22 | -2.66 | 2.74 | 0.52 | -2.26 | 2.98 | -2.77 | 2.47 |
|  |  | -3 | 0.91 | -1.38 | 2.63 | -2.29 | 1.72 | 0.27 | -2.22 | 2.95 | -2.49 | 2.69 | 0.31 | -2.33 | 2.51 | -2.64 | 2.20 |
|  |  | -2 | 0.73 | -1.26 | 2.29 | -1.99 | 1.56 | 0.14 | -2.10 | 2.31 | -2.24 | 2.17 | 0.13 | -2.19 | 2.25 | -2.32 | 2.12 |
|  |  | -1 | 0.57 | -1.11 | 1.94 | -1.68 | 1.37 | 0.04 | -1.86 | 2.01 | -1.90 | 1.97 | 0.12 | -1.94 | 1.95 | -2.06 | 1.83 |
|  | 2 | -4 | 0.95 | -0.74 | 2.35 | -1.69 | 1.41 | 0.46 | -1.41 | 2.24 | -1.87 | 1.78 | 0.48 | -1.51 | 2.22 | -1.99 | 1.74 |
|  |  | -3 | 0.70 | -0.83 | 2.03 | -1.52 | 1.33 | 0.31 | -1.42 | 1.93 | -1.73 | 1.62 | 0.28 | -1.46 | 1.86 | -1.74 | 1.58 |
|  |  | -2 | 0.55 | -0.78 | 1.76 | -1.33 | 1.22 | 0.11 | -1.35 | 1.56 | -1.46 | 1.46 | 0.15 | -1.41 | 1.61 | -1.56 | 1.45 |
|  |  | -1 | 0.41 | -0.73 | 1.43 | -1.13 | 1.02 | 0.03 | -1.27 | 1.35 | -1.30 | 1.32 | 0.09 | -1.40 | 1.45 | -1.49 | 1.35 |
|  | 3 | -4 | 0.84 | -0.55 | 2.04 | -1.39 | 1.20 | 0.43 | -1.05 | 1.88 | -1.49 | 1.45 | 0.48 | -1.10 | 1.92 | -1.59 | 1.44 |
|  |  | -3 | 0.61 | -0.66 | 1.74 | -1.27 | 1.13 | 0.28 | -1.14 | 1.60 | -1.43 | 1.32 | 0.29 | -1.15 | 1.61 | -1.44 | 1.32 |
|  |  | -2 | 0.46 | -0.63 | 1.46 | -1.09 | 0.99 | 0.10 | -1.08 | 1.29 | -1.18 | 1.19 | 0.13 | -1.10 | 1.32 | -1.23 | 1.20 |
|  |  | -1 | 0.33 | -0.57 | 1.18 | -0.90 | 0.86 | 0.02 | -1.02 | 1.07 | -1.04 | 1.05 | 0.08 | -1.14 | 1.25 | -1.22 | 1.17 |
| 4RE | 1 | -4 | 0.90 | -0.83 | 2.25 | -1.73 | 1.35 | 0.37 | -1.41 | 2.71 | -1.78 | 2.34 | 0.38 | -1.77 | 2.51 | -2.15 | 2.12 |
|  |  | -3 | 0.70 | -0.86 | 2.02 | -1.56 | 1.31 | 0.23 | -1.46 | 2.43 | -1.69 | 2.20 | 0.25 | -1.68 | 2.05 | -1.92 | 1.80 |
|  |  | -2 | 0.55 | -0.78 | 1.71 | -1.32 | 1.16 | 0.07 | -1.44 | 1.80 | -1.50 | 1.74 | 0.11 | -1.72 | 1.84 | -1.83 | 1.72 |
|  |  | -1 | 0.40 | -0.76 | 1.43 | -1.17 | 1.03 | 0.01 | -1.34 | 1.50 | -1.35 | 1.48 | 0.18 | -1.63 | 1.86 | -1.81 | 1.68 |
|  | 2 | -4 | 0.72 | -0.41 | 1.79 | -1.13 | 1.07 | 0.38 | -0.87 | 1.84 | -1.25 | 1.45 | 0.37 | -1.08 | 1.86 | -1.45 | 1.48 |
|  |  | -3 | 0.57 | -0.50 | 1.54 | -1.07 | 0.97 | 0.24 | -1.02 | 1.53 | -1.26 | 1.28 | 0.24 | -1.12 | 1.57 | -1.36 | 1.33 |
|  |  | -2 | 0.42 | -0.52 | 1.30 | -0.94 | 0.88 | 0.05 | -0.95 | 1.19 | -1.00 | 1.14 | 0.11 | -1.13 | 1.32 | -1.24 | 1.21 |
|  |  | -1 | 0.28 | -0.53 | 1.03 | -0.82 | 0.74 | 0.01 | -0.92 | 1.02 | -0.94 | 1.01 | 0.16 | -1.07 | 1.37 | -1.22 | 1.21 |
|  | 3 | -4 | 0.66 | -0.30 | 1.51 | -0.96 | 0.86 | 0.40 | -0.66 | 1.51 | -1.06 | 1.11 | 0.38 | -0.79 | 1.62 | -1.17 | 1.24 |
|  |  | -3 | 0.51 | -0.40 | 1.32 | -0.91 | 0.81 | 0.23 | -0.75 | 1.27 | -0.98 | 1.03 | 0.24 | -0.85 | 1.35 | -1.09 | 1.11 |
|  |  | -2 | 0.35 | -0.38 | 1.10 | -0.73 | 0.75 | 0.06 | -0.77 | 0.92 | -0.83 | 0.86 | 0.11 | -0.92 | 1.08 | -1.03 | 0.98 |
|  |  | -1 | 0.23 | -0.42 | 0.83 | -0.64 | 0.61 | 0.01 | -0.75 | 0.77 | -0.76 | 0.76 | 0.15 | -0.84 | 1.15 | -1.00 | 1.00 |
| 8RE | 1 | -4 | 0.73 | -0.44 | 1.82 | -1.17 | 1.09 | 0.31 | -0.99 | 2.31 | -1.31 | 2.00 | 0.33 | -1.38 | 2.09 | -1.71 | 1.76 |
|  |  | -3 | 0.56 | -0.50 | 1.54 | -1.06 | 0.98 | 0.16 | -1.03 | 1.99 | -1.19 | 1.83 | 0.20 | -1.38 | 1.83 | -1.58 | 1.63 |
|  |  | -2 | 0.42 | -0.51 | 1.31 | -0.94 | 0.89 | 0.05 | -1.02 | 1.54 | -1.06 | 1.49 | 0.08 | -1.49 | 1.58 | -1.57 | 1.50 |
|  |  | -1 | 0.30 | -0.51 | 1.08 | -0.82 | 0.78 | -0.01 | -0.95 | 1.15 | -0.93 | 1.16 | 0.05 | -1.41 | 1.40 | -1.46 | 1.35 |
|  | 2 | -4 | 0.63 | -0.22 | 1.42 | -0.85 | 0.80 | 0.36 | -0.59 | 1.63 | -0.95 | 1.27 | 0.36 | -0.85 | 1.55 | -1.21 | 1.19 |
|  |  | -3 | 0.45 | -0.31 | 1.19 | -0.76 | 0.74 | 0.20 | -0.66 | 1.26 | -0.86 | 1.06 | 0.21 | -0.89 | 1.34 | -1.10 | 1.13 |
|  |  | -2 | 0.33 | -0.35 | 1.00 | -0.68 | 0.67 | 0.05 | -0.66 | 0.98 | -0.72 | 0.93 | 0.08 | -0.96 | 1.16 | -1.04 | 1.08 |
|  |  | -1 | 0.22 | -0.37 | 0.77 | -0.59 | 0.56 | 0.01 | -0.63 | 0.77 | -0.64 | 0.76 | 0.03 | -0.98 | 0.99 | -1.02 | 0.95 |
|  | 3 | -4 | 0.57 | -0.13 | 1.23 | -0.70 | 0.66 | 0.37 | -0.43 | 1.33 | -0.80 | 0.96 | 0.35 | -0.63 | 1.35 | -0.97 | 1.00 |
|  |  | -3 | 0.40 | -0.21 | 1.03 | -0.61 | 0.63 | 0.19 | -0.49 | 1.02 | -0.68 | 0.82 | 0.22 | -0.69 | 1.16 | -0.92 | 0.94 |
|  |  | -2 | 0.29 | -0.25 | 0.83 | -0.54 | 0.54 | 0.06 | -0.55 | 0.76 | -0.61 | 0.70 | 0.07 | -0.80 | 0.92 | -0.87 | 0.85 |
|  |  | -1 | 0.18 | -0.30 | 0.63 | -0.48 | 0.45 | 0.01 | -0.51 | 0.61 | -0.51 | 0.60 | 0.04 | -0.81 | 0.83 | -0.85 | 0.80 |

# Table 3-3. CSI-RS based RSRP measurement accuracy with measurement bandwidth of 50RB

| Propagation condition | (i*160ms),i | Reference SNR[dB] | AWGN |  |  |  |  | EPA5 |  |  |  |  | ETU30 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  | 50% | 5% | 95% | [5-50%, 95-50%] |  |
| 2RE | 1 | -4 | 0.89 | -0.93 | 2.37 | -1.82 | 1.49 | 0.33 | -1.83 | 2.45 | -2.16 | 2.12 | 0.48 | -1.74 | 2.34 | -2.22 | 1.86 |
|  |  | -3 | 0.71 | -0.90 | 2.12 | -1.61 | 1.41 | 0.24 | -1.67 | 2.10 | -1.91 | 1.85 | 0.35 | -1.56 | 2.04 | -1.91 | 1.69 |
|  |  | -2 | 0.62 | -0.76 | 1.83 | -1.38 | 1.21 | 0.20 | -1.47 | 1.82 | -1.67 | 1.62 | 0.25 | -1.55 | 1.85 | -1.79 | 1.61 |
|  |  | -1 | 0.50 | -0.66 | 1.52 | -1.16 | 1.02 | 0.10 | -1.30 | 1.49 | -1.41 | 1.38 | 0.14 | -1.40 | 1.63 | -1.54 | 1.49 |
|  | 2 | -4 | 0.73 | -0.61 | 1.85 | -1.33 | 1.12 | 0.31 | -1.15 | 1.76 | -1.46 | 1.45 | 0.42 | -1.15 | 1.83 | -1.57 | 1.41 |
|  |  | -3 | 0.58 | -0.56 | 1.57 | -1.13 | 1.00 | 0.24 | -1.03 | 1.46 | -1.27 | 1.22 | 0.32 | -1.09 | 1.54 | -1.41 | 1.22 |
|  |  | -2 | 0.47 | -0.48 | 1.39 | -0.95 | 0.92 | 0.19 | -0.95 | 1.24 | -1.14 | 1.05 | 0.23 | -1.02 | 1.40 | -1.26 | 1.16 |
|  |  | -1 | 0.38 | -0.45 | 1.13 | -0.83 | 0.75 | 0.11 | -0.78 | 1.05 | -0.89 | 0.94 | 0.15 | -0.96 | 1.20 | -1.11 | 1.05 |
|  | 3 | -4 | 0.64 | -0.43 | 1.59 | -1.07 | 0.95 | 0.29 | -0.85 | 1.48 | -1.15 | 1.19 | 0.39 | -0.83 | 1.57 | -1.22 | 1.18 |
|  |  | -3 | 0.51 | -0.40 | 1.35 | -0.91 | 0.84 | 0.23 | -0.79 | 1.27 | -1.02 | 1.04 | 0.32 | -0.81 | 1.30 | -1.13 | 0.98 |
|  |  | -2 | 0.42 | -0.36 | 1.16 | -0.77 | 0.75 | 0.19 | -0.72 | 1.02 | -0.91 | 0.83 | 0.22 | -0.78 | 1.17 | -1.00 | 0.95 |
|  |  | -1 | 0.32 | -0.34 | 0.96 | -0.67 | 0.64 | 0.12 | -0.63 | 0.90 | -0.75 | 0.78 | 0.15 | -0.76 | 1.03 | -0.91 | 0.88 |
| 4RE | 1 | -4 | 0.70 | -0.58 | 1.85 | -1.28 | 1.15 | 0.28 | -1.17 | 1.92 | -1.45 | 1.64 | 0.34 | -1.40 | 1.92 | -1.74 | 1.58 |
|  |  | -3 | 0.59 | -0.58 | 1.59 | -1.17 | 1.00 | 0.21 | -1.15 | 1.60 | -1.36 | 1.39 | 0.23 | -1.25 | 1.72 | -1.48 | 1.48 |
|  |  | -2 | 0.48 | -0.46 | 1.35 | -0.93 | 0.88 | 0.17 | -1.00 | 1.38 | -1.17 | 1.21 | 0.18 | -1.23 | 1.57 | -1.41 | 1.39 |
|  |  | -1 | 0.38 | -0.45 | 1.18 | -0.83 | 0.80 | 0.08 | -0.93 | 1.16 | -1.01 | 1.08 | 0.17 | -1.22 | 1.52 | -1.39 | 1.36 |
|  | 2 | -4 | 0.57 | -0.39 | 1.42 | -0.96 | 0.85 | 0.27 | -0.77 | 1.39 | -1.03 | 1.13 | 0.33 | -0.91 | 1.50 | -1.24 | 1.18 |
|  |  | -3 | 0.48 | -0.34 | 1.21 | -0.82 | 0.73 | 0.21 | -0.66 | 1.11 | -0.87 | 0.89 | 0.23 | -0.83 | 1.28 | -1.06 | 1.05 |
|  |  | -2 | 0.38 | -0.31 | 1.04 | -0.69 | 0.66 | 0.18 | -0.61 | 0.97 | -0.79 | 0.79 | 0.19 | -0.84 | 1.16 | -1.03 | 0.97 |
|  |  | -1 | 0.29 | -0.28 | 0.86 | -0.58 | 0.57 | 0.09 | -0.56 | 0.82 | -0.65 | 0.73 | 0.17 | -0.83 | 1.12 | -1.00 | 0.95 |
|  | 3 | -4 | 0.51 | -0.27 | 1.23 | -0.78 | 0.72 | 0.26 | -0.54 | 1.17 | -0.79 | 0.92 | 0.31 | -0.68 | 1.27 | -0.99 | 0.95 |
|  |  | -3 | 0.43 | -0.23 | 1.01 | -0.66 | 0.59 | 0.22 | -0.51 | 0.97 | -0.73 | 0.75 | 0.23 | -0.63 | 1.07 | -0.87 | 0.84 |
|  |  | -2 | 0.33 | -0.22 | 0.88 | -0.56 | 0.55 | 0.19 | -0.47 | 0.79 | -0.66 | 0.61 | 0.19 | -0.66 | 0.96 | -0.85 | 0.77 |
|  |  | -1 | 0.25 | -0.22 | 0.74 | -0.48 | 0.48 | 0.09 | -0.45 | 0.70 | -0.54 | 0.61 | 0.18 | -0.67 | 0.96 | -0.85 | 0.78 |
| 8RE | 1 | -4 | 0.60 | -0.27 | 1.43 | -0.87 | 0.84 | 0.24 | -0.82 | 1.62 | -1.07 | 1.38 | 0.28 | -1.13 | 1.67 | -1.40 | 1.39 |
|  |  | -3 | 0.47 | -0.31 | 1.24 | -0.79 | 0.77 | 0.17 | -0.77 | 1.25 | -0.94 | 1.09 | 0.21 | -1.06 | 1.46 | -1.26 | 1.25 |
|  |  | -2 | 0.39 | -0.29 | 1.03 | -0.67 | 0.64 | 0.14 | -0.69 | 1.09 | -0.83 | 0.95 | 0.15 | -1.01 | 1.38 | -1.16 | 1.23 |
|  |  | -1 | 0.31 | -0.27 | 0.86 | -0.58 | 0.55 | 0.08 | -0.64 | 0.90 | -0.73 | 0.82 | 0.10 | -1.05 | 1.24 | -1.16 | 1.14 |
|  | 2 | -4 | 0.50 | -0.14 | 1.11 | -0.64 | 0.61 | 0.25 | -0.48 | 1.14 | -0.73 | 0.88 | 0.29 | -0.72 | 1.27 | -1.01 | 0.99 |
|  |  | -3 | 0.40 | -0.16 | 0.94 | -0.56 | 0.54 | 0.19 | -0.46 | 0.88 | -0.66 | 0.69 | 0.21 | -0.68 | 1.10 | -0.89 | 0.88 |
|  |  | -2 | 0.31 | -0.17 | 0.79 | -0.48 | 0.48 | 0.15 | -0.40 | 0.76 | -0.55 | 0.60 | 0.16 | -0.68 | 1.00 | -0.83 | 0.84 |
|  |  | -1 | 0.24 | -0.15 | 0.63 | -0.40 | 0.39 | 0.09 | -0.40 | 0.64 | -0.49 | 0.55 | 0.12 | -0.71 | 0.91 | -0.82 | 0.80 |
|  | 3 | -4 | 0.45 | -0.07 | 0.96 | -0.52 | 0.51 | 0.27 | -0.34 | 0.97 | -0.61 | 0.71 | 0.27 | -0.54 | 1.08 | -0.81 | 0.81 |
|  |  | -3 | 0.36 | -0.10 | 0.81 | -0.46 | 0.45 | 0.20 | -0.35 | 0.76 | -0.55 | 0.56 | 0.22 | -0.53 | 0.93 | -0.75 | 0.71 |
|  |  | -2 | 0.28 | -0.11 | 0.69 | -0.39 | 0.40 | 0.16 | -0.30 | 0.62 | -0.46 | 0.47 | 0.16 | -0.54 | 0.85 | -0.70 | 0.70 |
|  |  | -1 | 0.21 | -0.11 | 0.54 | -0.32 | 0.33 | 0.09 | -0.31 | 0.53 | -0.40 | 0.44 | 0.12 | -0.57 | 0.77 | -0.69 | 0.65 |

Based on the simulation results, we can see some observations as follows.

- Observation 1: For CSI-RS based RSRP measurement accuracy of LAA with 6RB measurement bandwidth, Rel-12 CSI-RS based RSRP measurement accuracy cannot be met with one shot measurement of  DRS period at  SNR of-1dB and 2REs/RB.

- Observation 2: For CSI-RS based RSRP measurement accuracy of LAA with 25RB measurement bandwidth, Rel-12 CSI-RS based RSRP measurement accuracy can be met with one shot measurement of  DRS period at  SNR of-1dB and 2REs/RB.

- Observation 3: For CSI-RS based RSRP measurement accuracy of LAA with 50RB measurement bandwidth, Rel-12 CSI-RS based RSRP measurement accuracy can be met with one shot measurement of  DRS period at  SNR of-1dB and 2REs/RB.

Based on these observations, we suggest following proposals.

- Proposal 1: For CSI-RS based RSRP measurement accuracy of LAA, we propose the measurement bandwidth of 25RB and 50RB to reuse Rel-12 DRS based CSI-RS RSRP measurement accuracy.

- Proposal 2: For CSI-RS based RSRP measurement accuracy of LAA, we propose the side condition of -1dB(Es/Iot)  to reuse Rel-12 DRS based CSI-RS RSRP measurement accuracy.

- Proposal 3: For CSI-RS based RSRP measurement accuracy of LAA, we propose the measurement period of one DRS period  to reuse Rel-12 DRS based CSI-RS RSRP measurement accuracy.

Conclusion

In this paper, we provided simulation results of a CSI-RS based RSRP measurement accuracy for LAA with agreed simulation assumptions. From simulation results, we also provided some observations as follows.

- Observation 1: For CSI-RS based RSRP measurement accuracy of LAA with 6RB measurement bandwidth, Rel-12 CSI-RS based RSRP measurement accuracy cannot be met with one shot measurement of  DRS period at  SNR of-1dB and 2REs/RB.

- Observation 2: For CSI-RS based RSRP measurement accuracy of LAA with 25RB measurement bandwidth, Rel-12 CSI-RS based RSRP measurement accuracy can be met with one shot measurement of  DRS period at  SNR of-1dB and 2REs/RB.

- Observation 3: For CSI-RS based RSRP measurement accuracy of LAA with 50RB measurement bandwidth, Rel-12 CSI-RS based RSRP measurement accuracy can be met with one shot measurement of  DRS period at  SNR of-1dB and 2REs/RB.

Based on these observations, we suggest following proposals.

- Proposal 1: For CSI-RS based RSRP measurement accuracy of LAA, we propose the measurement bandwidth of 25RB and 50RB to reuse Rel-12 DRS based CSI-RS RSRP measurement accuracy.

- Proposal 2: For CSI-RS based RSRP measurement accuracy of LAA, we propose the side condition of -1dB(Es/Iot)  to reuse Rel-12 DRS based CSI-RS RSRP measurement accuracy.

- Proposal 3: For CSI-RS based RSRP measurement accuracy of LAA, we propose the measurement period of one DRS period  to reuse Rel-12 DRS based CSI-RS RSRP measurement accuracy.

- Reference

- R4-156646, “Simulation assumptions for CSI-RSRP with LAA”, Ericsson