3GPP TSG-RAN WG4 Meeting #77                                       R4-158207

Anaheim, CA, USA, November 16 – 20, 2015

Agenda item:	7.50.3

Source: 	Intel, Ericsson, Huawei, Nokia, Qualcomm

Title: 	Way forward on coexistence simulation assumptions

Document for:	Approval

Proposal: It is proposed to accept this document as the way forward on simulation assumptions for the coexistence study between NB-IoT and legacy systems, except for the fields highlighted in yellow.

Table 1: Simulation assumptions and parameters

|  | IoT standalone | IoT in-band/guard-band | LTE | UMTS | GSM |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency in GHz | 0.9 and 2 | 0.9 and 2 | 0.9 and 2 | 0.9 or 2 | 0.9 |
| Size of each nominal channel BW in MHz | 0.2 | 0.18 | 10 | 5 | 0.2 |
| Transmission bandwidth in MHz | 0.18 | 0.18 | 9 | 3.84 | 0.2 |
| Environment | Urban macro | Urban macro | Urban macro | Urban macro | Urban macro |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around |
| Inter-site distance in meter | Same for aggressor and victim | Same for aggressor and victim | 750 for 0.9GHz band 500 for 2GHz band | 750 for 0.9GHz band 500 for 2GHz band | 1732 |
| System loading and activity | Full buffer 100% | Full buffer 100% | Full buffer 100% | Full buffer 100% 8kbps speech | Full buffer 100% |
| Network location | Non co-located (at cell edge of legacy) | Co-located with LTE | (see IoT) | Non co-located with IoT | Non co-located with IoT |
| DL subcarrier spacing | 15kHz | 15kHz | 15kHz |  |  |
| UL | See R1-157783 | See R1-157783 | SC-FDMA |  |  |
| DL power control | No | No | No | TR25.942 | No |
| UL power control | 36.942 section 5.1.1.6 (gamma=1, CLx is based on 5% at max TX power) or using the table in 36.942 section 5.1.1.6 by bandwidth scale | 36.942 section 5.1.1.6 (gamma=1, CLx is based on 5% at max power) or using the table in 36.942 section 5.1.1.6 by bandwidth scale | 36.942 section 5.1.1.6 (set=1) | TR25.942 | CS based on 25.816 |
| Frequency reuse | 1 | 1 | 1 | 1 | 4/12 |
| Number of scheduled UE per cell (DL) | 1 | 1 | 1 | According to 95% users achieving target of (Eb/No-0.5)dB; non orthogonality 0.4; target Eb/No=7.9dB |  |
| Number of scheduled UE per cell (UL) | FFS | FFS | 3 | according to 6dB noise rise; target Eb/No=6.1dB |  |
| BS antenna height in meter | 30 | 30 | 30 | 30 | 30 |
| BS max TX power in dBm | 43dBm/200kHz | 46 dbm | 46 | 43 | 43 |
| BS antenna gain including feeder loss in dBi | 15 | 15 | 15 | 15 | 15 |
| BS antenna pattern | Horizontal (36.942) | Horizontal (36.942) | Horizontal (36.942) | Horizontal (36.942) | Horizontal (36.942) |
| BS antenna front-back ratio in dB | 20 | 20 | 20 | 20 | 20 |
| UE antenna height in meter | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| UE TX power in dBm | -40 to 23 | -40 to 23 | -40 to 23 | -50 to 24 | 5 to 33 |
| UE antenna gain in dBi | 0 | 0 | 0 | 0 | 0 |
| Building penetration loss | 45.820 Annex D.1 | 45.820 Annex D.1 | n/a | n/a | n/a |
| Cell selection margin in dB | 3 | 3 | 3 | 3 | 3 |
| BS-MS min couple loss in dB | 70 | 70 | 70 | 70 | 70 |
| BS ACLR in dB | 40 to 60 | n/a | 45 | TR45.820 | TS45.005 (includes wideband noise emissions and IM products) |
| BS ACS in dB | 40 to 50 | n/a | 45 | TR45.820 | TS45.005 (guard-band of 100kHz or more between IoT and GSM) |
| UE ACLR in dB | 20 to 50 | n/a | 30 (ACLR1) 43 (ACLR2) | TR45.820 | TS45.005 |
| UE ACS in dB | 20 to 40 | n/a | 33 | TR45.820 | TS45.005 (guard-band of 100kHz or more between IoT and GSM) |
| BS noise figure in dB | 5 | 5 | 5 | 5 | 5 |
| UE noise figure in dB | 9 | 9 | 9 | 9 | 9 |
| BS-UE path-loss model | TR36.942 macro urban | TR36.942 macro urban | TR36.942 macro urban | TR36.942 macro urban | TR36.942 macro urban |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 | 10 | 10 | 10 | 8 |
| Shadowing correlation | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 |
| Link-level performance model |  |  | As in Annex A.1 in 36.942 |  |  |
| Evaluation metrics | SINR vs ACS (as victim) | SINR | SINR and throughput loss vs standalone IoT ACLR (as victim); SINR and throughput loss vs in-band/guard-band IoT | Capacity vs IoT ACLR (as victim). For DL, capacity is the number of satisfied users. For UL, capacity is the number of users when 6dB noise rise is reached | SINR and outage vs IoT ACLR (as victim) |
| Carrier separation | 0.3MHz to GSM 2.6MHz to UMTS 5.12MHz to LTE |  | See NB-IOT(standalone case) | See NB-IOT(standalone case) | See NB-IOT(standalone case) |
| Location of NB-IOT carrier | - | For in band operation, number 21 For guard band, adjacent to number 49 | - | - | - |