3GPP TSG-RAN WG4 Meeting #80BIS	R4-167505

Ljubljana, Slovenia, 10 - 14 Oct, 2016

Agenda Item:	6.3.4.1.1

Source: 	CATT

Title: 	simulation results of NPBCH Demodulation for NB-IoT

Document for:	Discussion

- Introduction

In RAN4#80 Meeting, the Narrow Band IoT (NB-IoT) work item (WI) has reached a way forward on demodulation performance and a performance gap between NPBCH and NPDCCH has been discussed [2]. Interested companies are encouraged to provide NPBCH performances including impairment error evaluated under ETU-1Hz and EPA-1Hz at SNR=-12dB target to testify the NPBCH coverage with different NPBCH capture time compared with NPDCCH coverage. And also the simulation results are provided to meet the 1% BLER requirement for simulation cases as described in [1]. In this contribution, we provide simulation results to show the BLER performance for NPBCH.

- NPBCH Simulation Assumptions and Results

For NPBCH, two test cases (i.e. Simulation number 1 and 2 in Table 1) have been agreed in RAN4#80 Meeting [1]. In this contribution, the main simulation parameters are accordingly configured as in Table 1 and Table 2, here for coverage study, ETU-1Hz (i.e. Simulation number 3 and 4 in Table 1) are also simulated for comparison.

# Table 1: Minimum performance for NPBCH

| Simulation number | Bandwidth | Deployment mode | Reference Channel | Propagation Condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| 1 | 180 KHz | Stand-alone | R.NB1.1 | EPA1 | 1 x 1 | 1 | TBD |
| 2 | 180kHz | In-band/Guard-band | R.NB1.2 | EPA1 | 2 x 1 Low | 1 | TBD |
| 3 | 180 KHz | Stand-alone | R.NB1.1 | ETU1 | 1 x 1 | 1 | TBD |
| 4 | 180kHz | In-band/Guard-band | R.NB1.2 | ETU1 | 2 x 1 Low | 1 | TBD |

Table 2	RMC table for NPBCH

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.NB1.1 | R.NB1.2 |
| Number of transmitter antennas |  | 1 | 2 |
| Channel bandwidth | MHz | 0.2 | 0.2 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 50/1600 | 50/1600 |
| Payload (without CRC) | Bits | 34 | 34 |

The alignment simulation results of BLER vs. SNR for Simulation Case 1&3 are shown in Fig.1.

Fig.1. BLER vs. SNR for NPBCH

Observation 1: For Simulation Case1 and Case3, the performance under ETU-1Hz and EPA-1Hz is similar,  the target SNR difference is less than 0.6dB.

Observation 2: When the NPBCH acquisition time is one TTI, for Simulation Case 1, the BLER of 1% is achieved at SNR around -3.9dB. For Simulation Case 3, the BLER of 1% is achieved at SNR around -3.7dB.

Observation 3: When the NPBCH acquisition time is 2 TTIs, for Simulation Case 1, the BLER of 1% is achieved at SNR around -6.8dB. For Simulation Case 3, the BLER of 1% is achieved at SNR around -7.1dB.

Observation 4: When the NPBCH acquisition time is 3 TTIs, for Simulation Case 1, the BLER of 1% is achieved at SNR around -8.6dB. For Simulation Case 3, the BLER of 1% is achieved at SNR around -9.2dB.

The alignment simulation results of BLER vs. SNR for Simulation Case 2&4 is shown in Fig.2.

Fig.2. BLER vs. SNR for NPBCH

Observation 1: For simulation Case2 and Case4, the performance under ETU-1Hz and EPA-1Hz is almost the same.

Observation 2: When the NPBCH acquisition time is one TTI, for Simulation Case 2&4, the BLER of 1% is achieved at SNR around -5.8dB.

Observation 3: When the NPBCH acquisition time is 2 TTIs, for Simulation Case 2&4, the BLER of 1% is achieved at SNR around -7.9dB.

Observation 4: When the NPBCH acquisition time is 3 TTIs, for Simulation Case 2, the BLER of 1% is achieved at SNR around -9.7dB. For Simulation Case 4, the BLER of 1% is achieved at SNR around -9.6dB.

- Summary

According to the alignment simulation results shown in section 2, the SNRs satisfying 1% Pm-bch for case 1-4 are shown below:

| Simulation number | Propagation Condition | Antenna configuration | Reference value | Target SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pm-dsg (%) | Max NPBCH Acquisition Time 640ms | Max NPBCH Acquisition Time 640ms*2 | Max NPBCH Acquisition Time 640ms*3 |
| 1 | EPA1 | 1x1 | 1 | -3.9 | -6.8 | -8.6 |
| 2 | EPA1 | 2x1 | 1 | -5.8 | -7.9 | -9.7 |
| 3 | ETU1 | 1x1 | 1 | -3.7 | -7.1 | -9.2 |
| 4 | ETU1 | 2x1 | 1 | -5.8 | -7.9 | -9.6 |

The impairment simulation results of the SNRs satisfying 1% Pm-bch for case 1-4 are shown below:

| Simulation number | Propagation Condition | Antenna configuration | Reference value | Target SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pm-dsg (%) | Max NPBCH Acquisition Time 640ms | Max NPBCH Acquisition Time 640ms*2 | Max NPBCH Acquisition Time 640ms*3 |
| 1 | EPA1 | 1x1 | 1 | -2.1 | -5 | -6.8 |
| 2 | EPA1 | 2x1 | 1 | -4 | -6.1 | -7.9 |
| 3 | ETU1 | 1x1 | 1 | -1.9 | -5.3 | -7.4 |
| 4 | ETU1 | 2x1 | 1 | -4 | -6.1 | -7.8 |

- Conclusion

In this contribution, we provided simulation results to show the BLER performance for NPBCH.

Observation  1:  Similar performance can be got under EPA-1Hz and ETU-1Hz.

Observation 2: As for NPDCCH of 1% BLER test cases, the target SNR can be as low as -12dB. For NPBCH achieving the same level of BLER, when considering the impairment factors, there can be a nearly 5dB SNR gap even when the acquisition time is hypothesized to be as long as 3 TTIs.

Proposal:   Further discussion is recommended on the TTI design of NPBCH.

- Reference

[1] R4-165352, “Updated simulation assumptions for NB-IoT UE demodulation”, Huawei, HiSilicon, Ericsson, Qualcomm, Samsung, ZTE, AUG 2016.

[2]  R4-167117, “REVISION Way Forward on NPBCH Coverage Study r4”, Intel, Ericsson, ZTE, Huawei, AUG 2016.