3GPP TSG-RAN WG4 Meeting #80BIS	R4-167506

Ljubljana, Slovenia, 10 - 14 Oct, 2016

Agenda Item:	6.3.4.1.2

Source: 	CATT

Title: 	simulation results of NPDCCH Demodulation for NB-IoT

Document for:	Discussion

- Introduction

In RAN4#80 Meeting, the Narrow Band IoT (NB-IoT) work item (WI) has reached a way forward [1] and further simulation assumptions for NB-IoT NPDCCH demodulation evaluations are given [2]. Interested companies are encouraged to provide the simulation results including impairment error using the assumption parameters suggested. In this contribution, we provide our simulation results to show the BLER performance for NPDCCH.

- NPDCCH Simulation Assumptions and Results

For NPDCCH, four simulation cases have been agreed in RAN4 [3]. In this contribution, the main simulation parameters are accordingly configured as in Table 1 and Table 2. And the reference channel configuration is given is Table 3.

Table 1: Minimum performance NPDCCH for Single-antenna cases

| Test number | Deployment mode | Repetition number (Rmax) | Operated carrier | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | Stand-alone/Guard-band | [64] | Anchor | R.NB.3 FDD | [EPA5] | 1 | 1 | TBD |
| 2 | Stand-alone/Guard-band | [512] | Non-anchor | R.NB.3 FDD | [ETU1] | 1 | 1 | TBD |

Table 2 Minimum performance NPDCCH for Transmit diversity cases

| Test number | Deployment mode | Repetition number (Rmax) | Operated carrier | Reference Channel | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 3 | In-band | [128] | Anchor | R.NB.4 FDD | [EPA5] | 2 | 1 | TBD |
| 4 | In-band | [512] | Non-anchor | R.NB.4 FDD | [ETU1] | 2 | 1 | TBD |

Table 3	   RMC table for NPDCCH

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.3 FDD | R.NB.4 FDD |
| Number of NRS ports |  | 1 | 2 |
| Channel bandwidth | MHz | 0.2 | 0.2 |
| Aggregation level | NCCE | 2 | 2 |
| DCI Format |  | N1 | N1 |
| Payload (without CRC) | Bits | 23 | 23 |

The simulation results of BLER vs. SNR for Simulation Case 1 is shown in Fig.1.

Fig.1. BLER vs. SNR for Simulation Case 1

The simulation results of BLER vs. SNR for Simulation Case 2 is shown in Fig.2.

Fig.2. BLER vs. SNR for Simulation Case 2

For Simulation Case 1 and 2 of Stand-alone/Guard-band configuration, the goal is to testify the recommended repetition level (RL) that achieves BLER of 1% for target SNR around -6dB and -12dB, respectively.

The alignment simulation results of case 1&2 are shown below:

| Simulation number | Propagation Condition | Antenna configuration | Reference value | Target SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pm-dsg (%) |  |  |  |  |
| 1 | EPA5 | 1x1 | 1 | RL=32 | RL=64 | RL=128 | RL=256 |
|  |  |  |  | 1.2 | -3.4 | -7.5 | -11.6 |
| 2 | ETU1 | 1x1 | 1 | RL=256 | RL=512 | RL=768 | RL=1024 |
|  |  |  |  | -5.4 | -9.9 | -12.2 | -13.9 |

The impairment simulation results of the SNRs satisfying 1% Pm-pdcch for case 1&2 are shown below:

| Simulation number | Propagation Condition | Antenna configuration | Reference value | Target SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pm-dsg (%) |  |  |  |  |
| 1 | EPA5 | 1x1 | 1 | RL=32 | RL=64 | RL=128 | RL=256 |
|  |  |  |  | 3 | -1.6 | -5.7 | -9.8 |
| 2 | ETU1 | 1x1 | 1 | RL=256 | RL=512 | RL=768 | RL=1024 |
|  |  |  |  | -3.6 | -8.1 | -10.4 | -12.1 |

Observation 1:  For Simulation Case 1 at target SNR = -6dB, if RL=64, BLER>1%; if RL=128, BLER<1%.

Observation 2: For Simulation Case 2 at target SNR = -12dB, if RL=512, BLER>1%; if RL=1024, BLER<1%.

Proposal 1: For Simulation Case 1 with target SNR = -6dB, the repetition level RL=128 is sufficient; for Simulation Case 2 with target SNR = -12dB, the repetition level RL=1024 is sufficient.

The simulation results of BLER vs. SNR for Simulation Case 3 is shown in Fig.3.

Fig.3. BLER vs. SNR for Simulation Case 3

The simulation results of BLER vs. SNR for Simulation Case 4 is shown in Fig.4.

Fig.4. BLER vs. SNR for Simulation Case 4

For Simulation Case 3 and 4 of In-band configuration, the goal is to testify the recommended repetition level (RL) that achieves BLER of 1% for target SNR around -6dB and -12dB, respectively.

The alignment simulation results of case 3&4 are shown below:

| Simulation number | Propagation Condition | Antenna configuration | Reference value | Target SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pm-dsg (%) |  |  |  |  |
| 3 | EPA5 | 2x1 | 1 | RL=64 | RL=128 | RL=256 |  |
|  |  |  |  | -6.5 | -10.3 | -13.4 |  |
| 4 | ETU1 | 2x1 | 1 | RL=256 | RL=512 | RL=768 | RL=1024 |
|  |  |  |  | -9.4 | -12.3 | -14 | -15.6 |

The impairment simulation results of  case 3&4 are shown below:

| Simulation number | Propagation Condition | Antenna configuration | Reference value | Target SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pm-dsg (%) |  |  |  |  |
| 3 | EPA5 | 2x1 | 1 | RL=64 | RL=128 | RL=256 |  |
|  |  |  |  | -4.7 | -8.5 | -11.6 |  |
| 4 | ETU1 | 2x1 | 1 | RL=256 | RL=512 | RL=768 | RL=1024 |
|  |  |  |  | -7.6 | -10.5 | -12.2- | -13.8 |

Observation 3: For Simulation Case 3 at target SNR = -6dB, if RL=64, BLER>1%; if RL=128, BLER<1%.

Observation 4: For Simulation Case 4 at target SNR = -12dB, if RL=512, BLER>1%; if RL=768, BLER<1%.

Proposal 2: For Simulation Case 3 with target SNR = -6dB, the repetition level RL=128 is sufficient; for Simulation Case 4 with target SNR = -12dB, the repetition level RL=768 is sufficient.

- Conclusion

In this contribution, we provided simulation results to show the BLER performance for NPDCCH and then share our view of recommendations on the repetition levels for different simulation cases.

Proposal 1: For Simulation Case 1 with target SNR = -6dB, the repetition level RL=128 is sufficient; for Simulation Case 2 with target SNR = -12dB, the repetition level RL=1024 is sufficient.

Proposal 2: For Simulation Case 3 with target SNR = -6dB, the repetition level RL=128 is sufficient; for Simulation Case 4 with target SNR = -12dB, the repetition level RL=768 is sufficient.

- Reference

[1] R4-166656, “Way forward on NB-IoT UE demodulation requirements”, Ericsson, Huawei, HiSilocon, ZTE, Qualcomm, Intel, Samsung , AUG  2016.

[2] R4-165352, “Updated simulation assumptions for NB-IoT UE demodulation”, Huawei, HiSilicon, Ericsson, Qualcomm, Samsung, ZTE, AUG 2016.

[3]  R4-166663, “CR NB-IoT NPDCCH demodulation requirements”, Ericsson, AUG 2016.