3GPP TSG-RAN WG4 #80bis	R4-168547

Ljubljana, Slovenia, 10-14 October 2016

Source: 	Ericsson

Title:  	TP on test equipment for TR36.789: Multi-node tests for LAA

Agenda Item:	7.1.1

Document for:	Approval

# Introduction

LAA multi-node tests will be studied under a new SI, which is approved in [1]. TR36.789 corresponds to documentation from this SI. A skeleton is proposed in [2]. In this contribution, we propose a text for this TR which includes contents related to coexistence tests for LAA.

# Text proposal

The following text proposal is intended for the introduction and Chapter 4 of TR 36.789.

<<<<< Start of TP <<<<<<

## 5.2	Test equipment

<Editor’s note: Include contents related to parameters and descriptions for devices under tests >

### 5.2.1	Measurement setup

Following test setup has been agreed as shown below in Figure 5.2.1-1.

ATT1

Companion victim device

TX/RX

TX/RX

Aggressor device (DUT)

TX/RX

Victim device

Iagg-vic

Ivic-agg

ATT2

ATT3

Companion aggressor device

TX/RX

ATT4

SNRvic

SNRagg

Figure 5.2.1-1 Test setup for multi-node tests for Rel-13 LAA

As seen in the figure above, the setup includes one LAA node, one IEEE 802.11 AP, one LAA UE and one IEEE 802.11 STA for the multi-node throughput tests.

The basic setup will be that both LAA node and IEEE 802.11 AP will operate in best effort traffic, i.e. in priority class 3.

It is worth noting that we only concentrate on DL traffic in this setup. Thus, traffic is only expected in the LAA node and IEEE 802.11 AP.

Applicable to both the sections 5.2.2 and 5.2.3, the parameter settings in the test should be documented for test repeatability. Moreover, repeatability of the tests shall be ensured.

### 5.2.2	LAA eNB and UE device configurations

Regarding choosing LAA devices for the tests:

- LAA BS is supplied by the vendors

- LAA UEs should be representative and taken off the shelf

### For LAA BS, the parameter setting should be out-of-the-box. Default settings can be used for LAA UE.

### 5.2.3	IEEE 802.11 AP and STA device configurations

For IEEE 802.11 devices to be used in the test, both APs and STAs, they should be representative and taken off the shelf

For IEEE 802.11 devices, both APs and STAs, the default setting is used

N commercial IEEE 802.11 APs and M commercial IEEE 802.11 STAs devices from different vendors should be used. For IEEE 802.11 APs and STAs, the devices should come from multi-vendor and multi-standard (i.e. multi-generation IEEE 802.11 systems). N is 3 and M is 2.

<<<<< End of TP <<<<<<

# Conclusion

We propose to adopt the above mentioned text proposal for TR 36.789.

# References

RP-161197, New Study Item proposal: Multi-node tests for LAA, Huawei, HiSilicon, Ericsson, Qualcomm, Nokia

R4-166543, Draft TR skeleton for LAA multi-node tests SI (TR 36.789), Ericsson, RAN4#80.