3GPP TSG-RAN WG4 Meeting #78 	R4-160029

St. Julian’s, Malta, 15 – 19 February, 2016

Agenda item:	9.1.3

Source: 	ZTE

Title: 	Updated Coexistence Results for +26dBm UE on Band 41

Document for:	Discussion

1 Introduction

The SI of new power class for Band 41 UE of 26dBm has been proposed in [1]. In RAN4#76bis meeting, the parameters for Band 41 HPUE simulations have been discussed and approved [2]. The methodology includes the following steps:

- Run the B41 UL (23dBm and ACLR according to TS 36.101) to adjacent B41 UL (fulfilling ACS as in TS 36.104) coexistence study and derive the performance degradation of B41 UL (performance degradation is related to B41UL without interference)

- Run the B41 UL (26dBm and ACLR+XdB) to adjacent B41 UL (fulfilling ACS as in TS 36.104) coexistence study and derive the performance degradation of B41 UL (performance degradation is related to B41UL without interference), for the same inter-site distance as in (1)

- Derive ACLR+X for B41 26dBm UE that implies similar system performance degradation towards B41 UL without interference as B41 23dBm UE with ACLR according to TS 36.101, i.e. 30dB

Further, updated the power control parameter to place the simulation power control curve close to the behaviour of represented network was proposed in RAN4#77, which leads the power class 2 UE has 3dB greater than power class 3 UE in terms of the CLx-ile value in both urban and rural scenarios when Gamma equals to 1, the CLx-ile value turn out to be 4 dB greater when Gamma equals to 0.8 in both urban and rural scenarios. At the mean time, the CLx-ile value has been decreased in rural scenario with both power class 2 and power class 3 UE. This is the updated coexistence study results for +26dBm UE on Band 41.

- Simulation configurations

The simulation configurations follow the parameters given in [2]. Here, the key parameters are summarized as follows.

2.1     Simulation methodology, inter-system interference

During the Band 14 HPUE WI, ACLR for Power Class 3 was derived. The simulation methodology is included in [3]. The similar methodology can be used now.

- Run the B41 UL (23dBm and ACLR according to TS 36.101) to adjacent B41 UL (fulfilling ACS as in TS 36.104) coexistence study and derive the performance of B41 UL (this performance is settled as baseline)

- Run the B41 UL (26dBm and ACLR+XdB) to adjacent B41 UL (fulfilling ACS as in TS 36.104) coexistence study and derive the performance degradation of B41 UL (performance degradation is related to the performance derived in step 1), for the same inter-site distance as in (1)

- Derive ACLR+X for B41 26dBm UE that implies similar system performance degradation towards performance derived in step 1 as B41 23dBm UE with ACLR according to TS 36.101, i.e. 30dB

2.2    Cell layout

Two scenarios are proposed in [2], here we use the first scenario for high power UE as Figure 1: Base stations with 3 sectors per site are placed on a hexagonal grid with distance of 3*R, where R is the cell radius, with wrap around. Uncoordinated macro cellular deployment is assumed, where interfering UE may be at cell edge of the serving base station but close to the victim base station (hence transmitting with highest power and causing highest interference).

Figure 1: Multi system cell layout for HPUE to increase power in an existing coverage area

2.3    Inter-site distance and propagation model

The propagation model we used derived in [3]. From [3]

Case 1:		d  0.6 km

Case 2:		d  0.6 km

where:

d is the base station-UE separation in kilometres

2.4	Power Control Modelling

In [4], the following power control equation is used for the uplink coexistence simulations:

(1)

where Pmax is the maximum transmit power, Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level, CL is the coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}, where path loss is propagation loss plus shadowfading, G_TX is the transmitter antenna gain in the direction of the receiver, G_RX is the receiver antenna gain in the direction of the transmitter and CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax. Finally, 0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel. For 26 dBm UE, it is assumed that Rmin has 3dB more dynamic range.

Detailed parameter configurations for Eq.1 can be found in [4], where 18 sets are defined. The power control parameters were updated in [5], the specified parameters are shown in the following Tables.

Table 1: Modified power control algorithm parameters for +23 dBm UE 
(a) For 0.75 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 109 | 112 |
| Set 1’ | 1 | 117 | 120 |
| Set 2 | 0,8 | 133 | 137 |

(b) For 2.8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 133 | 136 |
| Set 2 | 0,8 | 149 | 153 |

(c) For 6 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 117 133 | 120 136 |

(d) For 8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 122 133 | 124 136 |

Table 2: Modified power control algorithm parameters for +26 dBm UE 
(a) For 0.75 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 112 109 | 115 112 |
| Set 1’ | 1 | 120 117 | 123 120 |
| Set 2 | 0,8 | 137 133 | 141 137 |

(b) For 2.8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 136 133 | 139 136 |
| Set 2 | 0,8 | 153 149 | 157 153 |

(c) For 6 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 120 136 | 123 139 |

(d) For 8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 125 136 | 127 139 |

- Simulation results

  - Results on Performance Loss, Urban – ISD = 750m

The results for urban scenario with 0.75km inter-site distance are plotted in Figure 2 – 4, respectively. In case of PC set 1 and 1’, the ACLR value of the 26dBm UE need to be improved 3.5dB so that the 23dBm UE throughput degradation due to 26dBm interfering UE can be acceptable. While in case of PC set 2, simulation result shows that the ACLR value of the 26dBm UE need to improve about 3dB.

Figure 2: Throughput degradation due to ACI. Urban scenario – ISD=750m – PC set 1.

Figure 3: Throughput degradation due to ACI. Urban scenario – ISD=750m – PC set 1’.

Figure 4: Throughput degradation due to ACI. Urban scenario – ISD=750m – PC set 2.

  - Results on Performance Loss, suburban– ISD = 2.8km

Similar observations can be made from the results in Figure 5-6 in suburban scenario with 2.8km inter-site distance. It is found that the ACLR of the 26 dBm UE need to be improved about 3.5 dB so that the victim system performance degradation due to 26 dBm interfering UE is the same as that due to 23 dBm interfering UE.

Figure 5: Throughput degradation due to ACI. Suburban scenario – ISD=2.8km – PC set 1.

Figure 6: Throughput degradation due to ACI. Suburban scenario – ISD=2.8km – PC set 2.

  - Results on Performance Loss, Rural– ISD = 6km

In rural scenario with 6 km inter-site distance, due to the thermal noise is dominating, very low mean throughput and negligible cell edge throughput are achievable, which causes more sensitive throughput loss and unpredictable trends. It is hard to make a meaningful comparison in terms of ACLR impact.

Figure 7: Throughput degradation due to ACI. Rural scenario – ISD=6km – PC set 1.

  - Results on Performance Loss, Rural– ISD = 8km

Similar fluctuation is found in Figure 8 in rural scenario with 8 km inter-site distance. This is due to the fact that the thermal noise is dominating, therefore very low mean throughput and negligible cell edge throughput are achievable, which causes more sensitive throughput loss and unpredictable trends. It is hard to make a meaningful comparison in terms of ACLR impact.

Figure 8: Throughput degradation due to ACI. Rural scenario – ISD=8km – PC set 1.

- Conclusion

This contribution provides the system level simulation results using the approved assumptions in the urban and rural areas. In urban scenario, the ACLR value of the 26dBm UE need to be improved at least 3.5dB in the case of both PC set1 and PC set1’, so that the 23dBm UE throughput degradation due to 26dBm interfering UE can be acceptable. And in the case of PC set2, there is only 3dB need to be improved for ACLR value. In suburban scenario, the same result can be derived with urban scenario in PC set 1 and set1’.While in rural scenario with both 6km and 8km inter-site distance, it is hard to make meaningful comparison in terms of ACLR impact.

5  Reference

- RP-151104, “Study on- New Band 41 UE power class supporting +26 dBm”, Sprint, CMCC, Alcatel-Lucent, Softbank, C-Spire, Sequans, Ericsson, Intel, Samsung, SouthernLINC, Dish, Blackberry, China Unicom.

- R4-156495, “TP to TR ab.cde: Parameters for Band 41 HPUE simulations”, Ericsson, TSG-RAN Working Group 4 (Radio) meeting #76bis.

- R4-154000, “System level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm“,Alcatel-Lucent.

[4].	3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

[5].  R4-157921, “HPUE simulation anomalies”,Sprint Corporation.

[6]	R4-156402, “System level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm” , Alcatel-Lucent, 3GPP TSG-RAN WG4 (Radio) Meeting #76bis, Sophia, Antipolis, France, 12 – 16 October 2015