3GPP TSG-RAN4 Meeting #78	R4-160053

# Malta, Malta, 15th Feb 2016 - 19th Feb 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3386 | rev | - | Current version: | 13.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for correction to syncOffsetIndicator parameter in D2D resource pool configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_D2D_Prox-Perf |  |  |  |  |  | Date: |  |  | 2016-02-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To fix an error in the value for the RRC parameter syncOffsetIndicator in the D2D discovery resoruce pool configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To set a subframe ‘X’ as a synchronization subframe, the parameter syncOffsetIndicator needs to be set a value (X mod 40) [TS 36.331] In current specification, syncOffsetIndicator is erroneously mentioned as ‘X’. This CR changes the value of syncOffsetIndicator as (current value mod 40). Note this there is no change the actual subframe configured for transmission of synchrnoization signals. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Value of the RRC parameter syncOffsetIndicator will not be as per ASN.1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

## A.7.1	Reference resource pool configurations for ProSe Direct Discovery demodulation tests

### A.7.1.1	FDD

Table A.7.1.1-1: ProSe Direct Discovery configuration for E-UTRA FDD (Configuration #1-FDD)

| Information Element |  |  | Value |
| --- | --- | --- | --- |
| discRxPool | cp-Len |  | Normal |
|  | discPeriod |  | rf32 |
|  | numRetx |  | 0 |
|  | numRepetition |  | 1 |
|  | tf-ResourceConfig | prb-Num | 12 |
|  |  | prb-Start | 0 |
|  |  | prb-End | 23 |
|  |  | offsetIndicator | 160 |
|  |  | subframeBitmap | 10000000 00000000 00000000 00000000 00000000 |
|  | txParameters |  | not present |
|  | rxParameters |  | not present |
| discTxPoolCommon |  |  | not present |
| discTxPowerInfo |  |  | not present |
| SL-SyncConfig |  |  | not present |
| discInterFreqList |  |  | not present |

Table A.7.1.1-2: ProSe Direct Discovery configuration for E-UTRA FDD (Configuration #2-FDD)

| Information Element |  |  | Value |
| --- | --- | --- | --- |
| discRxPool(0) | cp-Len |  | Normal |
|  | discPeriod |  | rf32 |
|  | numRetx |  | 0 |
|  | numRepetition |  | 1 |
|  | tf-ResourceConfig | prb-Num | 12 |
|  |  | prb-Start | 0 |
|  |  | prb-End | 23 |
|  |  | offsetIndicator | 150 |
|  |  | subframeBitmap | 10000000 00000000 00000000 00000000 00000000 |
|  | txParameters |  | not present |
|  | rxParameters |  | not present |
| discRxPool(1) | cp-Len |  | Normal |
|  | discPeriod |  | rf32 |
|  | numRetx |  | 0 |
|  | numRepetition |  | 1 |
|  | tf-ResourceConfig | prb-Num | 12 |
|  |  | prb-Start | 0 |
|  |  | prb-End | 23 |
|  |  | offsetIndicator | 170 |
|  |  | subframeBitmap | 10000000 00000000 00000000 00000000 00000000 |
|  | txParameters |  | not present |
|  | rxParameters | tdd-Config | not present |
|  |  | syncConfigIndex | 0 |
| discTxPoolCommon |  |  | not present |
| discTxPowerInfo |  |  | not present |
| SL-SyncConfig(0) | syncCP-Len |  | Normal |
|  | syncOffsetIndicator |  | 0 (160 mod 40) |
|  | slssid |  | 30 |
|  | txParameters |  | not present |
|  | rxParamsNCell | physCellId | 1 |
|  |  | discSyncWindow | w1 |
| discInterFreqList |  |  | not present |

Table A.7.1.1-3: ProSe Direct Discovery configuration for E-UTRA FDD (Configuration #3-FDD)

| Information Element |  |  | Value |
| --- | --- | --- | --- |
| discRxPool(iPool), iPool = 0…NPool-1 | cp-Len |  | Normal |
|  | discPeriod |  | rf32 |
|  | numRetx |  | 3 |
|  | numRepetition |  | =2 if NPool > 10, =1 otherwise |
|  | tf-ResourceConfig | prb-Num | 5MHz: min{24, 2N-24*iPool} / 2 10MHz: 25 15MHz: min{74, 2N-74*iPool} / 2 20MHz: 50 |
|  |  | prb-Start | 0 |
|  |  | prb-End | 5 MHz: min{24, 2N-24*iPool} - 1 10 MHz: 49 15 MHz: min{74, 2N-74*iPool} - 1 20 MHz: 99 |
|  |  | offsetIndicator | 160 |
|  |  | subframeBitmap | a(0), a(1), …, a(39), s.t. a(i * NPool + iPool) = 1, i = 0,..,K; a(k) = 0 otherwise where K = 1 is NPool > 10, K = 3 otherwise |
|  | txParameters |  | not present |
|  | rxParameters |  | not present |
| discTxPoolCommon |  |  | not present |
| discTxPowerInfo |  |  | not present |
| SL-SyncConfig |  |  | not present |
| discInterFreqList |  |  | not present |
| NOTE 1: The resource pool configuration description is parameterized using channel BW, number of configured resource pools (NPool), and maximum number of configured Sidelink UEs to be supported (N). |  |  |  |

### A.7.1.2	TDD

Table A.7.1.2-1: ProSe Direct Discovery configuration for E-UTRA TDD Config 0 (Configuration #1-TDD)

| Information Element |  |  | Value |
| --- | --- | --- | --- |
| discRxPool | cp-Len |  | Normal |
|  | discPeriod |  | rf32 |
|  | numRetx |  | 0 |
|  | numRepetition |  | 1 |
|  | tf-ResourceConfig | prb-Num | 12 |
|  |  | prb-Start | 0 |
|  |  | prb-End | 23 |
|  |  | offsetIndicator | 163 |
|  |  | subframeBitmap | 10000000 00000000 00000000 00000000 00000000 00 |
|  | txParameters |  | not present |
|  | rxParameters |  | not present |
| discTxPoolCommon |  |  | not present |
| discTxPowerInfo |  |  | not present |
| SL-SyncConfig |  |  | not present |
| discInterFreqList |  |  | not present |

Table A.7.1.2-2: ProSe Direct Discovery configuration for E-UTRA TDD (Configuration #2-TDD)

| Information Element |  |  | Value |
| --- | --- | --- | --- |
| discRxPool(iPool), iPool = 0…NPool-1 | cp-Len |  | Normal |
|  | discPeriod |  | rf32 |
|  | numRetx |  | 3 |
|  | numRepetition |  | =2 if NPool > 10, =1 otherwise |
|  | tf-ResourceConfig | prb-Num | 5MHz: min{24, 2N-24*iPool} / 2 10MHz: 25 15MHz: min{74, 2N-74*iPool} / 2 20MHz: 50 |
|  |  | prb-Start | 0 |
|  |  | prb-End | 5 MHz: min{24, 2N-24*iPool} - 1 10 MHz: 49 15 MHz: min{74, 2N-74*iPool} - 1 20 MHz: 99 |
|  |  | offsetIndicator | 163 |
|  |  | subframeBitmap | a(0), a(1), …, a(39), s.t. a(i * NPool + iPool) = 1, i = 0,..,K; a(k) = 0 otherwise where K = 1 is NPool > 10, K = 3 otherwise |
|  | txParameters |  | not present |
|  | rxParameters |  | not present |
| discTxPoolCommon |  |  | not present |
| discTxPowerInfo |  |  | not present |
| SL-SyncConfig |  |  | not present |
| discInterFreqList |  |  | not present |
| NOTE 1: The resource pool configuration description is parameterized using channel BWs, number of configured resource pools (NPool), and maximum number of configured Sidelink UE to be supported (N). |  |  |  |

< END OF CHANGE #1 >