3GPP TSG-RAN WG4 Meeting #78 	R4-160088

St Julian’s, Malta, 15 – 19 February 2016

Source:	DISH Network

Title:	TP for TR36.749v.0.0.0 Section 8.1.2 (UE TX requirement)

Work item:	LTE_AWS_3_4

Agenda item:	8.9.1

Document for:	Approval

## Introduction

This document provides a TP for TR36.749v.0.0.0 Section 8.1.2(UE TX requirements). (pCR)

## 2.0	UE TX requirements

This section is a TP designed to capture the key aspect of the UE TX requirements to support Band [XX].

#### 2.2.1	Maximum output power

Band [XX] Maximum output power is also potentially impacted it is desired to share a common PA between Band 4 and Band [XX]. However, it is possible that a shared PA in this frequency range is also shared with other bands which may require more output power therefore Band 4 is typically not the limiting band when dimensioning the output PA power. Therefore, it is proposed that maximum output power can be maintained at 23dBm +/- 2 dB for Band [XX].

#### 2.2.2	A-MPR / Spectrum emission mask

#### The FCC specifies an OOBE level of 43+10log10(P) (i.e. -13dBm/MHz) to the proposed uplink frequency block [3]. This is similar to Band 2, 4, 10, 25, 35, 36 and 66. Therefore, in this case NS_03 would also be applicable for the Band [XX] uplink. The proposed changes to TS 36.101 are shown in Table 8.1.2.2-1

Table 8.1.2.2-1: Additional Maximum Power Reduction (A-MPR)

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |
| NS_03 | 6.6.2.2.1 | 2, 4,10, 23, 25, 35, 36, 66, [XX] | 3 | >5 | ≤ 1 |
|  |  |  | 5 | >6 | ≤ 1 |
|  |  |  | 10 | >6 | ≤ 1 |
|  |  |  | 15 | >8 | ≤ 1 |
|  |  |  | 20 | >10 | ≤ 1 |
| - | - | - | - | - |  |

## 3.0	References

[3]	3GPP TR 36.870: "Study on New AWS 3/4 Band for LTE".

## Proposal for TR36.749v0.0.0

---- Start of TP -------

# 8	Study of E-UTRA specific issues

## 8.1	UE specific (stand-alone band)

### 8.1.1	Filter data

### 8.1.2	UE TX requirements

#### 8.1.2.1	Maximum output power

Band [XX] Maximum output power is also potentially impacted it is desired to share a common PA between Band 4 and Band [XX]. However, it is possible that a shared PA in this frequency range is also shared with other bands which may require more output power therefore Band 4 is typically not the limiting band when dimensioning the output PA power. Therefore, it is proposed that maximum output power can be maintained at 23dBm +/- 2 dB for Band [XX].

#### 8.1.2.2	A-MPR / Spectrum emission mask

#### The FCC specifies an OOBE level of 43+10log10(P) (i.e. -13 dBm/MHz) to the proposed uplink frequency block [3]. This is similar to Band 2, 4, 10, 25, 35, 36 and 66. Therefore, in this case NS_03 would also be applicable for the Band [XX] uplink. The proposed changes to TS 36.101 are shown in Table 8.1.2.2-1

Table 8.1.2.2-1: Additional Maximum Power Reduction (A-MPR)

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |
| NS_03 | 6.6.2.2.1 | 2, 4,10, 23, 25, 35, 36, 66, [XX] | 3 | >5 | ≤ 1 |
|  |  |  | 5 | >6 | ≤ 1 |
|  |  |  | 10 | >6 | ≤ 1 |
|  |  |  | 15 | >8 | ≤ 1 |
|  |  |  | 20 | >10 | ≤ 1 |
| - | - | - | - | - |  |