3GPP TSG-RAN WG4 Meeting #78	R4-160116

# St. Julian’s, Malta, 15 – 19 February, 2016

Agenda Item:	8.13.1

Source: 	Ericsson

Title: 					Simulation Assumptions for Co-existence

Document for:		Approval

# 1.	Introduction

As per the LTE Work Item entitled “Support for V2V services based on LTE sidelink” [1] LTE-based V2X (i.e. vehicle-to-vehicle, vehicle-infrastructure or vehicle-to-pedestrian) is in demand  from a market requirement perspective as widely deployed LTE-based networks provide the opportunity for the automotive industry to realize V2X functionality. As noted in [1] the market for V2V communication in particular is time sensitive due to related research, field tests, and regulatory work that is currently ongoing or expected to commence.

From [1], RAN4 has a mandate to investigate and complete the following objectives:

- To specify UE Tx and Rx RF requirements covering operations at up to 6 GHz carrier [RAN4]

- To specify RRM core requirements [RAN4]

The work item should cover V2V services both with and without LTE network coverage, and cover both the operating scenarios where the carrier(s) is/are dedicated to V2V services and the operating scenarios where the carrier(s) is/are licensed spectrum and also used for normal LTE operation. This work should consider extensions to V2I/V2P. This work should also consider progress in SA WGs.

The specified enhancements should reuse the existing features of LTE as much as possible.

This contribution discusses the co-existence assumptions to be employed by RAN4 in Release 14 for the analysis of V2X performance.

# 2.	Discussion of V2X Co-existence Analysis

## In 3GPP RAN4 the co-existence evaluation methodology has been based on the methods and assumptions defined in 3GPP TR36.942 [3]. Furthermore, in TR36.885 [2] Appendix A, detailed assumptions are defined on use case scenarios, channel models, deployment scenarios, as well as drop and mobility models for the simulation of V2V performance. These details are reproduced in Appendix A for reference.  It is proposed that these assumptions be adopted for the Release 14 RAN4 co-existence analysis for V2X operation. Table 1 below summarizes the specific references for the given requirements. In particular, the co-existence impact of PC5 based V2X on legacy Uu operations needs to be assessed.

In addition note that candidates for bands and operations that should be considered for co-existence analysis are discussed in further detail in [5].

## Table 2: V2X Simulation Assumptions

| Parameter | Assumption |
| --- | --- |
| Evaluation Scenarios | TR36.885 Appendix A.1.1 |
| UE Drop and Mobility Model | TR36.885 Appendix A.1.2 |
| eNB and RSU deployment | TR36.885 Appendix A.1.3 |
| V2V and V2I Channel Model | TR36.885 Appendix A.1.4 |
| Traffic Model | TR36.885 Appendix A.1.5 |

Proposal #1:

- The RAN4 Release 14 co-existence evaluation methodology shall be based on the methods and assumptions defined in 3GPP TR36.942.

Proposal #2:

- For RAN4 Release 14 co-existence evaluations, the use case definitions, channel models and deployment scenarios of Appendix A of TR36.885 are to be employed.

Proposal #3:

- Working assumptions for the V2X design not fully agreed in RAN1 can be made for co-existence analysis.

Proposal #4:

## The co-existence impact of PC5 based V2X on legacy LTE Uu operations is to be assessed.

# 3	Conclusions

The following proposals should be taken into consideration when defining the Release 14 V2X RF co-existence requirements:

Proposal #1:

- The RAN4 Release 14 co-existence evaluation methodology shall be based on the methods and assumptions defined in 3GPP TR36.942.

Proposal #2:

- For RAN4 Release 14 co-existence evaluations, the use case definitions, channel models and deployment scenarios of Appendix A of TR36.885 are to be employed.

Proposal #3:

- Working assumptions for the V2X design not fully agreed in RAN1 can be made for co-existence analysis.

Proposal #4:

## The co-existence impact of PC5 based V2X on legacy LTE Uu operations is to be assessed.

# 4			References

[1]	RP-151109, Feasibility Study on LTE-based V2X Services, LG Electronics, CATT, Vodafone, Huawei.

[2]		3GPP TR36.885, “3GPP E-UTRA Study on LTE-based V2X Services; (Release 14)

[3]		3GPP TR36.942, “3GPP E-UTRA RF System Scenarios”, v13.0

[4]	3GPP TR36.101, “3GPP E-UTRA UE Radio Transmission and Reception”, v13.0

[5]		R4-160114, “Candidate Bands for Vehicle-to-Vehicle Communication”, Ericsson

Appendix A:
Evaluation methodology

For PC5-based V2V, tradeoff between system and link level performance can be studied. Assumption on the target link budget in link level is as follows:

  - 150 m in NLOS Urban case

  - 320 m in Freeway case

## A.1 System level simulation assumptions

For PC5-based V2V, the following general assumptions apply:

- Each vehicle UE’s reception is subject to the half duplex constraint.

## A.1.1 Evaluation scenarios

Two vehicle UE dropping cases are defined: Urban case and Freeway case. See Section A.1.2 for the UE drop and mobility model in each case. See Section A.1.4 for the channel model in each case.

Macro eNB may or may not be deployed in the evaluations. If deployed, the assumptions in Section A.1.3 should be used. If not, simple wrap around can be used as long as it is aligned with the evaluation assumptions in this TR.

Details of evaluation scenarios are in Table A.1.1.-1.

Table A.1.1-1: Details of evaluation scenarios

| Parameter |  | Assumption |
| --- | --- | --- |
| Carrier frequency for PC5-based V2V |  | 6 GHz, 2 GHz |
| Bandwidth |  | 10 MHz |
| Number of carriers |  | One carrier is baseline. Other numbers can be evaluated based on inter-carrier interference model from the existing UE-UE link ACIR [28] dB according to [5] for the adjacent channel case. |
| Synchronization |  | Time and frequency error should be considered in system and link level simulations. Companies should explain the assumed error model and the method to achieve the error range. Until RAN4 provides an answer, RAN1 will assume at least the case where frequency error (i.e., error in the oscillator) is in the range of +- 0.1 PPM. |
| Vehicle UE parameters | In-band emission | In-band emission model in Section A.2.1.5 in [4] is reused with {W, X, Y, Z} = {3, 6, 3, 3} for single cluster SC-FDMA. |
|  | Antenna height | 1.5 m |
|  | Antenna pattern | Omni 2D |
|  | Antenna gain | 3 dBi |
|  | Maximum transmit power | 23 dBm |
|  | Number of antennas | 1 TX and 2 RX antennas. Baseline is that 2 RX antennas are separated by wavelength/2. |
|  | Noise figure | 9 dB |

## A.1.2 UE drop and mobility model

Vehicle UEs are dropped on the roads according to spatial Poisson process. The vehicle density is determined by the assumption on the vehicle speed, and the vehicle location should be updated every 100 ms in the simulation.

In Urban case, a vehicle changes its direction at the intersection as follows:

- Go straight with probability 0.5

- Turn left with probability 0.25

- Turn right with probability 0.25

Details of vehicle UE drop and mobility model for each of Urban and Freeway cases are in Table A.1.2-1. Figures A.1.2-1 and A.1.2-2 illustrate the road configuration of the two cases.

Table A.1.2-1: Details of vehicle UE drop and mobility model

| Parameter | Urban case | Freeway case |
| --- | --- | --- |
| Number of lanes | 2 in each direction (4 lanes in total in each street) | 3 in each direction (6 lanes in total in the freeway) |
| Lane width | 3.5 m | 4 m |
| Road grid size by the distance between intersections | 433 m * 250 m. Note that 3 m is reserved for sidewalk per direction (i.e., no vehicle or building in this reserved space) | N/A |
| Simulation area size | Minimum [1299 m * 750 m] | Freeway length >= 2000 m. Wrap around should be applied to the simulation area. |
| Vehicle density | Average inter-vehicle distance in the same lane is 2.5 sec * absolute vehicle speed. Baseline: The same density/speed in all the lanes in one simulation. |  |
| Absolute vehicle speed | 15 km/h, 60 km/h | 140 km/h, 70 km/h |

Figure A.1.2-1: Road configuration for Urban case

Figure A.1.2-2: Road configuration for Freeway case

## A.1.3 eNB and RSU deployment

If macro eNBs are deployed for Urban case, ISD of macro eNB is 500 m and the wrap around model in Figure A.1.3-1 is used.

If macro eNBs are deployed for Freeway case,

- Option 1 (baseline): eNBs are located along the freeway 35m away with 1732m ISD in Figure A.1.3-2.

- Option 2 (optional): Wrap around method of 19*3 hexagonal cells with 500m ISD in Figure A.1.3-3.

Figure A.1.3-1: Wrap around model for Urban case

ISD=1732m

Simulation region=2ISD=3464m

Wrapping around region

Wrapping around region

Figure A.1.3-2: Wrap around model option 1 (baseline) for Freeway case

Figure A.1.3-3: Wrap around model option 2 (optional) for Freeway case

FFS on how to handle mobility and handover related issue

## A.1.4 Channel model

Assumptions for channel between two vehicle UEs are in Table A.1.4-1.

Table A.1.4-1: Assumptions for vehicle-to-vehicle channel

| Parameter | Urban case | Freeway case |
| --- | --- | --- |
| Pathloss model | WINNER+ B1 Manhattan grid layout (note that the antenna height should be set to 1.5 m.). Pathloss at 3 m is used if the distance is less than 3 m. | LOS in WINNER+ B1 (note that the antenna height should be set to 1.5 m.). Pathloss at 3 m is used if the distance is less than 3 m. |
| Shadowing distribution | Log-normal | Log-normal |
| Shadowing standard deviation | 3 dB for LOS and 4 dB for NLOS | 3 dB |
| Decorrelation distance | 10 m | 25 m |
| Fast fading | NLOS in Section A.2.1.2.1.1 or A.2.1.2.1.2 in [4] with fixed large scale parameters during the simulation. |  |

Vehicle-to-vehicle channels are updated during the simulation as follows:

- Let N be the number of vehicle UE in system simulation

- Initialization (at time 0)

  - N vehicle locations are generated per agreed drop model

  - PL (0) – NxN matrix generated as per vehicle locations and agreed channel models

  - Shadowing (in log domain): S(0) – NxN i.i.d. (with the exception that shadowing between two vehicles should be the same in the two directions) normal matrix generated as per agreed shadowing model

  - Fading (0) – NxN i.i.d. processes with a common distribution

- Update (at time 100*n ms)

  - Vehicle locations are updated as per agreed update rules

  - PL(n) – N x N matrix generated as per updated vehicle locations

  - S(n) = exp(-D/D_corr) .* S(n-1) +sqrt{ (1-exp(-2*D/D_corr))}.*N_S(n)

    - where N_S(n) is an NxN  i.i.d. (with the exception that shadowing between two vehicles should be the same in the two directions) normal matrix generated  as per the agreed shadowing model

    - D is the update distance matrix where D(i,j) is change in distance of link i to j from time n-1 to time n

  - Fading process is not impacted due to vehicle location updates – fading is only updated due to time

  - UE performance should reflect fast fading variation within the subframe

Assumptions for channel between a UE and a macro eNB in the cell layout in Figures A.1.3-1, A.1.3-2, and A.1.3-3 are in Table A.1.4-2. For Urban case and option 2 of Freeway case, assumptions not in Table A.1.4-2 are the same as the assumptions of 3GPP case 1 in A.2.1.1.1 in [7]. For option 1 of Freeway case, assumptions not in Table A.1.4-2 are the same as the assumptions of 3GPP case 3 in A.2.1.1.1 in [7].

Table A.1.4-2: Assumptions for channel between UE and macro eNB

| Parameter | Assumption |
| --- | --- |
| Pathloss model | 128.1 + 37.6log10(R), R in kilometers |
| Penetration loss | 0 dB |
| Shadowing distribution | Log-normal |
| Shadowing standard deviation | 8 dB |
| Decorrelation distance | 50 m |
| Fast fading | 3GPP Spatial Channel Model (SCM) NLOS in [7] with fixed large scale parameters during the simulation. |

Shadowing is updated as follows:

  - Let M be the number of eNB sites

  - Initialization (at time 0)

    - Shadowing: SeNB2UE,i(0) =R*Ni (0)

      - R is a MxM matrix to generate shadowing correlation between eNB sites.

        - A Shadowing correlation factor of 0.5 for the shadowing between eNB sites and of 1.0 between sectors of the same eNB site are used

      - SeNB2UE,i(0): Mx1 shadowing values between the ith UE and eNB sites

      - Ni(0): Mx1 i.i.d. normal vector generated for the ith UE.

  - Update (at time 100*n ms)

    - UE locations are updated as per A.1.2.

    - SeNB2UE,i (n) = exp(-Di/D_corr) .* SeNB2UE,i (n-1) +sqrt{(1-exp(-2*Di/D_corr))}.*(R*Ni (n))

      - where Ni (n) is an Mx1 i.i.d. normal vector for the ith UE.

      - Di denotes the update distance matrix for the ith UE where Di(k,k) is change in distance of the ith UE to the kth eNB site from time n-1 to time n. Note that Di is a diagonal matrix.

      - D_corr = 50m

## A.1.5 Traffic model

- Traffic model for V2V

There are two traffic models used in evaluation: Periodic traffic case and Event-triggered traffic case. Periodic traffic case is mandatory. Event-triggered traffic case can be evaluated optionally with or without Periodic traffic.

Every vehicle in the simulation generates messages according to the traffic model.

For Periodic traffic, message generation periods are defined in the following 5 distinctive scenarios in Table A.1.5-1.

Table A. 1.5-1:  Message generation period for Periodic traffic

| Index | Vehicle dropping scenarios | Absolute vehicle speed (km/h) | Message generation period (ms) |
| --- | --- | --- | --- |
| 1 | Freeway | 140 | 100 |
| 2 | Freeway | 70 | 100 |
| 3 | Urban | 60 | 100 |
| 4 | Urban | 15 | 100 |
| 5 | Urban | 15 | 500 |

For Periodic traffic, working assumption of message size is that one 300-byte message followed by four 190-byte messages, and the time instance of 300-byte size message generation is randomized among vehicles. Note that it is allowed not to consider message size in calculating the performance metric.

For Event-triggered traffic, event arrival follows Poisson process with the arrival rate X (up to company choice) per second for each vehicle. Once event triggered, 6 messages are generated with space of 100ms. Working assumption of message size for Event-trigger traffic at L1 is 800bytes.

- Traffic model for xxx [Editor notes: placeholder of traffic model for V2I/N, V2P in case they are different from that for V2V ]

## A.1.6 Performance metric

For evaluation of proposed schemes for V2V, the following metric(s) shall be considered.

- Packet Reception Ratio (PRR) :

  - For one Tx packet, the PRR is calculated by X/Y, where Y is the number of UE/vehicles that located in the range (a, b) from the TX, and X is the number of UE/vehicles with successful reception among Y. CDF of PRR and the following average PRR are used in evaluation

    - CDF of PRR with a = 0, b = baseline of 320 meters for freeway and 150 meters for urban. Optionally, b = 50 meters for urban with 15 km/h vehicle speed.

    - Average PRR, calculated as (X1+X2+X3….+Xn)/(Y1+Y2+Y3…+Yn) where n denotes the number of generated messages in simulation. with a = i*20 meters, b = (i+1)*20 meters for i=0, 1, …, 25

- FFS Packet Inter-Reception (PIR): time elapsed between two successive successful receptions of two different packets transmitted from node A to node B

Companies should explain the amount of time and frequency resources used for simulation.

Note that the system should work for all the bands up to 6 GHz, including 5.9 GHz. This study is not intended to make any implication for the study on channel above 6 GHz.

The intention is to capture the sparse and medium cases in [6].

This optional value shall not be used to justify the fulfilment of SA1 requirements. It shall only apply to system-level evaluations.