3GPP TSG-RAN WG4 Meeting #78        	        R4-160121

# Malta, Malta, 15- 19 Feb., 2016

Agenda Item:	6.6.2

Source: 	Samsung

Title: 	Simulation results of synchronous network for UL MMSE-IRC Evaluation

Document for:	Discussion

At last RAN4#77 conference, we submitted link level performance evaluation of Phase-II in synchronous network [1]. In this contribution, after further check, we update the performance of MMSE-IRC algorithms based on the agreed simulation assumptions.

# Simulation Assumptions

As given in [2], the common parameters for link level evaluation are given in Table 1. The evaluation cases with specific parameters are given in Table 2. For the interference channel modeling, if the serving channel is EPA5, then ETU5 channel  model should be used. While the serving channel is EVA70, ETU70 should be used instead.

For link level simulation in Phase II, the simulation results of throughput versus SINR of MMSE-IRC receiver at the 70% relative throughput will be provided. The ideal simulation results will be provided without the impairments.

Table 1: Common parameters of link level evaluation assumptions

| Parameters |  | Unit | Values |
| --- | --- | --- | --- |
| Cyclic prefix |  |  | Normal |
| Interference modelling | Number of explicitly modelled interferers |  | 2 |
|  | Noc | dBm/15K | [-98] |
|  | Interference modulation |  | 16QAM |
|  | Timing delay and frequency offset for synchronous case |  | Well aligned: no timing delay and frequency offset between the serving UE and interfering UEs |
| HARQ | HARQ combining |  | Incremental redundancy |
|  | Redundancy version sequence |  | 0, 2, 3, 1, 0, 2, 3, 1 |
|  | Maximal number of HARQ transmissions (including 1st transmission and re-transmissions) |  | 4 |
| Frequency hopping, TTI bundling |  |  | Disable |

Table 2: Cases for link level evaluations

| Num | MCS | Propagation condition (Serving, interferers) | Antenna configuration for serving and interferers | (DIP1, DIP2) dB |
| --- | --- | --- | --- | --- |
| 2 | 6 | (EPA5, ETU5) | 1x2 Low | (-0.43, N/A) |
| 4 | 15 | (EPA5, ETU5) | 1x4 Low | (-0.43, -13.78) |
| 6 | 20 | (EPA5, ETU5) | 1x8 Low | (-0.43, -13.78) |
| 7 | 6 | (EVA70, ETU70) | 1x2 Low | (-1.11, N/A) |
| 9 | 15 | (EVA70, ETU70) | 1x4 Low | (-1.11, -10.91) |
| 11 | 20 | (EVA70, ETU70) | 1x8 Low | (-1.11, -10.91) |

# Simulation Results

Based on the assumptions stated above, the simulation results of throughput of MMSE-IRC versus SINR at the 70% relative throughput for 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz bandwidths are provided in Table 3 ~ Table 8 respectively.

The detailed results are provided in the attached Excel file.

Table 3: Comparison of MMSE-IRC and MMSE by Link Level Evaluation of 1.4MHz Bandwidth

| Num | PRB allocation/bandwidth | MCS | Propagation condition (Serving, interferers) | Antenna configuration | (DIP1, DIP2) | Samsung |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC performance |
| 2 | 6PRB/1.4MHz | 6 | (EPA5, ETU5) | 1x2 Low | (-0.43, N/A) | -6.26 |
| 4 | 6PRB/1.4MHz | 15 | (EPA5, ETU5) | 1x4 Low | (-0.43, -13.78) | -5.13 |
| 6 | 6PRB/1.4MHz | 20 | (EPA5, ETU5) | 1x8 Low | (-0.43, -13.78) | -5.63 |
| 7 | 6PRB/1.4MHz | 6 | (EVA70, TU70) | 1x2 Low | (-1.11, N/A) | -3.45 |
| 9 | 6PRB/1.4MHz | 15 | (EVA70, TU70) | 1x4 Low | (-1.11, -10.91) | -1.48 |
| 11 | 6PRB/1.4MHz | 20 | (EVA70, TU70) | 1x8 Low | (-1.11, -10.91) | -1.81 |

Table 4: Comparison of MMSE-IRC and MMSE by Link Level Evaluation of 3MHz Bandwidth

| Num | PRB allocation/bandwidth | MCS | Propagation condition (Serving, interferers) | Antenna configuration | (DIP1, DIP2) | Samsung |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC performance |
| 2 | 15PRB/3MHz | 6 | (EPA5, ETU5) | 1x2 Low | (-0.43, N/A) | -6.76 |
| 4 | 15PRB/3MHz | 15 | (EPA5, ETU5) | 1x4 Low | (-0.43, -13.78) | -5.37 |
| 6 | 15PRB/3MHz | 20 | (EPA5, ETU5) | 1x8 Low | (-0.43, -13.78) | -5.45 |
| 7 | 15PRB/3MHz | 6 | (EVA70, TU70) | 1x2 Low | (-1.11, N/A) | -4.03 |
| 9 | 15PRB/3MHz | 15 | (EVA70, TU70) | 1x4 Low | (-1.11, -10.91) | -1.65 |
| 11 | 15PRB/3MHz | 20 | (EVA70, TU70) | 1x8 Low | (-1.11, -10.91) | -1.60 |

Table 5: Comparison of MMSE-IRC and MMSE by Link Level Evaluation of 5MHz Bandwidth

| Num | PRB allocation/bandwidth | MCS | Propagation condition (Serving, interferers) | Antenna configuration | (DIP1, DIP2) | Samsung |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC performance |
| 2 | 25PRB/5MHz | 6 | (EPA5, ETU5) | 1x2 Low | (-0.43, N/A) | -7.15 |
| 4 | 25PRB/5MHz | 15 | (EPA5, ETU5) | 1x4 Low | (-0.43, -13.78) | -5.22 |
| 6 | 25PRB/5MHz | 20 | (EPA5, ETU5) | 1x8 Low | (-0.43, -13.78) | -5.42 |
| 7 | 25PRB/5MHz | 6 | (EVA70, TU70) | 1x2 Low | (-1.11, N/A) | -4.23 |
| 9 | 25PRB/5MHz | 15 | (EVA70, TU70) | 1x4 Low | (-1.11, -10.91) | -1.43 |
| 11 | 25PRB/5MHz | 20 | (EVA70, TU70) | 1x8 Low | (-1.11, -10.91) | -1.69 |

Table 6: Comparison of MMSE-IRC and MMSE by Link Level Evaluation of 10MHz Bandwidth

| Num | PRB allocation/bandwidth | MCS | Propagation condition (Serving, interferers) | Antenna configuration | (DIP1, DIP2) | Samsung |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC performance |
| 2 | 50PRB/10MHz | 6 | (EPA5, ETU5) | 1x2 Low | (-0.43, N/A) | -7.19 |
| 4 | 50PRB/10MHz | 15 | (EPA5, ETU5) | 1x4 Low | (-0.43, -13.78) | -5.42 |
| 6 | 50PRB/10MHz | 20 | (EPA5, ETU5) | 1x8 Low | (-0.43, -13.78) | -5.34 |
| 7 | 50PRB/10MHz | 6 | (EVA70, TU70) | 1x2 Low | (-1.11, N/A) | -4.35 |
| 9 | 50PRB/10MHz | 15 | (EVA70, TU70) | 1x4 Low | (-1.11, -10.91) | -1.60 |
| 11 | 50PRB/10MHz | 20 | (EVA70, TU70) | 1x8 Low | (-1.11, -10.91) | -1.53 |

Table 7: Comparison of MMSE-IRC and MMSE by Link Level Evaluation of 15MHz Bandwidth

| Num | PRB allocation/bandwidth | MCS | Propagation condition (Serving, interferers) | Antenna configuration | (DIP1, DIP2) | Samsung |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC performance |
| 2 | 75PRB/15MHz | 6 | (EPA5, ETU5) | 1x2 Low | (-0.43, N/A) | -6.85 |
| 4 | 75PRB/15MHz | 15 | (EPA5, ETU5) | 1x4 Low | (-0.43, -13.78) | -4.84 |
| 6 | 75PRB/15MHz | 20 | (EPA5, ETU5) | 1x8 Low | (-0.43, -13.78) | -5.15 |
| 7 | 75PRB/15MHz | 6 | (EVA70, TU70) | 1x2 Low | (-1.11, N/A) | -4.14 |
| 9 | 75PRB/15MHz | 15 | (EVA70, TU70) | 1x4 Low | (-1.11, -10.91) | -1.48 |
| 11 | 75PRB/15MHz | 20 | (EVA70, TU70) | 1x8 Low | (-1.11, -10.91) | -1.35 |

Table 8: Comparison of MMSE-IRC and MMSE by Link Level Evaluation of 20MHz Bandwidth

| Num | PRB allocation/bandwidth | MCS | Propagation condition (Serving, interferers) | Antenna configuration | (DIP1, DIP2) | Samsung |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC performance |
| 2 | 100PRB/20MHz | 6 | (EPA5, ETU5) | 1x2 Low | (-0.43, N/A) | -7.25 |
| 4 | 100PRB/20MHz | 15 | (EPA5, ETU5) | 1x4 Low | (-0.43, -13.78) | -5.27 |
| 6 | 100PRB/20MHz | 20 | (EPA5, ETU5) | 1x8 Low | (-0.43, -13.78) | -5.41 |
| 7 | 100PRB/20MHz | 6 | (EVA70, TU70) | 1x2 Low | (-1.11, N/A) | -4.43 |
| 9 | 100PRB/20MHz | 15 | (EVA70, TU70) | 1x4 Low | (-1.11, -10.91) | -1.63 |
| 11 | 100PRB/20MHz | 20 | (EVA70, TU70) | 1x8 Low | (-1.11, -10.91) | -1.34 |

# Conclusion

We further  update link level simulation results as per case defined in [2].

# Reference

1.	R4-157332, Updated Phase-II Link Level Simulations Results for Synchronous Network. Samsung, 3GPP TSG-RAN WG4 meeting #77, Anaheim, CA, US, 16 – 20 Nov, 2015.

2.	R4-156866, WF on phase-II test parameters for BS MMSE-IRC in synchronous network. ZTE, China Telecom, etc. 3GPP TSG-RAN WG4 meeting #76bis, Sophia Antipolis, France, 12-16 Oct., 2015.