3GPP TSG-RAN WG4 Meeting #78	R4-160156

St. Julian’s, Malta, 15th – 19th February, 2016

Agenda Item:	8.12.3

Source: 	Intel Corporation

Title: 	Discussion on recognition of high speed scenarios

Document for:	Discussion

- Introduction

The work item (WI) “Performance enhancements for high speed scenario” was approved at RAN plenary meeting #70 [1]. One objective of this WI is to specify higher-layer assistance signalling for UEs complying with the enhanced downlink demodulation performance requirements under the high speed scenarios.

Because of different properties between the HST SFN channel model and conventional fading channels [2] – [6], such as EVA channels, it would be beneficial for the UE to recognize it entering/leaving the HST SFN environment. In this contribution, we discuss a network-assisted signalling to indicate that a train is entering/leaving the HST SFN area.

- Network-assisted signalling for entering/leaving HST SFN area

Fig. 1 Network-assisted signaling for entering/leaving HST SFN.

Figure 1 shows that when the train is entering/leaving the HST SFN area, the non-SFN BS may send signals to UEs indicating that they are entering/leaving HST SFN area; alternatively, the signaling operation can be performed by SFN RRH/BS on the edge of the SFN area as well.

- Conclusion

Proposal: when a high speed train is entering/leaving the HST SFN area, the network entities, such as non-SFN and/or SFN RRH/BS may assist and signal UEs that they are entering/leaving HST SFN area. Then, the UE can take corresponding actions, such as channel estimation for high speed scenario.

- Reference

[1] RP-152263, “New WI proposal: Performance enhancements for high speed scenario”, NTT DOCOMO INC., Huawei, HiSilicon, Dec. 2015.

[2] R4-156999, “Discussion on UE demodulation performances for high speed train scenario with SFN deployment”, Intel Corporation, Nov.2015.

[3] R4-157944, “Improvement in UE demodulation performance in SFN channel”, Qualcomm Incorporated, Nov. 2015.

[4] R4-156001, “UE demodulation performance evaluation of the SFN channel”, Ericsson, Oct. 2015.

[5] R4-155657, “Evaluation of UE demodulation performance under the identified SFN scenario”, Huawei, HiSilicon, Oct. 2015.

[6] TR-36.878, “Study on performance enhancements for high speed scenario in LTE”, Dec. 2015.