3GPP TSG-RAN WG4 Meeting #78	R4-160158

St. Julian’s, Malta, 15th – 19th February, 2016

Agenda Item:	8.12.3

Source: 	Intel Corporation

Title: 	High speed enhanced UE performance in SFN scenarios

Document for:	Discussion

- Introduction

The work item (WI) “Performance enhancements for high speed scenario” was approved at RAN plenary meeting #70 [1]. One objective of this WI is to identify UE demodulation requirements under the identified scenarios. In the previous high speed scenario study item stage, throughput performance was observed to degrade by 20% to 50% [2] – [5] (depending on different system modelling factors, such as transmission mode, MCS, RRH SFN channels) when the train is traveling at high speed of 350km/h (Doppler shift 875Hz) compared with low speed of 30km/h (Doppler shift 70Hz).

Therefore, enhanced performance is required for UE in high speed scenarios. In this contribution, we provide evaluation of an enhanced UE’s demodulation performance under HST SFN channels.

- Simulation assumptions and results

The two RAN4 agreed bi-directional RRH deployment scenarios in HST SFN are shown below [6]:

| Parameter | Value |
| --- | --- |
| Ds | 1000 m |
| Dmin | 10 m |
| v | 350 km/h |
| fd | 875 Hz |

Table I. Simulation parameters for Scenario I

| Parameter | Value |
| --- | --- |
| Ds | 500 m |
| Dmin | 5 m |
| v | 350 km/h |
| fd | 875 Hz |

Table II. Simulation parameters for Scenario II

Additional necessary simulation parameters are listed below:

| Parameter | Value |
| --- | --- |
| Bandwidth | 10 MHz |
| Duplex mode | FDD |
| MCS | 19 |
| Antenna configuration | 2×2 |
| Transmission mode | TM3 (rank-2) |
| Reference receiver | MMSE |

Table III. Simulation parameters for HST SFN

Figure 1 and Figure 2 illustrate the simulation results for legacy UE, HST enhanced UE and HST ideal UE. For the HST ideal UE, it has the full knowledge of the high speed channel state information including path delay and Doppler shifts. The HST enhanced UE is able to properly track the path delay and Doppler shifts, even though the train is traveling at the very middle of two RRHs.

Fig. 1 Ideal, enhanced and legacy UE’s normalized throughput in HST SFN Scenario I channel model.

Fig. 2 Ideal, enhanced and legacy UE’s normalized throughput in HST SFN Scenario II channel model.

Observation 1: the legacy UE’s demodulation performance in the HST SFN channels degrades significantly compared to the enhanced and ideal UE, due to the difficulties to properly track the fast-changing path delay and large Doppler shifts. In addition, different properties between the new two-path HST SFN channel model and the conventional fading channel models, such as EVA and ETU channels would impact the accuracy of channel estimation. The most challenging part is when the train is traveling at or around the middle of two RRHs. It is where UE is very likely to receive two paths’ signals with equal power, opposite Doppler shifts and near-to-zero time delay. In this case, the conventional channel estimation filter is not able to work properly because of the mismatch between HST SFN channel and conventional channel models.

Observation 2: the throughput performance of the enhanced UE is very close to the ideal UE who has the full knowledge of channel state information. However, it should be noted that, there can be other limitations and factors making impact on the present results. In a more detrimental environment, the channel can be of fading properties with a greater number (larger than two) of multi-paths. In that case, the performance of both enhanced UE and ideal UE may vary.

Proposal: Enhancement algorithms are necessary to retain satisfactory demodulation performance for the high speed scenario.

- Conclusion

In this contribution, we provide the evaluation of an enhanced UE’s demodulation performance under HST SFN channels.

Observation 1: the legacy UE’s demodulation performance in the HST SFN channels degrades significantly compared to the enhanced and ideal UE.

Observation 2: the throughput performance of the enhanced UE is very close to the ideal UE.

Proposal: Enhancement algorithms are necessary to retain satisfactory demodulation performance for the high speed scenario.

- Reference

[1] RP-152263, “New WI proposal: Performance enhancements for high speed scenario”, NTT DOCOMO INC., Huawei, HiSilicon, Dec. 2015.

[2] R4-156999, “Discussion on UE demodulation performances for high speed train scenario with SFN deployment”, Intel Corporation, Nov.2015.

[3] R4-157944, “Improvement in UE demodulation performance in SFN channel”, Qualcomm Incorporated, Nov. 2015.

[4] R4-156001, “UE demodulation performance evaluation of the SFN channel”, Ericsson, Oct. 2015.

[5] R4-155657, “Evaluation of UE demodulation performance under the identified SFN scenario”, Huawei, HiSilicon, Oct. 2015.

[6] TR-36.878, “Study on performance enhancements for high speed scenario in LTE”, Dec. 2015.