3GPP TSG-RAN WG4 Meeting #78	R4-160252

St. Julian’s, Malta, 15 – 19 February, 2016

Agenda Item:	8.4.2

Source: 	Nokia Networks, Alcatel-Lucent, U.S. Cellular

Title: 	TP for 36.714-04-01: BS studies of Harmonics and Intermodulation Products caused by LTE Advanced 4DL/1UL Carrier Aggregation with Bands 2, 4, 5 and 12

Document for:	Approval

# 1.	Introduction

The basket WI proposal including support of the LTE Advanced Carrier Aggregation (CA) of 3 band combinations with Bands 2, 4, 5 and 12 were approved in RAN#70 [1]. As discussed in [2], no additional BS study of harmonics and IMD products is required for these band combinations.

In this contribution, we provide a text proposal to complete the BS studies in the 4DL/1UL TR 36.714-04-01 [3].

# 2.	Text Proposal

<New clause>

## 6.x	CA_4DL_2A-4A-12B_1UL_BCS0

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: Channel bandwidths per operating band

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-4A-12B |  | 2 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |

### 6.x. 2	Co-existence studies

The 3rd IMD products caused in the BS by transmitting of CA_2A-4A-12B (i.e. 4DL CA with one component carrier in band 2 and 4 and two contiguous component carriers in band 12) are derived in the constituent 3DL and 2DL CAs, which have been already analyzed and captured into 3GPP TR 36.852-11, TR 36.852-12, TR 36.852-13, TR36.853-12 and TR36.853-13.

## 6.y	CA_4DL_4A-4A-12B_1UL_BCS0

### 6.y.1	Channel bandwidths per operating band for CA

Table 6.y.1-1: Channel bandwidths per operating band

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_4A-4A-12B |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS 36.101 |  |  |  |  |  | 55 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |

### 6.y. 2	Co-existence studies

The 3rd IMD products caused in the BS by transmitting of CA_4A-4A-12B (i.e. 4DL CA with two non-contiguous component carriers in band 4 and two contiguous component carriers in band 12) are derived in the constituent 3DL and 2DL CAs, which have been already analyzed and captured into 3GPP TR 36.852-11, TR 36.852-12, TR 36.852-13, TR36.853-12 and TR36.853-13.

## 6.z	CA_4DL_4A-5A-12B_1UL_BCS0

### 6.z.1	Channel bandwidths per operating band for CA

Table 6.z.1-1: Channel bandwidths per operating band

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_4A-5A-12B |  | 4 |  |  | Yes | Yes | Yes | Yes | 45 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |

### 6.z. 2	Co-existence studies

The 3rd IMD products caused in the BS by transmitting of CA_4A-5A-12B (i.e. 4DL CA with one component carrier in band 4 and 5 and two contiguous component carriers in band 12) are derived in the constituent 3DL and 2DL CAs, which have been already analyzed and captured into 3GPP TR 36.852-11, TR 36.852-12, TR 36.852-13, TR36.853-12 and TR36.853-13.

<End of change>

# References

[1]	RP-152249, “New WI: LTE Advanced inter-band CA Rel-14 for 4DL/1UL”, Ericsson.

[2]	R4-160251, “Required BS studies of Harmonics and Intermodulation Products caused by LTE Advanced 4DL/1UL Carrier Aggregation with Bands 2, 4, 5 and 12 (RAN#70)”, Nokia Networks, Alcatel-Lucent, U.S. Cellular.

[3]	R4-160451, “TR 4DL/1UL CA TR 36.714-04-01”, Ericsson.