3GPP TSG-RAN WG4 (Radio) Meeting #78	R4-1560260

St. Julian’s, Malta, 15 – 19 February 2016

Agenda Item:	9.1.3

Source: 	Nokia Networks, Alcatel-Lucent

Title: 	System level simulation results for coexistence study on new Band 41 UE power class supporting +26 dBm (urban and suburban areas)

Document for:	Discussion

# 1.	Introduction

The study item on New Band 41 UE power class supporting +26 dBm was approved at RAN#68 [1]. The objectives of this study item include the co-existence and compatibility of LTE systems deployed in the 2.5 GHz band and the impact on eNode B blocking requirements. Hence system level simulations need to be performed for the coexistence study.

The revised system level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm was approved at RAN4#77 [2]. This contribution provides the system level simulation results using the approved assumptions in the urban and suburban areas.

# 2.	Simulation Results and Discussion

The inter-site distances considered in this study are provided in Table 2.1 below. Note that the UL power control parameters are adjusted according to the UE maximum output power according to the approved assumptions in the urban and suburban areas. This represents the case where the 26 dBm UE is used to enhance the UL coverage in the urban and suburban areas. The channel bandwidth of 20 MHz is simulated.

Table 2.1: Inter-site distances (ISD)

| Environment | ISD (km) | ISD (miles) |
| --- | --- | --- |
| Urban | 0.75 | 0.47 |
| Suburban | 2.8 | 1.74 |

The CDFs of the UE transmit power as well as the victim system UL throughput loss Vs ACLR offset (with different power control parameter sets) for 0.75 km inter-site distance are shown in Figure 2.1 below.

| ACLR offset X [dB] | 0 | 1 |
| --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 2.53% |  |
| 5%-tile throughput loss (23 dBm interfering UE) | 2.20% |  |
| Average throughput loss (26 dBm interfering UE) | 2.65% | 2.34% |
| 5%-tile throughput loss (26 dBm interfering UE) | 2.33% | 1.84% |

(a) With Power Control Parameter Set 1

| ACLR offset X [dB] | 0 | 1 |
| --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 2.20% |  |
| 5%-tile throughput loss (23 dBm interfering UE) | 2.20% |  |
| Average throughput loss (26 dBm interfering UE) | 2.21% | 1.94% |
| 5%-tile throughput loss (26 dBm interfering UE) | 2.20% | 1.87% |

(b) With Power Control Parameter Set 1’

| ACLR offset X [dB] | 0 |
| --- | --- |
| Average throughput loss (23 dBm interfering UE) | 0.86% |
| 5%-tile throughput loss (23 dBm interfering UE) | 1.20% |
| Average throughput loss (26 dBm interfering UE) | 0.83% |
| 5%-tile throughput loss (26 dBm interfering UE) | 1.19% |

(c) With Power Control Parameter Set 2

Figure 2.1: For 0.75 km inter-site distance

It can be seen from the CDFs of the UE transmit power in Figure 2.1 that the CDFs of the 23 dBm UE and the 26 dBm UE are identical until the UE reach their maximum output power. This is expected as the CLx-ile is adjusted according to the UE maximum output power. Comparing the CDFs of the UE transmit power with Set 1 and Set 1’, it can be seen that more (~5% of 23 dBm UE and ~2.5% of 26 dBm UE) of the UE population transmitted with their maximum output power with the more aggressive Set 1. Note that with Set 2, the 26 dBm UE transmitted marginally less power than the 23 dBm UE, due to the rounding of the CLx-ile.

Moreover, it can be seen from the victim system UL throughput loss Vs ACLR offset results in Figure 2.1 that with the more aggressive Set 1, the ACLR of the 26 dBm UE need to be improved (~1 dB) so that the victim system performance degradation due to 26 dBm interfering UE is the same as that due to 23 dBm interfering UE. Note that with Set 2, the victim system performance degradation due to 26 dBm interfering UE is marginally less than that due to 23 dBm interfering UE, due to the fact that the 26 dBm UE transmitted marginally less power than the 23 dBm UE.

The CDFs of the UE transmit power as well as the victim system UL throughput loss Vs ACLR offset (with different power control parameter sets) for 2.8 km inter-site distance are shown in Figure 2.2 below.

| ACLR offset X [dB] | 0 | 1 |
| --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 1.28% |  |
| 5%-tile throughput loss (23 dBm interfering UE) | 3.97% |  |
| Average throughput loss (26 dBm interfering UE) | 1.30% | 1.17% |
| 5%-tile throughput loss (26 dBm interfering UE) | 4.10% | 3.64% |

(a) With Power Control Parameter Set 1

| ACLR offset X [dB] | 0 |
| --- | --- |
| Average throughput loss (23 dBm interfering UE) | 0.28% |
| 5%-tile throughput loss (23 dBm interfering UE) | 0.56% |
| Average throughput loss (26 dBm interfering UE) | 0.27% |
| 5%-tile throughput loss (26 dBm interfering UE) | 0.56% |

(b) With Power Control Parameter Set 2

Figure 2.2: For 2.8 km inter-site distance

Similar observations can be made from the results in Figure 2.2, namely that the CDFs of the 23 dBm UE and the 26 dBm UE are identical until the UE reach their maximum output power, more (~3% of 23 dBm UE and ~1.5% of 26 dBm UE) of the UE population transmitted with their maximum output power with the more aggressive Set 1, and the ACLR of the 26 dBm UE need to be improved (~1 dB) so that the victim system performance degradation due to 26 dBm interfering UE is the same as that due to 23 dBm interfering UE.

# 3.	Conclusion

This contribution provides the system level simulation results using the approved assumptions in the urban and suburban areas. The simulation results show that if the UL power control parameters are adjusted according to the UE maximum output power, the ACLR of the 26 dBm UE need to be improved (~1 dB) so that the victim system performance degradation due to 26 dBm interfering UE is the same as that due to 23 dBm interfering UE.

# References

[1]	RP-151104, “Study on- New Band 41 UE power class supporting +26 dBm”, Sprint, CMCC, Alcatel-Lucent, Softbank, C-Spire, Sequans, Ericsson, Intel, Samsung, SouthernLINC, Dish, Blackberry, China Unicom.

[2]	R4-158342, “HPUE simulation anomalies”, Sprint, Intel, CMCC, Qualcomm, Ericsson, Alcatel-Lucent.