3GPP TSG-RAN WG4 Meeting #78	R4-160267

St. Julian’s, Malta, 15 – 19 February, 2016

Title:	Draft LS on A New PRACH Cyclic Shift Restriction Set

Release:	Release 14

Work Item:	LTE_high_speed

Source:	RAN4

To:	RAN1

Contact Person:

#### Name:	Ren Da

#### E-mail Address:	Ren.Da@Nokia.com

Attachments:	None

# Overall Description

A new Rel-14 work item for the “Performance enhancements for high speed scenario” was approved in RAN#70 with the objective of “the requirements for at least 350 km/hr shall be developed”[1]. The issue for PRACH detection under the high speed scenario was instigated during the study phase on performance enhancements for high speed scenario and was identified the exiting PRACH cyclic shift restriction set defined in TS36.211 for high speed cells has a theoretical limitation of Doppler offset within +/-1.5*1.25KHz=+/-1.875KHz. Consider that the uplink Doppler shift will reach 2.269KHz for a high speed UE travelling at the speed of 350km/hr with carrier frequency of 3.5GHz, it is clear that the Doppler shift is out of the range that can be properly handled by the exiting cyclic shift restriction set.

During the study item on performance enhancements for high speed scenario, a new restricted set of cyclic shifts for extra high speed cells was proposed at Section 6.5.3.1 for supporting Doppler offset, at least, up to +/-2.5KHz in [2] as possible essential enhancement option (with more details on the proposal available in [3]). RAN4 would like RAN1 to investigate whether the proposal is suitable for supporting Doppler offset, at least, up to +/-2.5KHz. If yes, RAN4 would like RAN1 to consider the introduction of the new restricted set of cyclic shifts for extra high speed cells in to TS 36.211 in Rel-14.

# Actions

To RAN WG1

ACTION: RAN4 would kindly ask RAN1 to investigate whether the new restricted set of cyclic shifts for extra high speed cells proposed at Section 6.5.3.1 in [2] is suitable for supporting uplink Doppler offset, at least, up to +/-2.5KHz. If yes, RAN4 would ask RAN1 to consider introducing the new restricted set of cyclic shifts for extra high speed cells into TS 36.211 in Rel-14.

# References:

[1] RP-152263, New WI proposal: Performance enhancements for high speed scenario, NTT DOCOMO, INC., Huawei, HiSilicon

[2] 3GPP TS 36.878, “Study on performance enhancements for high speed scenario in LTE”.

[3] R4-154363, “A New Cyclic Shift Restriction Set for Very High Speed Cells”, Alcatel-Lucent

# Date of Next TSG-RAN WG4 Meeting:

3GPP TSG-RAN WG4 Meeting #78-bis				11 – 15 Apr 2016, San Jose del Cabo

3GPP TSG-RAN WG4 Meeting #79				23 - 27 May 2016, China