3GPP TSG RAN4 Meeting #78	R4-160436

Malta, 15th – 19th Feb, 2016

Title: 	MSD for 3+42 2UL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.6.1

Document for:	Discussion

# 1	Introduction

This contribution discusses MSD requirements for 3+42 UL CA.

# 2	Discussion

We use the following RF front-end architectures in our UL CA analysis. Architecture 2 (Common triplexer) was used in 1UL/2DL B3+B42 CA.

Figure 1 Cascaded diplexers -architecture

Figure 2 Common triplexer -architecture

3+42 UL CA is a tricky one in the sense that it has IMD2, IMD4, and IMD5 relation. B3 DL is naturally the victim in all of these cases.

We analyze the amount of IMD using the following component linearity assumptions:

| Component | IP2 (dBm) |  | IP4 (dBm) | IP5 (dBm) |
| --- | --- | --- | --- | --- |
| Antenna switch | 110 | 55 |  | 53 |
| Diplexer | 112 | 55 |  | 53 |
| Duplexer | 94 | 55 |  | 53 |
| PA forward mixing | 27 | 32 |  | 27 |
| PA reverse mixing | 38 | 33 |  | 32 |
| LNA | 5 | -6 |  | -10 |

Table 1 General linearity parameters

B3 duplexer TX rejection at B42	20

B3 duplexer RX rejection at B42	40

B42 filter attenuation at B3 RX		45

B42 filter attenuation at B3 TX		45

Antenna isolation 					10

PCB isolation PA-PA 				60

PCB isolation TX-RX				65

RFIC Pout B3/B42					0

B3 duplexer TX-RX isolation		50

Diplexer attenuation at B42 			15

Diplexer attenuation at B3 TX		15

Diplexer attenuation at B3 RX		15

Triplexer isolation					15

Based on our analysis the following MSD excluding CF was achieved. The most limiting factor is B3 PA forward mixing.

| CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |  |  |
| B3+B42 | B3 | IMD2 | fB42 - fB3 | 1740 | 5 | 25 | 1835 | 5 | - | 35.1 |
|  | B42 |  |  | 3575 | 5 | 25 |  |  |  |  |
|  | B3 | IMD4 | 3*fB3 - fB42 | 1765 | 5 | 25 | 1860 | 5 | - | 21.0 |
|  | B42 |  |  | 3435 | 5 | 25 |  |  |  |  |
|  | B3 | IMD5 | 2*fB42 - 3*fB3 | 1740 | 5 | 25 | 1835 | 5 | - | 1.3 |
|  | B42 |  |  | 3527.5 | 5 | 25 |  |  |  |  |

Table 2MSD with cascaded diplexers architecture

| CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |  |  |
| B3+B42 | B3 | IMD2 | fB42 - fB3 | 1740 | 5 | 25 | 1835 | 5 | - | 32.8 |
|  | B42 |  |  | 3575 | 5 | 25 |  |  |  |  |
|  | B3 | IMD4 | 3*fB3 - fB42 | 1765 | 5 | 25 | 1860 | 5 | - | 14.9 |
|  | B42 |  |  | 3435 | 5 | 25 |  |  |  |  |
|  | B3 | IMD5 | 2*fB42 - 3*fB3 | 1740 | 5 | 25 | 1835 | 5 | - | 0 |
|  | B42 |  |  | 3527.5 | 5 | 25 |  |  |  |  |

Table 3 MSD with common triplexer architecture

# 3	Conclusion

MSD for 2UL B3+B42 was analyzed with the following result. Please note that CF which slightly improved MSD is not yet included.

| CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |  |  |
| B3+B42 | B3 | IMD2 | fB42 - fB3 | 1740 | 5 | 25 | 1835 | 5 | - | 35.1 |
|  | B42 |  |  | 3575 | 5 | 25 |  |  |  |  |
|  | B3 | IMD4 | 3*fB3 - fB42 | 1765 | 5 | 25 | 1860 | 5 | - | 21.0 |
|  | B42 |  |  | 3435 | 5 | 25 |  |  |  |  |
|  | B3 | IMD5 | 2*fB42 - 3*fB3 | 1740 | 5 | 25 | 1835 | 5 | - | 1.3 |
|  | B42 |  |  | 3527.5 | 5 | 25 |  |  |  |  |

Table 4MSD with cascaded diplexers architecture

| CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |  |  |
| B3+B42 | B3 | IMD2 | fB42 - fB3 | 1740 | 5 | 25 | 1835 | 5 | - | 32.8 |
|  | B42 |  |  | 3575 | 5 | 25 |  |  |  |  |
|  | B3 | IMD4 | 3*fB3 - fB42 | 1765 | 5 | 25 | 1860 | 5 | - | 14.9 |
|  | B42 |  |  | 3435 | 5 | 25 |  |  |  |  |
|  | B3 | IMD5 | 2*fB42 - 3*fB3 | 1740 | 5 | 25 | 1835 | 5 | - | 0 |
|  | B42 |  |  | 3527.5 | 5 | 25 |  |  |  |  |

Table 5 MSD with common triplexer architecture

# 4	References