3GPP TSG-RAN WG4 Meeting #78	R4-160468

St. Julian’s, Malta, 15 - 19 February 2016

Source:	Nokia Networks, Alcatel-Lucent

Title:	LAA RSRP measurement requirements

Agenda Item:	5.5.3

Document for:	Discussion

# Introduction

In the last meeting, initial values for SINR side condition, measurement bandwidth and the time (shots) needed for CRS based RSRP/RSRQ were agreed in [1]. The agreed values were left in brackets and are as simplified in Table 1.

Table 1: Requirements for CRS based discovery signal measurements under operation with FS3.

| Measurement bandwidth [RB] | CRS Ês/Iot [dB] | Tmeasure [ms] |
| --- | --- | --- |
| 6 | [0] ≤ Ês/Iot | [3] + N |
| 6 | [-6] ≤ Ês/Iot < [0] | [5] + N |
| 25 | [0] ≤ Ês/Iot | [1] + N |
| 25 | [-6] ≤ Ês/Iot < [0] | [3] + N |

In Table 1, N is the number of configured discovery signal occasions, which are not available during measurements.

In this paper, we present our simulation results for RSRP and RSRQ measurements for different bandwidths and SINR side conditions, and compare the results to the agreed values. Simulations are done based on simulation assumptions agreed in [2].

# Discussion

Simulation results for delta RSRP with 6 PRB measurement bandwidth and 1 Tx transmission are shown in Table 2. Corresponding results for 25 PRB measurement bandwidth are shown in Table 3. RSRP measurement accuracy is calculated as: delta RSRP = estimated RSRP – ideal RSRP.

For 6 PRB measurement bandwidth it was agreed that when CRS Ês/Iot ≥ 0 dB, the UE should be able to carry out CRS based RSRP measurements based on maximum 3 DRS occasions [1]. As can be seen in the blue cells of Table 2, RSRP accuracy requirements defined in sub-sections 9.1.2 and 9.1.3 [3] are well satisfied under these conditions in all channels.

Observation 1: RSRP measurement accuracy requirements for 6 PRB measurement bandwidth and CRS Ês/Iot ≥ 0 dB can be fulfilled with the agreed 3 DRS occasions or less.

For 6 PRB measurement bandwidth, with -6 dB ≤ CRS Ês/Iot < 0 dB, UE should be able to carry out CRS based RSRP measurements based on 5 DRS occasions, as was initially agreed. Also this requirement is fulfilled in our simulations, as can be seen in the red cells in Table 2.

Observation 2: RSRP measurement accuracy requirements for 6 PRB measurement bandwidth and -6 dB ≤ CRS Ês/Iot < 0 dB can be fulfilled with the agreed 5 DRS occasions or less.

Table 2: Delta RSRP with 6 PRB measurement bandwidth and 1 Tx transmission.

| Delta RSRP |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN |  |  |  |  |  |  |  |
|  | # Subframes |  |  |  |  |  |  |
| SINR | 1 | 2 | 3 | 4 | 5 | 6 |  |
| -6 | 3.04 | 1.87 | 1.43 | 1.31 | 1.04 | 1.02 |  |
| -5 | 2.55 | 1.79 | 1.32 | 1.07 | 0.96 | 0.94 |  |
| -4 | 1.92 | 1.37 | 1.12 | 0.93 | 0.78 | 0.75 |  |
| -3 | 1.62 | 1.29 | 0.85 | 0.79 | 0.64 | 0.74 |  |
| -2 | 1.55 | 0.99 | 0.92 | 0.68 | 0.59 | 0.58 |  |
| -1 | 1.30 | 0.81 | 0.76 | 0.61 | 0.51 | 0.48 |  |
| 0 | 1.23 | 0.73 | 0.57 | 0.57 | 0.50 | 0.41 |  |
| 1 | 1.19 | 0.72 | 0.52 | 0.51 | 0.44 | 0.37 |  |
| 2 | 0.79 | 0.52 | 0.46 | 0.45 | 0.39 | 0.32 |  |
| 3 | 0.76 | 0.51 | 0.36 | 0.34 | 0.34 | 0.26 |  |
| 4 | 0.63 | 0.48 | 0.38 | 0.30 | 0.29 | 0.25 |  |
| EPA5 |  |  |  |  |  |  |  |
|  | # Subframes |  |  |  |  |  |  |
| SINR | 1 | 2 | 3 | 4 | 5 | 6 |  |
| -6 | 2.75 | 1.98 | 1.46 | 1.34 | 1.20 | 1.06 |  |
| -5 | 3.29 | 1.86 | 1.39 | 1.31 | 1.34 | 0.92 |  |
| -4 | 2.12 | 1.40 | 1.06 | 1.61 | 0.89 | 0.72 |  |
| -3 | 1.78 | 1.57 | 1.01 | 0.75 | 0.81 | 0.77 |  |
| -2 | 1.32 | 0.96 | 0.89 | 0.72 | 0.55 | 0.62 |  |
| -1 | 1.41 | 0.93 | 0.68 | 0.62 | 0.59 | 0.62 |  |
| 0 | 1.24 | 0.70 | 0.61 | 0.63 | 0.61 | 0.48 |  |
| 1 | 1.26 | 0.70 | 0.74 | 0.51 | 0.43 | 0.41 |  |
| 2 | 0.79 | 0.59 | 0.63 | 0.46 | 0.37 | 0.34 |  |
| 3 | 0.84 | 0.60 | 0.53 | 0.46 | 0.39 | 0.37 |  |
| 4 | 0.76 | 0.52 | 0.42 | 0.37 | 0.32 | 0.26 |  |
| ETU30 |  |  |  |  |  |  |  |
|  | # Subframes |  |  |  |  |  |  |
| SINR | 1 | 2 | 3 | 4 | 5 | 6 |  |
| -6 | 3.17 | 2.34 | 1.40 | 1.18 | 1.25 | 0.99 |  |
| -5 | 3.06 | 1.84 | 1.32 | 1.10 | 1.07 | 0.89 |  |
| -4 | 2.17 | 1.85 | 1.05 | 0.99 | 0.86 | 0.75 |  |
| -3 | 2.07 | 1.30 | 1.03 | 0.92 | 0.79 | 0.63 |  |
| -2 | 1.70 | 1.44 | 0.79 | 0.83 | 0.62 | 0.64 |  |
| -1 | 1.84 | 1.21 | 0.70 | 0.66 | 0.58 | 0.49 |  |
| 0 | 1.22 | 0.80 | 0.80 | 0.63 | 0.44 | 0.40 |  |
| 1 | 1.01 | 0.87 | 0.57 | 0.55 | 0.45 | 0.40 |  |
| 2 | 1.04 | 0.68 | 0.53 | 0.43 | 0.38 | 0.33 |  |
| 3 | 0.99 | 0.60 | 0.47 | 0.39 | 0.36 | 0.34 |  |
| 4 | 0.88 | 0.68 | 0.45 | 0.35 | 0.37 | 0.32 |  |

Table 3: Delta RSRP with 25 PRB measurement bandwidth and 1 Tx transmission.

| Delta RSRP |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN |  |  |  |  |  |  |  |
|  | # Subframes |  |  |  |  |  |  |
| SINR | 1 | 2 | 3 | 4 | 5 | 6 |  |
| -6 | 1.15 | 0.88 | 0.69 | 0.62 | 0.54 | 0.48 |  |
| -5 | 1.08 | 0.79 | 0.60 | 0.55 | 0.53 | 0.38 |  |
| -4 | 1.01 | 0.62 | 0.51 | 0.41 | 0.38 | 0.36 |  |
| -3 | 0.76 | 0.56 | 0.45 | 0.42 | 0.36 | 0.32 |  |
| -2 | 0.63 | 0.46 | 0.40 | 0.29 | 0.29 | 0.28 |  |
| -1 | 0.53 | 0.42 | 0.31 | 0.29 | 0.25 | 0.22 |  |
| 0 | 0.57 | 0.34 | 0.28 | 0.22 | 0.23 | 0.17 |  |
| 1 | 0.46 | 0.30 | 0.25 | 0.19 | 0.19 | 0.17 |  |
| 2 | 0.41 | 0.29 | 0.22 | 0.18 | 0.16 | 0.14 |  |
| 3 | 0.35 | 0.21 | 0.17 | 0.19 | 0.15 | 0.13 |  |
| 4 | 0.29 | 0.20 | 0.19 | 0.15 | 0.13 | 0.13 |  |
| EPA5 |  |  |  |  |  |  |  |
|  | # Subframes |  |  |  |  |  |  |
| SINR | 1 | 2 | 3 | 4 | 5 | 6 |  |
| -6 | 1.36 | 0.84 | 0.65 | 0.59 | 0.49 | 0.46 |  |
| -5 | 1.07 | 0.82 | 0.62 | 0.50 | 0.42 | 0.47 |  |
| -4 | 0.96 | 0.60 | 0.49 | 0.40 | 0.46 | 0.38 |  |
| -3 | 0.79 | 0.62 | 0.48 | 0.40 | 0.39 | 0.40 |  |
| -2 | 0.69 | 0.45 | 0.40 | 0.43 | 0.29 | 0.27 |  |
| -1 | 0.64 | 0.41 | 0.36 | 0.35 | 0.30 | 0.26 |  |
| 0 | 0.53 | 0.43 | 0.36 | 0.27 | 0.23 | 0.22 |  |
| 1 | 0.45 | 0.30 | 0.29 | 0.20 | 0.22 | 0.17 |  |
| 2 | 0.37 | 0.29 | 0.25 | 0.21 | 0.17 | 0.18 |  |
| 3 | 0.40 | 0.24 | 0.21 | 0.16 | 0.17 | 0.15 |  |
| 4 | 0.35 | 0.23 | 0.20 | 0.16 | 0.13 | 0.12 |  |
| ETU30 |  |  |  |  |  |  |  |
|  | # Subframes |  |  |  |  |  |  |
| SINR | 1 | 2 | 3 | 4 | 5 | 6 |  |
| -6 | 1.38 | 0.83 | 0.81 | 0.58 | 0.46 | 0.45 |  |
| -5 | 1.07 | 0.78 | 0.56 | 0.48 | 0.48 | 0.47 |  |
| -4 | 0.86 | 0.59 | 0.49 | 0.48 | 0.35 | 0.37 |  |
| -3 | 0.75 | 0.49 | 0.46 | 0.38 | 0.34 | 0.28 |  |
| -2 | 0.65 | 0.52 | 0.36 | 0.39 | 0.32 | 0.28 |  |
| -1 | 0.53 | 0.43 | 0.33 | 0.28 | 0.28 | 0.25 |  |
| 0 | 0.59 | 0.37 | 0.30 | 0.24 | 0.24 | 0.22 |  |
| 1 | 0.44 | 0.38 | 0.26 | 0.24 | 0.23 | 0.19 |  |
| 2 | 0.46 | 0.31 | 0.25 | 0.23 | 0.20 | 0.17 |  |
| 3 | 0.38 | 0.27 | 0.23 | 0.18 | 0.17 | 0.16 |  |
| 4 | 0.32 | 0.25 | 0.20 | 0.19 | 0.16 | 0.14 |  |

For 25 PRB measurement bandwidth it was agreed that for CRS Ês/Iot ≥ 0 dB, the UE should be able to carry out CRS based RSRP measurements based on a single DRS occasion [1]. As can be seen in the blue cells of Table 3, RSRP accuracy requirements defined in sub-sections 9.1.2 and 9.1.3 [3] are well satisfied under these conditions in all channels.

Observation 3: RSRP measurement accuracy requirements for 25 PRB measurement bandwidth and CRS Ês/Iot ≥ 0 dB can be fulfilled with the agreed single DRS occasion.

For 25 PRB measurement bandwidth, with -6 dB ≤ CRS Ês/Iot < 0 dB, UE should be able to carry out CRS based RSRP measurements based on 3 DRS occasions. Also this requirement is fulfilled in our simulations, as can be seen in the red cells in Table 3.

Observation 4: RSRP measurement accuracy requirements for 25 PRB measurement bandwidth and -6 dB ≤ CRS Ês/Iot < 0 dB can be fulfilled with the agreed 3 DRS occasions or less.

Based on the simulation results, the requirements are even fulfilled with lower amount of shots than was initially agreed. Therefore, based on the results and observations, we suggest that the brackets around CRS Ês/Iot and measurement time can be removed from tables 8.11.2.1.1.1-2, 8.11.2.1.1.2-2, 8.12.2.4.1-2, and 8.12.2.4.2-2.

Proposal 1: Remove the brackets from tables 8.11.2.1.1.1-2, 8.11.2.1.1.2-2, 8.12.2.4.1-2, and 8.12.2.4.2-2.

# Conclusion

In this paper we have presented simulation results for LAA SCell RSRP and RSRQ measurements. The results were compared to the agreed CRS Ês/Iot and measurement time values. Based on the simulation results, we have made the following observations:
Observation 1: RSRP measurement accuracy requirements for 6 PRB measurement bandwidth and CRS Ês/Iot ≥ 0 dB can be fulfilled with the agreed 3 DRS occasions or less.

Observation 2: RSRP measurement accuracy requirements for 6 PRB measurement bandwidth and -6 dB ≤ CRS Ês/Iot < 0 dB can be fulfilled with the agreed 5 DRS occasions or less.

Observation 3: RSRP measurement accuracy requirements for 25 PRB measurement bandwidth and CRS Ês/Iot ≥ 0 dB can be fulfilled with the agreed single DRS occasion.

Observation 4: RSRP measurement accuracy requirements for 25 PRB measurement bandwidth and -6 dB ≤ CRS Ês/Iot < 0 dB can be fulfilled with the agreed 3 DRS occasions or less.

Based on these observations we have proposed the following:

Proposal 1: Remove the brackets from tables 8.11.2.1.1.1-2, 8.11.2.1.1.2-2, 8.12.2.4.1-2, and 8.12.2.4.2-2.

# References

- R4-158388: "LAA measurement requirements” (CR), 3GPP TSG-RAN WG4 Meeting #77, Anaheim, USA, November 2015.

- R4-156645: “Simulation assumptions for RSRPRSRQ evaluation for LAA”; Ericsson; 3GPP TSG RAN WG4 Meeting #76bis; Sophia Antipolis, France, October 2015.

- 3GPP TS 36.133, “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”, v. 13.2.0.