# 3GPP TSG-RAN WG4 Meeting #78                                                            	R4-160543

St. Julian’s, Malta, 15 Feb-19 Feb, 2016

Title: TP for TR 36.714-05-01 on operating bands, channel bandwidths per operating band and co-existence analysis for 5DL CA_39C-41D

Source: 	CMCC

Agenda Item:	8.5.1

Document for:	Approval

1.	Introduction

The WI proposal to support the LTE Advanced Carrier Aggregation (5DL/1UL) of Band 39, Band 39, Band 41, Band 41 and Band 41 was approved in RAN#70.

In this contribution, CA operating bands, bandwidths configuration and co-existence analysis are proposed for LTE Advanced Carrier Aggregation (5DL/1UL) of Band 39, Band 39, Band 41, Band 41 and Band 41.

2.	Discussion

2.1 Channel bandwidths per operating band for CA

This contribution provides a text proposal to include the Channel bandwidths per operating band and Co-existence analysis for this work item into the TR 36.714-05-01. It can be defined as in following tables.

Table 2-1-1: 5DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_39-41 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz |  |

Table 2-1-2: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| CA configuration | Operating bands | Channel bandwidth(MHz) |  |  |  |  |  | Bandwidth Combination Set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 | 3 | 5 | 10 | 15 | 20 |  |  |
| CA_39C-41D | 39 | See CA_39C in Table 5.6A.1-1 in TS 36.101 of Bandwidth Combination Set 0 |  |  |  |  |  | 0 | 95 |
|  | 41 | See CA_41D in Table 5.6A.1-1 in TS 36.101 of Bandwidth Combination Set 0 |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all for the constituent bands as defined in table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

2.2     Co-existence analysis

2.2.1 For the BS

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting the 5DL CA configurations of CA_39C_41D can be calculated as shown in Table 2-1 below.

Table 2-1: Band 39 and Band 41 5DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1880 | 1920 | 2496 | 2690 |
| 2nd harmonics frequency limits (MHz) | 3760 | 3840 | 4992 | 5380 |
| 3rd harmonics frequency limits (MHz) | 5640 | 5760 | 7488 | 8070 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 576 | 810 | 4376 | 4610 |
| 3rd order IMD products | (2*f1-low – f2-high) | (2*f1-high – f2-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 1070 | 1344 | 3072 | 3500 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 6256 | 6530 | 6872 | 7300 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 1820 | 1980 | 2461 | 2725 |

It can be seen from Table 2-1 that 2nd order harmonics may fall into BS receive band of band 43, two-tone 2nd order IMD products may fall into BS receive band of band 12, 13, 14, 17, 27, 28, 44. 3rd order IMD products may fall to UL frequencies of Band 1, 2, 7, 22, 25, 33, 35, 36, 37, 38 or 42.

It is suggested BS transmitters supporting CA of Band 39, Band 39, Band 41, Band 41and Band 41 should not share the same antenna with Band 1, 2, 7, 12, 13, 14, 17, 22, 25, 27, 28, 33, 35, 36, 37, 38, 42, 43 or 44 BS receiver, unless the antenna path meets very stringent third order PIM specification so that the PIM will not cause Band 1, 2, 7, 12, 13, 14, 17, 22, 25, 27, 28, 33, 35, 36, 37, 38, 42, 43 or 44 BS receiver desensitization.

2.2.2 For the UE

For single UL non-simultaneous UE, the 2nd and 3rd harmonic analysis for B39 and B41 is done in the 2DL/1UL TR 36.851. None of the products falls into the B39 or B41 UE receiving bands.  For UE coexistence the single UL is already considered for these bands in TS 36.101.

3 Text Proposal

<Start of change>

1	Scope

The present document is a technical report for 5DL/1UL Inter-band Carrier Aggregation under Rel-14 time frame according to the New CA WI approach in Release 14 [1]. The purpose is to gather the relevant background information and studies in order to address 5DL/1UL Inter-band Carrier Aggregation requirements for the Rel-14 band combinations in Table 1-1.

Table 1-1: Release 14 5DL/1UL inter-band carrier aggregation combinations

| CA combination | REL independent from |
| --- | --- |
| CA_39C-41D | 12 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<Next change>

6           5 Band Carrier Aggregation with Single UL: Specific Band Combination Part

### 6.X	CA_39C-41D_BCS0

6.X.1	Channel bandwidths per operating band for CA

Table 6.X.1-1: 5DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_39-41 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz |  |

Table 6.X.1-2: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| CA configuration | Operating bands | Channel bandwidth(MHz) |  |  |  |  |  | Bandwidth Combination Set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 | 3 | 5 | 10 | 15 | 20 |  |  |
| CA_39C-41D | 39 | See CA_39C in Table 5.6A.1-1 in TS 36.101 of Bandwidth Combination Set 0 |  |  |  |  |  | 0 | 95 |
|  | 41 | See CA_41D in Table 5.6A.1-1 in TS 36.101 of Bandwidth Combination Set 0 |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all for the constituent bands as defined in table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

6.X.2	Co-existence studies

6.X.2.1    For the BS

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting the 5DL CA configurations of CA_39C_41D can be calculated as shown in Table 6.X.2.1-1 below:

Table 6.X.2.1-1: Band 39 and Band 41 5DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1880 | 1920 | 2496 | 2690 |
| 2nd harmonics frequency limits (MHz) | 3760 | 3840 | 4992 | 5380 |
| 3rd harmonics frequency limits (MHz) | 5640 | 5760 | 7488 | 8070 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 576 | 810 | 4376 | 4610 |
| 3rd order IMD products | (2*f1-low – f2-high) | (2*f1-high – f2-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 1070 | 1344 | 3072 | 3500 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 6256 | 6530 | 6872 | 7300 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 1820 | 1980 | 2461 | 2725 |

It can be seen from Table 6.X.2.1-1 that 2nd order harmonics may fall into BS receive band of band 43, two-tone 2nd order IMD products may fall into BS receive band of band 12, 13, 14, 17, 27, 28, 44. 3rd order IMD products may fall to UL frequencies of Band 1, 2, 7, 22, 25, 33, 35, 36, 37, 38 or 42.

It is suggested BS transmitters supporting CA of Band 39, Band 39, Band 41, Band 41 and Band 41 should not share the same antenna with Band 1, 2, 7, 12, 13, 14, 17, 22, 25, 27, 28, 33, 35, 36, 37, 38, 42, 43 or 44 BS receiver, unless the antenna path meets very stringent third order PIM specification so that the PIM will not cause Band 1, 2, 7, 12, 13, 14, 17, 22, 25, 27, 28, 33, 35, 36, 37, 38, 42, 43 or 44 BS receiver desensitization.

6.X.2.2    For the UE

For single UL non-simultaneous UE, the 2nd and 3rd harmonic analysis for B39 and B41 is done in the 2DL/1UL TR 36.851. None of the products falls into the B39 or B41 UE receiving bands.  For UE coexistence the single UL is already considered for these bands in TS 36.101.

<End of change>