# 3GPP TSG-RAN WG4 Meeting #78                                                            	R4-160545

St. Julian’s, Malta, 15 Feb-19 Feb, 2016

Title: TP for TR 36.714-05-01 on operating bands, channel bandwidths per operating band and co-existence analysis for 5DL CA_8B-41D

Source: 	CMCC

Agenda Item:	8.5.1

Document for:	Approval

1.	Introduction

The WI proposal to support the LTE Advanced Carrier Aggregation (5DL/1UL) of Band 8, Band 8, Band 41, Band 41 and Band 41 was approved in RAN#70.

In this contribution, CA operating bands, bandwidths configuration and co-existence analysis are proposed for LTE Advanced Carrier Aggregation (5DL/1UL) of Band 8, Band 8, Band 41, Band 41 and Band 41.

2.	Discussion

2.1 Channel bandwidths per operating band for CA

This contribution provides a text proposal to include the Channel bandwidths per operating band and Co-existence analysis for this work item into the TR 36.714-05-01. It can be define as in following tables.

Table 2-1-1: 5DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8-41 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 2-1-2: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| CA configuration | Operating bands | Channel bandwidth(MHz) |  |  |  |  |  | Bandwidth Combination Set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 | 3 | 5 | 10 | 15 | 20 |  |  |
| CA_8B-41D | 8 | See table 2-1-3 below |  |  |  |  |  | 0 | 80 |
|  | 41 | See CA_41D in Table 5.6A.1-1 in TS 36.101 of Bandwidth Combination Set 0 |  |  |  |  |  |  |  |

Table 2-1-3 Bandwidth combination sets defined for CA_8B

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_8B | 5, 10 | 10 | 20 | 0 |
|  | 10 | 5 |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

2.2     Co-existence analysis

2.2.1     For the BS

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting the 5DL CA configurations of CA_8B_41D can be calculated as shown in Table 2-2-1-1 and Table 2-2-1-2.

Table 2-2-1-1: DL harmonics frequency limits for 5DL CA of Band 8, Band 8, Band41, Band41 and      Band 41

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 925 | 960 | 2496 | 2690 |
| 2nd order harmonics frequency range (MHz) | 1850 to 1920 |  | 4992 to 5380 |  |
| 3rd order harmonics frequency range (MHz) | 2775 to 2880 |  | 7488 to 8070 |  |

As shown in Table 2-2-1-1, second harmonics may fall to UL frequencies of Band 2, 25, 33, 35, 37 or 39.

# Table 2-2-1-2: DL IMD products frequency limits for 5DL CA of Band 8, Band 8, Band41, Band41 and Band 41

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 925 | 960 | 2496 | 2690 |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 1536 to 1765 |  | 3421 to 3650 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2* f2_high – f1_low |
| IMD frequency range (MHz) | 576 to 840 |  | 4032 to 4455 |  |
| Two-tone 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency range (MHz) | 4346 to 4610 |  | 5917 to 6340 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 865 to 1020 |  | 2476 to 2710 |  |

As shown in Table 2-2-1-2, second order intermodulation products may fall to UL frequencies of Band 3, 4, 9, 10, 22, 24, 42 or 43. Third intermodulation products may fall to UL frequencies of Band 5, 6, 7, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 38 or 44. It is suggested BS transmitters supporting CA of Band 8, Band 8, Band 41, Band 41 and Band 41 should not share the same antenna with Band 2, 3, 4, 5, 6, 7, 9, 10, 12, 13, 14, 17, 18, 19, 20, 22, 24, 25, 26, 27, 28, 33, 35, 37, 38, 39, 42, 43 or 44 BS receiver, unless the antenna path meets very stringent third order PIM specification so that the PIM will not cause Band 2, 3, 4, 5, 6, 7, 9, 10, 12, 13, 14, 17, 18, 19, 20, 22, 24, 25, 26, 27, 28, 33, 35, 37, 38, 39,  42, 43 or 44 BS receiver desensitization.

2.2.2	For the UE

As shown in the table 2-2-2-1, the 3rd harmonic of band 8 will fall into DL of band 41.

# Table 2-2-2-1: Impact of UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 |
| 41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |

However, no harmonic issues were found due to operators’ spectrum holdings. Therefore, it is not necessary to introduce harmonic trap filter for this specific combination.

3 Text Proposal

<Start of change>

1	Scope

The present document is a technical report for 5DL/1UL Inter-band Carrier Aggregation under Rel-14 time frame according to the New CA WI approach in Release 14 [1]. The purpose is to gather the relevant background information and studies in order to address 5DL/1UL Inter-band Carrier Aggregation requirements for the Rel-14 band combinations in Table 1-1.

Table 1-1: Release 14 5DL/1UL inter-band carrier aggregation combinations

| CA combination | REL independent from |
| --- | --- |
| CA_8B-41D | 12 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<Next change>

6           5 Band Carrier Aggregation with Single UL: Specific Band Combination Part

### 6.X	CA_8B-41D_BCS0

6.X.1	Channel bandwidths per operating band for CA

Table 6.X.1-1: 5DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8-41 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 6.X.1-2: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| CA configuration | Operating bands | Channel bandwidth(MHz) |  |  |  |  |  | Bandwidth Combination Set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 | 3 | 5 | 10 | 15 | 20 |  |  |
| CA_8B-41D | 8 | See table 6.X.1-3 below |  |  |  |  |  | 0 | 80 |
|  | 41 | See CA_41D in Table 5.6A.1-1 in TS 36.101 of Bandwidth Combination Set 0 |  |  |  |  |  |  |  |

Table 6.X.1-3 Bandwidth combination sets defined for CA_8B

|  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| CA_8B | 5, 10 | 10 | 20 | 0 |
|  | 10 | 5 |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

6.X.2	Co-existence studies

6.X.2.1    For the BS

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting the 5DL CA configurations of CA_8B_41D can be calculated as shown in Table 6.X.2.1-1 and Table 6.X.2.1-2.

Table 6.X.2.1-1: DL harmonics frequency limits for 5DL CA of Band 8, Band 8, Band41, Band 41 and Band 41

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 925 | 960 | 2496 | 2690 |
| 2nd order harmonics frequency range (MHz) | 1850 to 1920 |  | 4992 to 5380 |  |
| 3rd order harmonics frequency range (MHz) | 2775 to 2880 |  | 7488 to 8070 |  |

As shown in Table 6.X.2.1-1, second harmonics may fall to UL frequencies of Band 2, 25, 33, 35, 37 or 39.

# Table 6.X.2.1-2: DL IMD products frequency limits for 5DL CA of Band 8, Band 8, Band 41,Band41 and Band 41

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 925 | 960 | 2496 | 2690 |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 1536 to 1765 |  | 3421 to 3650 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2* f2_high – f1_low |
| IMD frequency range (MHz) | 576 to 840 |  | 4032 to 4455 |  |
| Two-tone 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency range (MHz) | 4346 to 4610 |  | 5917 to 6340 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 865 to 1020 |  | 2476 to 2710 |  |

As shown in Table 6.X.2.1-2, second order intermodulation products may fall to UL frequencies of Band 3, 4, 9, 10, 22, 24, 42 or 43. Third intermodulation products may fall to UL frequencies of Band 5, 6, 7,  12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 38 or 44. It is suggested BS transmitters supporting CA of Band 8, Band8, Band 41, Band 41 and Band 41 should not share the same antenna with Band 2, 3, 4, 5, 6, 7, 9, 10, 12, 13, 14, 17, 18, 19, 20, 22, 24, 25, 26, 27, 28, 33, 35, 37, 38, 39, 42, 43 or 44 BS receiver, unless the antenna path meets very stringent third order PIM specification so that the PIM will not cause Band 2, 3, 4, 5, 6, 7, 9, 10, 12, 13, 14, 17, 18, 19, 20, 22, 24, 25, 26, 27, 28, 33, 35, 37, 38, 39,  42, 43 or 44 BS receiver desensitization.

6.X.2.2	For the UE

As shown in the table 6.X.2.2-1, the 3rd harmonic of band 8 will fall into DL of band 41.

# Table 6.X.2.2-1: Impact of UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 |
| 41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 |

However, no harmonic issues were found due to operators’ spectrum holdings. Therefore, it is not necessary to introduce harmonic trap filter for this specific combination.

<End of change>