# 3GPP TSG-RAN WG4 Meeting #78                                                            	R4-160553

St. Julian’s, Malta, 15 Feb-19 Feb, 2016

Title: TP for TR 36.714-02-02 on operating bands, channel bandwidths per operating band and co-existence analysis for dual uplink of CA_8A-41A

Source: 	CMCC

Agenda Item:	8.6.1

Document for:	Approval

1.	Introduction

The WI proposal to support the LTE Advanced 2 Band Carrier Aggregation (2DL/2UL) of Band 8 and Band 41 was approved in RAN#70.

In this contribution, CA operating bands, bandwidths configuration and co-existence analysis are proposed for LTE Advanced 2 Band Carrier Aggregation (2DL/2UL) of Band 8 and Band 41.

2.	Discussion

2.1 Channel bandwidths per operating band for CA

This contribution provides a text proposal to include the Channel bandwidths per operating band and Co-existence analysis for this work item into the TR 36.714-02-02. It can be defined as in following tables.

Table 2-1-1:  Inter-band CA operating bands for CA_8-41

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8-41 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Table 2-1-2: E-UTRA CA configuration and bandwidth combination sets for 2UL CA_8A-41A

| CA configuration | Operating bands | Channel bandwidth(MHz) |  |  |  |  |  | Bandwidth Combination Set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 | 3 | 5 | 10 | 15 | 20 |  |  |
| CA_8A-41A | 8 | Yes | Yes | Yes | Yes |  |  | 0 | 30 |
|  | 41 |  |  |  | Yes |  | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

2.2     Harmonics and IMD study

Table 2-2-1 lists B8+B41 2UL CA 2nd and 3rd order harmonics and 3rd order IMD for the UE-to-UE coexistence analysis.

Table 2-2-1: Band 8 and Band 41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 2496 | 2690 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  |  | 4992 – 5380 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 – 2745 |  |  | 7488 – 8070 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1581 – 1810 |  |  | 3376 – 3605 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 666–930 |  |  | 4077–4500 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4256 – 4520 |  |  | 5872 – 6295 |  |  |

Table 2-2-2: Band 8 and Band 41 UL IMD4 products with difference of frequency components

| ¦3*fx - 1*fy¦ | 50 to 249 MHz |
| --- | --- |
| ¦3*fy - 1*fx¦ | 6573 to 7190 MHz |
| ¦2*fx - 2*fy¦ | 3162 to 3620 MHz |

Frequency limits for 5th order IMD are presented in Table 2-2-3. IMD5 of CA_8A-41A may fall into Band 41 or Band 8 receiving band.

Table 2-2-3: CA_8A-41A 5th order UL IMD product frequency ranges

| ¦2*fx - 3*fy¦ | 5658 – 6310 |
| --- | --- |
| ¦2*fx + 3*fy¦ | 9318 – 9830 |
| ¦2*fy - 3*fx¦ | 2247– 2740 |
| ¦2*fy + 3*fx¦ | 7737 – 8020 |
| ¦fx - 4*fy¦ | 9069 – 9880 |
| ¦fx + 4*fy¦ | 10899 – 11640 |
| ¦fy – 4*fx¦ | 830 –1164 |
| ¦fy + 4*fx¦ | 6156 – 6210 |

When 2ULs inter-band CA UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 2-2-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 2-2-4: 2UL B8+ B41 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd order harmonics of B41,IMD3 and IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd order harmonics of B41 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd order harmonics of B41,IMD5 |

3 Text Proposal

<Start of change>

- Scope

The present document is a technical report for 2DL/2UL Inter-band Carrier Aggregation under Rel-14 time frame according to the New CA WI approach in Release 14 [1]. The purpose is to gather the relevant background information and studies in order to address 2DL/2UL Inter-band Carrier Aggregation requirements for the Rel-14 band combinations in Table 1-1.

Table 1-1: Release 14 2DL/2UL inter-band carrier aggregation combinations

| CA combination | REL independent from |
| --- | --- |
| CA_8A-41A | REL-12 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<Next change>

6          Dual Uplink Inter-band Carrier Aggregation: Specific Band Combination Part

### 6.X	CA_8A-41A

6.X.1	Operating bands for CA

Table 6.X.1-1: Inter-band CA operating bands for CA_8-41

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_8-41 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: E-UTRA CA configuration and bandwidth combination sets for 2UL CA_8A-41A

| CA configuration | Operating bands | Channel bandwidth(MHz) |  |  |  |  |  | Bandwidth Combination Set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 | 3 | 5 | 10 | 15 | 20 |  |  |
| CA_8A-41A | 8 | Yes | Yes | Yes | Yes |  |  | 0 | 30 |
|  | 41 |  |  |  | Yes |  | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

6.X.3	Co-existence studies

Table 6.X.3-1 lists B8+B41 2UL CA 2nd and 3rd order harmonics and 3rd order IMD for the UE-to-UE coexistence analysis.

Table 6.X.3-1: Band 8 and Band 41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 |  | 2496 | 2690 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  |  | 4992 – 5380 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 – 2745 |  |  | 7488 – 8070 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1581 – 1810 |  |  | 3376 – 3605 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 666–930 |  |  | 4077–4500 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4256 – 4520 |  |  | 5872 – 6295 |  |  |

Table 6.X.3-2: Band 8 and Band 41 UL IMD4 products with difference of frequency components

| ¦3*fx - 1*fy¦ | 50 to 249 MHz |
| --- | --- |
| ¦3*fy - 1*fx¦ | 6573 to 7190 MHz |
| ¦2*fx - 2*fy¦ | 3162 to 3620 MHz |

Frequency limits for 5th order IMD are presented in Table 6.X.3-3. IMD5 of CA_8A-41A may fall into Band 41 or Band 8 receiving band.

Table 6.X.3-3: CA_8A-41A 5th order UL IMD product frequency ranges

| ¦2*fx - 3*fy¦ | 5658 – 6310 |
| --- | --- |
| ¦2*fx + 3*fy¦ | 9318 – 9830 |
| ¦2*fy - 3*fx¦ | 2247– 2740 |
| ¦2*fy + 3*fx¦ | 7737 – 8020 |
| ¦fx - 4*fy¦ | 9069 – 9880 |
| ¦fx + 4*fy¦ | 10899 – 11640 |
| ¦fy – 4*fx¦ | 830 –1164 |
| ¦fy + 4*fx¦ | 6156 – 6210 |

When 2ULs inter-band CA UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.X.3-4 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.X.3-4: 2UL B8+ B41 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD5 |
|  | 2400 | - | 2494 | Yes | Asia | IMD5 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd order harmonics of B41,IMD3 and IMD5 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd order harmonics of B41 |
|  | 5470 | - | 5725 | Yes |  | IMD5 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd order harmonics of B41,IMD5 |

<End of change>