3GPP TSG-RAN WG4 Meeting #78	R4-160823

St. Julian’s, Malta, 15 – 19 February, 2016

Agenda Item:	6.2.3.3 OTA requirements

Source: 	SEI, NTT DOCOMO

Title:  	TP for TR: EIS test with Indoor Anechoic Chamber - Uncertainty list

Document for:	Approval

- Introduction

In RAN4#76 in Beijing, a framework for AAS BS conformance testing was approved [1] and it was agreed that multiple test methods may exist for each requirement. For the OTA test of EIS, so far there have been two test methods discussed, near field scanner method [2] and far field in compact antenna test range (CATR) [3].

Another test method for EIS, far field in indoor anechoic chamber was proposed in [4]. This paper discusses the uncertainty contribution factors of the test method and proposes a related TP for the TR [5].

- Discussion

The test procedure of the indoor anechoic chamber method has been proposed in [4] as a TP for the TR. This section discusses the uncertainty contribution factors introduced in each step.

Calibration:

  - Connect the transmitting antenna to the RF signal source generator.

The impedance mismatch in the transmitting chain would lead to measurement uncertainty. The transmitting chain includes the transmitting antenna, RF signal source generator, cable connecting the transmitting antenna and the RF signal source generator, and possibly some attenuators etc. The impedance mismatch may occur at any interconnection point.

  - Install the reference antenna with its beam peak direction and the height of its phase center aligned with the transmitting antenna, and connect the reference antenna to the measurement equipment

There are two factors that would lead to measurement uncertainty at this step.

- Positioning and pointing misalignment between the reference antenna and the transmitting antenna.

- Impedance mismatch at the reference antenna and measurement equipment.

  - Generate an RF signal with the same carrier frequency as the test signal for EIS measurement of AAS BS. The received power at the measurement equipment is denoted by PREIS_cal.

There are several factors that would lead to measurement uncertainty at this step:

- Quality of quiet zone

- Polarization mismatch for reference antenna

- Mutual coupling between the reference antenna and the transmitting antenna

- Phase curvature across the reference antenna.

- Uncertainty due to antenna mounting apparatus

- Uncertainty of the measurement equipment

  - The calibration value is calculated with the following formula:

LEIS_cal= PSEIS_cal+GSEIS_cal –LFEIS_cal –PREIS_cal

LEIS_cal: Calibration value

PSEIS_cal: Output power of the RF signal source generator

GSEIS_cal: Antenna gain of the reference antenna

LFEIS_cal: Cable loss between the reference antenna and the measurement equipment.

There are several factors that would lead to measurement uncertainty at this step.

- Uncertainty of the RF signal source generator

- Cable loss variation between the reference antenna and the measurement equipment

- Uncertainty of the absolute gain of the reference antenna

Measurement:

- Uninstall the reference antenna and install the AAS with its phase center in the same place as the reference antenna.

The factor that would lead to the measurement uncertainty at this step is positioning misalignment between the AAS BS and the reference antenna/transmitting antenna.

- Set the AAS BS to be satisfied with the sensitivity RoAoA covering conformance testing receiving direction.

- Rotate the AAS to make the beam peak direction of the transmitting antenna aligned with the conformance testing receiving direction.

The factor that would lead to the measurement uncertainty at this step is the pointing misalignment between the AAS BS and the transmitting antenna.

- For FDD AAS BS start BS transmission according to E-TM 1.1 at manufacturer’s declared rated output power.

- Set the test signal mean power at the RF signal source generator as the declared minimum EIS level plus LEIS_cal and the reference measurement channel as given in subclause 7.2.5 of TS36.141.

There are several factors that would lead to measurement uncertainty at this step.

- Quality of quiet zone

- Polarization mismatch between the AAS BS and the transmitting antenna

- Mutual coupling between the AAS BS and the transmitting antenna

- Phase curvature across the AAS BS.

- Uncertainty due to antenna mounting apparatus

- Uncertainty of the RF signal source generator

- Cable loss variation between the transmitting antenna and the RF signal source generator

- Uncertainty of the absolute gain of the transmitting antenna

- Measure the throughput according to Annex E of the TS36.141.

- Repeat the steps 6)~10) above per conformance testing direction.

- Conclusion

This paper discusses the uncertainty contribution factors of EIRP measurement with indoor anechoic chamber and proposes a related TP to the TR.

- Text Proposal:

TR37.842 v1.9.0

--------------Start of text proposal-------------

- 10.3.1.1.2.4  Uncertainty assessment

Table 10.3.1.1.2.4-1: Indoor Anechoic Chamber Uncertainty contributions

in AAS EIS test

| UID | Description of uncertainty contribution | Details in paragraph |
| --- | --- | --- |
| Stage 1, DUT measurement |  |  |
| 1 | Positioning misalignment between the AAS BS and the reference antenna/transmitting antenna | [tbd] |
| 2 | Pointing misalignment between the AAS BS and the transmitting antenna | [tbd] |
| 3 | Quality of quiet zone | [tbd] |
| 4 | Polarization mismatch between the AAS BS and the transmitting antenna | [tbd] |
| 5 | Mutual coupling between the AAS BS and the transmitting antenna | [tbd] |
| 6 | Phase curvature across the AAS BS | [tbd] |
| 7 | Uncertainty due to antenna mounting apparatus | [tbd] |
| 8 | Uncertainty of the RF signal source generator | [tbd] |
| 9 | Cable loss variation between the transmitting antenna and the RF signal source generator | [tbd] |
| 10 | Uncertainty of the absolute gain of the transmitting antenna | [tbd] |
|  |  | … |
| Stage 2, Calibration |  |  |
| 11 | The impedance mismatch in the transmitting chain | [tbd] |
| 12 | Positioning and pointing misalignment between the reference antenna and the transmitting antenna | [tbd] |
| 13 | Impedance Mismatch at the reference antenna and measurement equipment | [tbd] |
| 14 | Quality of quiet zone | [tbd] |
| 15 | Polarization mismatch for reference antenna | [tbd] |
| 16 | Mutual coupling between the reference antenna and the transmitting antenna | [tbd] |
| 17 | Phase curvature across the reference antenna | [tbd] |
| 18 | Uncertainty due to antenna mounting apparatus | [tbd] |
| 19 | Uncertainty of the measurement equipment | [tbd] |
| 20 | Uncertainty of the RF signal source generator | [tbd] |
| 21 | Cable loss variation between the reference antenna and the measurement equipment | [tbd] |
| 22 | Uncertainty of the absolute gain of the reference antenna | [tbd] |
| 23 | … | … |

--------------End of text proposal-------------

References

[1] R4-155022, WF on conformance testing framework, Huawei.

[2] R4-158059, TP to TR 37.842: Adding uncertainty contributors list for EIS measurement in Near Field Test Range, MVG Industries.

[3] R4-158312, TP for TR 37.842: Adding uncertainty list for EIS in CATR in Section 10, Ericsson, Nokia Networks.

[4] R4-160822, TP for TR: EIS test with indoor far-field anechoic chamber - Description and Procedure, SEI

[5] TR 37.842 v190, Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) (Release 13)