3GPP TSG-RAN WG4 Meeting #78	R4-161228

St. Julian’s, Malta, 15 – 19 February, 2016

Agenda item:			6.7.6

Source:	Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Ericsson, Huawei, Hisilicon

Title:	Cat-M PUCCH Simulation Assumptions

Document for:		Approval

# Introduction

The PUCCH performance requirements for supporting Cat-M UEs are discussed in [1]. Based on the discussion, we provide the link level simulation assumptions for defining Cat-M PUCCH performance requirements.

# Simulation Assumptions

Table 1 provides the simulation assumptions for Cat-M PUCCH requirements of

- ACK missed detection requirements for single user PUCCH format 1a

- CQI performance requirements for PUCCH format 2

For the simulation of the ACK missed detection requirements, the false alarm (DTXACK) rate shall not exceed 1% and the ACK missed detection (ACKNAK, DTX) probability shall not exceed 1%.

For the simulation of the CQI performance requirements, the CQI block error probability (BLER) is defined as the conditional probability of incorrectly decoding the CQI information when the CQI information is sent. All CQI information shall be decoded (no exclusion due to DTX). The CQI information bit payload per sub-frame is equal to 4 bits. The CQI block error probability shall not exceed 1% at the given SNR.

Table 1.	Cat-M PUCCH simulation assumptions

| Parameters | Values |
| --- | --- |
| PUCCH Format | PUCCH Format 1a, Format 2 |
| Propagation conditions | EPA5 (PUCCH Format 1a) EVA5 (Format 2) |
| Number of TX antennas | 1 |
| Number of RX antennas | 2,[4,8] (Format 1a) 2(Format 2) |
| Antenna correlation | Low |
| Noise model | AWGN |
| Channel bandwidth (MHz) | [1.4,] 3, 5, 10, 15, 20 |
| Number of PRBs for PUCCH | 1 (edge PRB of the channel bandwidth) |
| Cyclic prefix | Normal (PUCCH Format 1a, Format 2) |
| DM RS sequence group hopping | Disabled |
| Channel estimation | Practical channel estimation and noise estimation |
| Cyclic shift separation | 2 |
| Number of users | single user PUCCH format 1a and PUCCH format 2 |
| Payload per PUCCH transmission | 4 bits (PUCCH format 2) |
| Number of PUCCH repetitions | {4,8,32} |
| Frequency Hopping Intervals | FDD: 4 TDD: 5 |
| Note 1: Assume no scheduling request for PUCCH format 1a |  |

Table 2 provides a template for the simulation results for Cat-M PUCCH format 1a.

Table 2 Minimum requirements for single user PUCCH format 1a

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Repetitions | Channel Bandwidth / SNR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | [1.4 MHz] | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 1 | 2 | normal | EPA5 Low | 4 |  |  |  |  |  |  |
|  |  |  |  | 8 |  |  |  |  |  |  |
|  |  |  |  | 32 |  |  |  |  |  |  |
|  | 4 | normal | EPA5 Low | 4 |  |  |  |  |  |  |
|  |  |  |  | 8 |  |  |  |  |  |  |
|  |  |  |  | 32 |  |  |  |  |  |  |
|  | 8 | normal | EPA5 Low | 4 |  |  |  |  |  |  |
|  |  |  |  | 8 |  |  |  |  |  |  |
|  |  |  |  | 32 |  |  |  |  |  |  |

Table 3 Minimum requirements for single user PUCCH format 2

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Repetitions | Channel Bandwidth / SNR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | [1.4 MHz] | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 1 | 2 | normal | EVA5 Low | 4 |  |  |  |  |  |  |
|  |  |  |  | 8 |  |  |  |  |  |  |
|  |  |  |  | 32 |  |  |  |  |  |  |

# Summary

In this contribution, we provided the link level simulation assumptions for Cat-M PUCCH performance requirements.

# References

- R4-160271, Cat-M PUCCH performance requirement, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- 3GPP TS 36.104, E-UTRA: Base Station (BS) radio transmission and reception.

- R4- 131776, Simulation assumptions for UL 8 antenna receiver, China Telecom.