3GPP TSG-RAN WG4 Meeting #78bis	   					       R4-161531

San Jose Del Cabo, Mexico, 11 - 15 Apr, 2016

Source:	China Telecom

Title:	TP on BS MMSE-IRC receiver in asynchronous network

Agenda Item:	6.4.1.2

Document for:	Approval

Introduction

In the last meeting, it was agreed to specify BS MMSE-IRC demodulation requirements for asynchronous network operation [1]. This contribution provides a text proposal for TR 36.884 to add the text related to asynchronous network in clause 5.2 and clause 9. In addition, small editorial modifications for clause 8 are made.

Reference

[1]	R4-161169, China Telecom, Huawei, ZTE, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, “WF on BS MMSE-IRC receiver in asynchronous network,” RAN4 #78, Aug 2015.

Text Proposal

< Start of first change >

## 5.2	Targeted and interference channels

On performance requirements of MMSE-IRC receiver for PUSCH, it was agreed that:

- The first priority for phase I of this WI is SIMO PUSCH to SIMO PUSCH collision under synchronous network.

- BS MMSE-IRC demodulation requirements for both synchronous and asynchronous network operations are specified in phase II of this WI.

- For both target PUSCH and interference PUSCH, 1Tx UE scenario is prioritized and 2Tx UE scenario is not covered.

< End of first change >

< Start of second change >

# 8	BS demodulation performance requirements with synchronous interference

For PUSCH, enhanced demodulation performance requirements will be defined for verification of BS receiver’s ability on inter-cell interference suppression.

This clause provides the phase-II link level evaluation parameters and results in the scenario where the interferer(s) are time-synchronous with the wanted signal.

## 8.1	Parameters for link level evaluation

For deriving the enhanced demodulation requirements (clause 8 in 36.104) in phase II, generally the phase-I link level evaluation parameters in clause 7.1 can be re-used, with some exceptions / clarifications as follows:

- For uplink 2Rx cases, when the explicit interferer number is increased from 1 to 2, the additional MMSE-IRC gain over MMSE is not significant, thus it was decided to model only 1 explicit interferer for 2Rx cases. 2 explicit interferers are modeled for 4Rx and 8Rx cases.

- To check the per-PRB interference covariance estimation, it was decided to use ETU channel with high delay spread for the interferers. For cases with EPA5 and EVA70 as serving channel, ETU5 and ETU70 are respectively used as the interfering channel.

- In clause 7, 12 evaluation cases are introduced for synchronous network. To limit the test efforts, the number of synchronous network test case is reduced to 6 for each channel bandwidth.

- Different DM-RS base sequences are configured for the serving UE and interfering UEs, and both group hopping and sequence hopping are disabled.

The common parameters for phase-II link level evaluation are given in Table 8.1-1 and Table 8.1-2. The phase-II evaluation cases with specific parameters are given in Table 8.1-3.

Table 8.1-1: Common parameters of link level evaluation assumptions

| Parameters |  | Unit | Values |
| --- | --- | --- | --- |
| Cyclic prefix |  |  | Normal |
| Interference modelling | Number of explicitly modelled interferers |  | 1 or 2 depending on the cases |
|  | Interference modulation |  | 16QAM |
|  | Timing delay and frequency offset for synchronous case |  | Well aligned: no timing delay and frequency offset between the serving UE and interfering UEs |
| Frequency hopping, TTI bundling |  |  | Disable |
| HARQ combining |  |  | Incremental redundancy |
| Redundancy version sequence |  |  | 0, 2, 3, 1, 0, 2, 3, 1 |
| Maximal number of HARQ transmissions (including 1st transmission and re-transmissions) |  |  | 4 |
| Bandwidth |  |  | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz |
| Resource allocation |  |  | Full PRB allocation |
| DM-RS configuration |  |  | The serving UE, interfering UE 1 and interfering UE 2 are served by cells with cell id #0, 1, 2 respectively. For the wanted PUSCH and interfering PUSCHs, =0, =0 and =0. |

Table 8.1-2: AWGN power level

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 1.4 | -92.7dBm / 1.08MHz |
| 3 | -88.7dBm / 2.7MHz |
| 5 | -86.5dBm / 4.5MHz |
| 10 | -83.5dBm / 9MHz |
| 15 | -81.7dBm / 13.5MHz |
| 20 | -80.4dBm / 18MHz |

Table 8.1-3: Cases for link level evaluations

| Case Num | MCS | Propagation condition (Serving, interferers) | Antenna configuration for serving and interferers | (DIP1, DIP2) dB |
| --- | --- | --- | --- | --- |
| 1 | 6 | (EPA5, ETU5) | 1x2 Low | (-0.43, N/A) |
| 2 | 15 | (EPA5, ETU5) | 1x4 Low | (-0.43, -13.78) |
| 3 | 20 | (EPA5, ETU5) | 1x8 Low | (-0.43, -13.78) |
| 4 | 6 | (EVA70, ETU70) | 1x2 Low | (-1.11, N/A) |
| 5 | 15 | (EVA70, ETU70) | 1x4 Low | (-1.11, -10.91) |
| 6 | 20 | (EVA70, ETU70) | 1x8 Low | (-1.11, -10.91) |

< End of second change >

< Start of third change: to be inserted after clause 8>

# 9	BS demodulation performance requirements with asynchronous interference

This clause provides the phase-II link level evaluation parameters and results in the scenario where the interferer(s) are time-asynchronous with the wanted signal.

## 9.1	Parameters for link level evaluation

For the link level evaluation parameters in asynchronous interference scenario:

- Two interfering UEs from the same interfering cell are explicitly modeled, and the two interferers are configured to transmit respectively in the even subframes and odd subframes. The propagation conditions of the two interferers are statistically independent, so that the BS MMSE-IRC receiver needs to suppress the interference from two different spatial directions.

- To limit the test efforts, the number of asynchronous network test case is reduced to 3 for each channel bandwidth.

The common parameters for phase-II link level evaluation with asynchronous interference are given in Table 9.1-1 and Table 9.1-2. The configuration of the asynchronous interferers is illustrated in Figure 9.1-1. The phase-II evaluation cases with specific parameters are given in Table 9.1-3.

Table 9.1-1: Common parameters of link level evaluation assumptions

| Parameters |  | Unit | Values |
| --- | --- | --- | --- |
| Cyclic prefix |  |  | Normal |
| Interference modelling | Number of explicitly modelled interferers |  | Two interferers, named interferer 1-1 and interferer 1-2, are connected to the same cell and configured to transmit respectively in the even subframes and odd subframes. |
|  | Interference modulation |  | 16QAM |
|  | Timing delay and frequency offset for asynchronous case |  | The transmissions of both interferer 1-1 and interferer 1-2 are delayed with respect to the wanted signal by 0.33 ms. No frequency offset between the wanted signal and interferers. |
| Frequency hopping, TTI bundling |  |  | Disable |
| HARQ combining |  |  | Incremental redundancy |
| Redundancy version sequence |  |  | 0, 2, 3, 1, 0, 2, 3, 1 |
| Maximal number of HARQ transmissions (including 1st transmission and re-transmissions) |  |  | 4 |
| Bandwidth |  |  | 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz |
| Resource allocation |  |  | Full PRB allocation |
| DM-RS configuration |  |  | The serving UE is served by cell with cell id #0, and interferer 1-1 and interferer 1-2 are served by cell with cell id #1. For the wanted PUSCH and interfering PUSCHs, =0, =0 and =0. |
| Interference estimation |  |  | Interference covariance matrix estimation should be conducted per PRB and per TTI. |

Table 9.1-2: AWGN power level

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 1.4 | -92.7dBm / 1.08MHz |
| 3 | -88.7dBm / 2.7MHz |
| 5 | -86.5dBm / 4.5MHz |
| 10 | -83.5dBm / 9MHz |
| 15 | -81.7dBm / 13.5MHz |
| 20 | -80.4dBm / 18MHz |

Figure 9.1-1: Configuration of asynchronous interferers

Table 9.1-3: Cases for link level evaluations

| Case Num | MCS | Propagation condition (Serving, interferers) | Antenna configuration for serving and interferers | (DIP1-1, DIP1-2) dB |
| --- | --- | --- | --- | --- |
| 1 | 6 | (EPA5, ETU5) | 1x2 Low | (-0.43, -0.43) |
| 2 | 15 | (EPA5, ETU5) | 1x4 Low | (-0.43, -0.43) |
| 3 | 20 | (EPA5, ETU5) | 1x8 Low | (-0.43, -0.43) |
| Note: The propagation conditions for the serving UE, interferer 1-1 and interferer 1-2 are statistically independent. |  |  |  |  |

## 9.2	Link level simulation results

<Text will be added>

## 9.3	Demodulation performance requirements

<Text will be added>

# 10	Conclusions

< End of third change >