3GPP TSG-RAN WG4 Meeting #78Bis	R4-161553

San Jose del Cabo, Mexico, 11 – 15 April, 2016

Agenda Item:	8.1.2

Source: 	Samsung

Title: 	ACLR Simulation Results for HPUE Coexistence Study

Document for:	Discussion

# Introduction

The study item on New Band 41 UE power class supporting +26 dBm was approved at RAN #68. In RAN4 #76bis meeting, the system level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm was approved in [1]. The power control (PC) settings are further discussed in RAN4 #77 meeting and the modified PC parameters are approved in [2].

In RAN4 #78, the following agreement has been reached, which has been captured in the WF [3]:

- In parallel for the next meeting, companies are requested to

  - Adopt 1 dB ACLR tightening as one option as the basis for continuing work into the next meeting on MPR/A-MPR determination.

  - Companies to provide system coexistence simulation results based on expanded cell in aggressor network vs. conventional cell size in victim network. China Telecom to provide specific simulation parameters by 03/15/16.

  - Companies are encouraged to provide simulation results with more aggressive power control parameters for information.

In the following RAN4 Email discussion, China Telecom has provided the detailed simulation parameters for two scenarios indicated in the above WF to be included in TR36.886 for informational purposes [4].

In this contribution, we provide the system level simulation results for coexistence study on new Band 41 HPUE for Scenario A2 proposed by China Telecom.

# Simulation assumptions

For Scenario A2, the key assumptions for this high power UE coexistence simulation are provided for path loss model, inter site distance (ISD) and power control (PC) settings. The macro cell propagation model should follow the model in section 5.6.1.1 of the HPUE TR 36.886 v0.3.0 [5]. The PC settings are provided as follows [4].

Table 1: Power control parameters for +23 dBm UE, Scenario A2

(a) Parameters for +23 dBm UE with 0.75 km ISD

| Parameter set | Gamma | CLx-ile |
| --- | --- | --- |
|  |  | 20 MHz bandwidth |
| Set 4A | 1 | 107 |
| Set 4B | 1 | 103 |
| Set 4C | 1 | 101 |

(b) Parameters for +23 dBm UE with 2.8 km ISD

| Parameter set | Gamma | CLx-ile |
| --- | --- | --- |
|  |  | 20 MHz bandwidth |
| Set 4A | 1 | 128 |
| Set 4B | 1 | 125 |
| Set 4C | 1 | 123 |

(c) Parameters for +23 dBm UE with 6 km ISD

| Parameter set | Gamma | CLx-ile |
| --- | --- | --- |
|  |  | 20 MHz bandwidth |
| Set 4A | 1 | 112 |
| Set 4B | 1 | 108 |
| Set 4C | 1 | 106 |

(d) Parameters for +23 dBm UE with 8 km ISD

| Parameter set | Gamma | CLx-ile |
| --- | --- | --- |
|  |  | 20 MHz bandwidth |
| Set 4A | 1 | 116 |
| Set 4B | 1 | 112 |
| Set 4C | 1 | 110 |

Table 2: Power control parameters for +26 dBm UE, Scenario A2

(a) Parameters for +26 dBm UE with 0.75 km ISD

| Parameter set | Gamma | CLx-ile |
| --- | --- | --- |
|  |  | 20 MHz bandwidth |
| Set 4A | 1 | 110 |
| Set 4B | 1 | 106 |
| Set 4C | 1 | 104 |

(b) Parameters for +26 dBm UE with 2.8 km ISD

| Parameter set | Gamma | CLx-ile |
| --- | --- | --- |
|  |  | 20 MHz bandwidth |
| Set 4A | 1 | 131 |
| Set 4B | 1 | 128 |
| Set 4C | 1 | 126 |

(c) Parameters for +26 dBm UE with 6 km ISD

| Parameter set | Gamma | CLx-ile |
| --- | --- | --- |
|  |  | 20 MHz bandwidth |
| Set 4A | 1 | 115 |
| Set 4B | 1 | 111 |
| Set 4C | 1 | 109 |

(d) Parameters for +26 dBm UE with 8 km ISD

| Parameter set | Gamma | CLx-ile |
| --- | --- | --- |
|  |  | 20 MHz bandwidth |
| Set 4A | 1 | 119 |
| Set 4B | 1 | 115 |
| Set 4C | 1 | 113 |

The cell layout in section 5.6.1.3 of the HPUE TR 36.886 v0.3.0 [4] is followed, and inter-site distances are copied in Table 3 below.

Table 3: Inter-site distances and Propagation model

| Environment | ISD of aggressor/victim system (km) |
| --- | --- |
| Urban | .75 |
| Suburban | 2.8 |
| Rural | 6 |
| Rural | 8 |

Other simulation parameters and assumptions are specified in [4].

# Simulation results

## Results for Scenario A2

The simulation results for Scenario A2 are summarized as below table. The detailed results for transmission power CDF curves can be found in the Appendix of this contribution.

Table 4. Summarized simulation results for additional CLx-ile settings

| PC Set | ISD | Gamma | CLx-ile (23dBm/ 26dBm) | Percentage of Ptx > 23dBm | Average loss |  |  |  |  |  |  | 5%-tile loss |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 23dBm X=0dB (baseline) | 26dBm X=0dB | 26dBm X=1dB | 26dBm X=2dB | 26dBm X=3dB | 26dBm X=4dB | 26dBm X=5dB | 23dBm X=0dB (baseline) | 26dBm X=0dB | 26dBm X=1dB | 26dBm X=2dB | 26dBm X=3dB | 26dBm X=4dB | 26dBm X=5dB |
| PC4A | 750m | 1 | 107/110 | 6.58% | 1.83% | 2.09% | 1.70% | 1.36% | 1.06% | 0.84% | 0.67% | 1.32% | 1.42% | 1.11% | 0.92% | 0.74% | 0.63% | 0.54% |
|  | 2.8km | 1 | 128/131 | 7.48% | 1.73% | 1.83% | 1.58% | 1.36% | 1.16% | 1.00% | 0.85% | 6.95% | 7.46% | 6.06% | 5.05% | 4.20% | 3.35% | 2.64% |
|  | 6km | 1 | 112/115 | 7.47% | 1.00% | 1.12% | 0.91% | 0.74% | 0.61% | 0.51% | 0.41% | 0.49% | 0.55% | 0.45% | 0.31% | 0.23% | 0.17% | 0.14% |
|  | 8km | 1 | 116/119 | 7.79% | 1.48% | 1.65% | 1.33% | 1.06% | 0.84% | 0.70% | 0.56% | 0.92% | 1.00% | 0.71% | 0.54% | 0.43% | 0.33% | 0.26% |
| PC4B | 750m | 1 | 103/106 | 13.87% | 1.27% | 1.58% | 1.26% | 0.97% | 0.76% | 0.60% | 0.48% | 1.34% | 1.58% | 1.28% | 1.05% | 0.88% | 0.60% | 0.41% |
|  | 2.8km | 1 | 125/128 | 12.90% | 2.04% | 2.22% | 1.93% | 1.66% | 1.43% | 1.23% | 1.05% | 10.77% | 11.73% | 10.39% | 8.42% | 7.02% | 5.64% | 4.91% |
|  | 6km | 1 | 108/111 | 15.48% | 0.77% | 0.92% | 0.74% | 0.60% | 0.48% | 0.37% | 0.29% | 0.48% | 0.56% | 0.44% | 0.37% | 0.32% | 0.25% | 0.18% |
|  | 8km | 1 | 112/115 | 15.93% | 1.01% | 1.21% | 0.98% | 0.80% | 0.65% | 0.53% | 0.42% | 0.75% | 1.05% | 0.86% | 0.65% | 0.52% | 0.44% | 0.32% |
| PC4C | 750m | 1 | 101/104 | 18.95% | 1.03% | 1.33% | 1.06% | 0.86% | 0.69% | 0.54% | 0.42% | 1.46% | 2.06% | 1.55% | 1.20% | 0.94% | 0.74% | 0.62% |
|  | 2.8km | 1 | 123/126 | 17.55% | 5.14% | 5.64% | 4.97% | 4.38% | 3.82% | 3.34% | 2.91% | 10.61% | 11.95% | 9.41% | 7.67% | 6.59% | 5.62% | 4.40% |
|  | 6km | 1 | 106/109 | 20.85% | 0.63% | 0.78% | 0.62% | 0.50% | 0.39% | 0.30% | 0.23% | 0.76% | 0.92% | 0.75% | 0.63% | 0.47% | 0.33% | 0.27% |
|  | 8km | 1 | 110/113 | 21.43% | 0.87% | 1.06% | 0.87% | 0.70% | 0.56% | 0.45% | 0.36% | 1.11% | 1.47% | 1.19% | 0.96% | 0.85% | 0.67% | 0.49% |

Based on the additional simulation results, it can be observed:

- For average throughput loss, 1 dB of additional ACLR is enough to guarantee the same level of performance degradation, at least for PC set 4A and 4B. For PC set 4C, 1 dB of additional ACLR is enough for all ISD cases expect ISD=750m.

- For 5%-tile throughput loss, around 1 dB of additional ACLR is enough to guarantee the same level of performance degradation, at least for PC set 4A and 4B (except the case of ISD=8km and PC set B), while at most 2 dB of additional ACLR is needed for PC set 4C.

# Conclusion

In this contribution we presented simulation results showing adjacent channel coexistence performance for High Power UE operating in Band 41. Based on the simulation results, one observation is made.

Observation 1: Less than 1 dB of additional ACLR is needed for Band 41 UE supporting +26 dBm power class.

# Reference

- R4-156846, “Way Forward on System level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm”, Sprint, Alcatel-Lucent.

- R4-158342, “HPUE simulation anomalies”, Sprint, Intel, CMCC, Qualcomm, Ericsson, Alcatel-Lucent.

- R4-161488, Band 41 HPUE Way Forward, Qualcomm Incorporated, Sprint, China Telecom, China Unicom, CMCC, RAN4 #78, Feb 2016.

- RAN4 Email Discussion, “[B41 HPUE] Additional simulation assumptions for coexistence study”.

- R4-160464, HPUE TR 36.886 v0.3.0, Sprint, RAN4 #78, Feb 2016.

# Annex: detailed simulation results

## Urban: ISD = 750m and 20MHz

| Figure 1(a). UE Tx power for PC set 4A | Figure 1(b). UE Tx power for PC set 4B |
| --- | --- |
| Figure 1(c). UE Tx power for PC set 4C |  |

Figure 1. UE Tx Power CDF for [750m ISD, 20MHz].

## Suburban: ISD = 2.8km and 20MHz

| Figure 2(a). UE Tx power for PC set 4A | Figure 2(b). UE Tx power for PC set 4B |
| --- | --- |
| Figure 2(c). UE Tx power for PC set 4C |  |

Figure 2. UE Tx Power CDF for [2.8km ISD, 20MHz].

## Rural: ISD = 6km and 20MHz

| Figure 3(a). UE Tx power for PC set 4A | Figure 3(b). UE Tx power for PC set 4B |
| --- | --- |
| Figure 3(c). UE Tx power for PC set 4C |  |

Figure 3. UE Tx Power CDF for [6km ISD, 20MHz].

## Rural: ISD = 8km and 20MHz

| Figure 4(a). UE Tx power for PC set 4A | Figure 4(b). UE Tx power for PC set 4B |
| --- | --- |
| Figure 4(c). UE Tx power for PC set 4C |  |

Figure 4. UE Tx Power CDF for [8km ISD, 20MHz].