3GPP TSG-RAN WG4 Meeting #78bis	R4-161592

San Jose Del Cabo, Mexico, 11 – 15 April, 2016

Agenda Item:	7.4.2

Source: 	Nokia, U.S. Cellular

Title: 	Required BS studies of Harmonics and Intermodulation Products caused by LTE Advanced 4DL/1UL Carrier Aggregation with Bands 2, 4, 5 and 12A (RAN#71)

Document for:	Discussion

# 1.	Introduction

The basket WI proposal including support of the LTE Advanced 4DL/1UL Carrier Aggregation (CA) of 5 band combinations with Bands 2, 4, 5 and 12A were approved in RAN#71 [1]. One objective of the WIs is to specify the band-combination specific Radio Frequency (RF) requirements for the new inter-band CA scenarios. Base Station (BS) studies required to be performed on the new inter-band CA scenarios are the impacts of harmonics and InterModulation Distortion (IMD) products caused by LTE-A BS supporting 4DL CA configurations to the receiver of own or different BS.

In this contribution, we provide an analysis on the required BS studies of harmonics and IMD products for the 5 band combinations with Bands 2, 4, 5 and 12A approved in RAN#71 on LTE-A 4DL/1UL CA.

# 2.	Discussion

As we discussed and agreed in the past [2], no additional BS study of 2nd and 3rd harmonics and IMD products will be required for a 4DL CA configuration, if it does not use a larger carrier channel bandwidth or aggregated channel bandwidth in any of the CA bands when compared to the constituent 3 DL CA configurations, and the BS studies of 2nd and 3rd harmonics and IMD products for all the constituent 3 DL CA configurations have been performed.

In the following, we provide an overview on the required BS studies of harmonics and IMD products for the 5 band combinations with Bands 2, 4, 5 and 12A approved in RAN#71 on LTE-A 4DL/1UL CA, considering the BS studies that have been performed for the constituent 3 DL CA configurations.

## 2.1	CA_2A-2A-12A-12A

The CA scenario is provided in Table 1 below [1]:

Table 1 – Interband 4DL/1UL CA Scenario

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_2A-2A-12A-12A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS 36.101 |  |  |  |  |  | 50 | 0 |
|  | 12 |  |  | Yes |  |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |

For the constituent 3 DL CA configurations:

1.	CA_2A-2A-12A is already specified with the same CC channel bandwidth in each CA band.

2.	CA_2A-12A-12A is already analyzed with the same CC channel bandwidth in each CA band [3].

Therefore, no additional BS study of harmonics and IMD products is required for this band combination.

## 2.2	CA_2A-4A-12A-12A

The CA scenario is provided in Table 2 below [1]:

Table 2 – Interband 4DL/1UL CA Scenario

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_2A-4A-12A-12A | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |

For the constituent 3 DL CA configurations:

1.	CA_2A-4A-12A is already specified with the same CC channel bandwidth in each CA band.

2.	CA_2A-12A-12A is already analyzed with the same CC channel bandwidth in each CA band [3].

3.	CA_4A-12A-12A is already analyzed with the same CC channel bandwidth in each CA band [4].

Therefore, no additional BS study of harmonics and IMD products is required for this band combination.

## 2.3	CA_2A-5A-12A-12A

The CA scenario is provided in Table 3 below [1]:

Table 3 – Interband 4DL/1UL CA Scenario

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_2A-5A-12A-12A | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |

For the constituent 3 DL CA configurations:

1.	CA_2A-5A-12A is already specified with the same CC channel bandwidth in each CA band.

2.	CA_2A-12A-12A is already analyzed with the same CC channel bandwidth in each CA band [3].

3.	CA_5A-12A-12A is already analyzed with the same CC channel bandwidth in each CA band [5].

Therefore, no additional BS study of harmonics and IMD products is required for this band combination.

## 2.4	CA_4A-4A-12A-12A

The CA scenario is provided in Table 4 below [1]:

Table 4 – Interband 4DL/1UL CA Scenario

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_4A-4A-12A-12A | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS 36.101 |  |  |  |  |  | 50 | 0 |
|  | 12 |  |  | Yes |  |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |

For the constituent 3 DL CA configurations:

1.	CA_4A-4A-12A is already specified with the same CC channel bandwidth in each CA band.

2.	CA_4A-12A-12A is already analyzed with the same CC channel bandwidth in each CA band [4].

Therefore, no additional BS study of harmonics and IMD products is required for this band combination.

## 2.5	CA_4A-5A-12A-12A

The CA scenario is provided in Table 5 below [1]:

Table 5 – Interband 4DL/1UL CA Scenario

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_4A-5A-12A-12A | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 5 |  |  | Yes | Yes |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |
|  | 12 |  |  | Yes |  |  |  |  |  |

For the constituent 3 DL CA configurations:

1.	CA_4A-5A-12A is already specified with the same CC channel bandwidth in each CA band.

2.	CA_4A-12A-12A is already analyzed with the same CC channel bandwidth in each CA band [4].

3.	CA_5A-12A-12A is already analyzed with the same CC channel bandwidth in each CA band [5].

Therefore, no additional BS study of harmonics and IMD products is required for this band combination.

# 3.	Conclusions

In this contribution, we have provided an overview on the required BS studies of harmonics and IMD products for the 5 band combinations with Bands 2, 4, 5 and 12A approved in RAN#71 on LTE-A 4DL/1UL CA, considering the BS studies that have been performed for the constituent 3 DL CA configurations.

As a summary, no additional BS study of harmonics and IMD products is required for these band combinations.

# References

[1]	RP-160614, “Revised WID: LTE Advanced inter-band CA Rel-14 for 4DL/1UL”, Ericsson.

[2]	R4-151293, “TP for TR 36.854-13: Required BS studies of Harmonics and Intermodulation Products caused by LTE Advanced 4DL/1UL Carrier Aggregation”, Alcatel-Lucent.

[3]	R4-156399, “TP for TR 36.853-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 12 + 12)”, Alcatel-Lucent, U.S. Cellular.

[4]	R4-156400, “TP for TR 36.853-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (4 + 12 + 12)”, Alcatel-Lucent, U.S. Cellular.

[5]	R4-156401, “TP for TR 36.853-13: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (5 + 12 + 12)”, Alcatel-Lucent, U.S. Cellular.