3GPP TSG-RAN WG4 Meeting #78bis	R4-161599

San Jose Del Cabo, Mexico, 11 – 15 April, 2016

Agenda Item:	7.11.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	TP for TR 37.871: Necessary changes to the core requirements for Multi-Band Base Station testing with three or more bands

Document for:	Approval

# 1.	Introduction

The Multi-Band (MB) Base Station (BS) testing with three or more bands work item was approved at RAN#70 [1]. The necessary changes to the core requirements were discussed at RAN4#78 [2, 3]. In this contribution, we discuss how to implement the identified changes (if necessary) in the specifications, and provide a text proposal to include the proposals into the TR [4] of this work item.

# 2.	Discussion

Three clauses for changes were identified in [2, 3], namely: Definitions, Symbols, and Operating band unwanted emissions. Below we further discuss the identified changes and provide proposals to implement them (if necessary) in the specifications.

2.1	Definitions

It was discussed in [2] that the current MB BS definitions cannot be used to distinguish a MB BS that has a transceiver supporting three bands from a MB BS that has a common transceiver supporting two bands and a separate transceiver supporting the third band. While we agree this is indeed the case, we are not sure it is necessary to distinguish between these two MB BS in the definitions. This is because the current MB BS definitions do not state the number of supported bands in the MB BS or each transceiver. If we start stating the number of supported bands in the definitions, then the definitions would become cumbersome as the number of supported bands continue to increase in the future with advance in technology. Moreover, it may not be feasible to verify the actual number of bands supported by each transceiver in the conformance testing if different transceivers are mapped to the same antenna connector of the BS.

On the other hand, it was discussed in [3] that the current MB BS definitions cannot be used for a MB BS that support Bands 20 and 28 in Europe (where only the lower 2x30MHz of Band 28 are used) as these bands are overlapping. We agree this is a problem to be solved, but the current ‘non-overlapping operating band’ definitions were requested by operators who were concerned that a single band BS could be declared as a MB BS when the supported band is a sub-band or superseding-band of another band, e.g. Band 5 and Band 26. To solve this problem while avoiding the concern from the operators, we propose to change the MB BS definitions as follows:

Multi-band base station: base station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not an sub-band or superseding-band of another supported operating band) than the other carrier(s).

Multi-band transmitter: transmitter characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not an sub-band or superseding-band of another supported operating band) than the other carrier(s).

Multi-band receiver: receiver characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not an sub-band or superseding-band of another supported operating band) than the other carrier(s).

2.2	Symbols

It was discussed in [2] that Figure 3.2-3 in TS37.104 illustrates a dual-band BS only. While we agree this is indeed the case, we are not sure it is necessary to change the figure to include the case with three or more bands, as the figure is only used for illustration of the Radio Bandwidth and the figure would become cumbersome as the number of supported bands continue to increase in the future with advance in technology. Instead we propose to simply clarify the title of the figure as follows:

Figure 3.2-3: Illustration of Radio Bandwidth related symbols and definitions for Multi-band Multi-standard Radio (Dual-band Base Station)

2.3	Operating band unwanted emissions

It was discussed in [2] that the phrase “in another operating band” in the Operating band unwanted emissions clause may implicitly hint that it is a singular band. We agree that there may be ambiguity, and propose to clarify the phrase as follows:

For BS capable of multi-band operation where multiple bands are mapped on the same antenna connector, the operating band unwanted emission limits apply also in a supported operating band without any carriers transmitted, in the case where there are carriers transmitted in other supported operating band(s). In this case where there is no carrier transmitted in an operating band, no cumulative limits are applied in the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted and

# 3.	Conclusion

In this contribution, we have discussed how to implement the identified changes (if necessary) in the specifications. It is proposed to include the text proposal below into TR 37.871.

# 4.	Text proposal

<Start of change>

## 5	Necessary changes to the core requirements

## 5.1	Definitions

The current MB BS definitions cannot be used for a MB BS that support Bands 20 and 28 in Europe (where only the lower 2x30MHz of Band 28 are used) as these bands are overlapping. This is a problem to be solved, but the current ‘non-overlapping operating band’ definitions were requested by operators who were concerned that a single band BS could be declared as MB BS when the supported band is a sub-band or superseding-band of another band, e.g. Band 5 and Band 26. To solve this problem while avoiding the concern from the operators, it is agreed to change the MB BS definitions as follows:

Multi-band base station: base station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not an sub-band or superseding-band of another supported operating band) than the other carrier(s).

Multi-band transmitter: transmitter characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not an sub-band or superseding-band of another supported operating band) than the other carrier(s).

Multi-band receiver: receiver characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not an sub-band or superseding-band of another supported operating band) than the other carrier(s).

5.2	Symbols

Figure 3.2-3 in TS37.104 illustrates a dual-band BS only. As the figure is only used for illustration of the Radio Bandwidth and the figure would become cumbersome as the number of supported bands continue to increase in the future with advance in technology, it is agreed to simply clarify the title of the figure as follows:

Figure 3.2-3: Illustration of Radio Bandwidth related symbols and definitions for Multi-band Multi-standard Radio (Dual-band Base Station)

5.3	Operating band unwanted emissions

The phrase “in another operating band” in the Operating band unwanted emissions clause may implicitly hint that it is a singular band. To avoid such ambiguity, it is agreed to clarify the phrase as follows:

For BS capable of multi-band operation where multiple bands are mapped on the same antenna connector, the operating band unwanted emission limits apply also in a supported operating band without any carriers transmitted, in the case where there are carriers transmitted in other supported operating band(s). In this case where there is no carrier transmitted in an operating band, no cumulative limits are applied in the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted and

<End of change>

# References

[1]	RP-152205, “New Work Item proposal: Multi-Band Base Station testing with three or more bands”, Alcatel-Lucent.

[2]	R4-160430, “Impact on core requirements”, Ericsson.

[3]	R4-160579, “Discussion on Multi-band definition”, Huawei.

[4]	R4-161598, “TR 37.871 V0.1.0”, Nokia