3GPP TSG-RAN WG4 Meeting #78bis	R4-161603

SAN JOSE DEL CABO, MEXICO, APRIL 11 – 15, 2016

Agenda item:			6.5.4

Source:	Nokia

Title:	Initial Cat-M PRACH Simulation Results

Document for:		Discussion

# Introduction

PRACH simulation assumptions for supporting Cat-M UEs were agreed in RAN4#78 [1]. In this paper, we provide the ideal Cat-M PRACH link level simulation results based on the agreed simulation assumptions.

# Simulation Assumptions

Tables 1 and 2 provide the simulation assumptions for Cat-M PRACH detection performance based on the PRACH simulation assumptions agreed in [1]. Additionally, prach-ConfigurationIndex of 3 and PRACH Frequency Hopping Offsets of 24 PRBs and 40 PRBs were assumed in the PRACH simulations.

| Parameters | Values | Comments |
| --- | --- | --- |
| PRACH format | Format 0, Format 1, Format 2, Format 3 |  |
| Propagation conditions | AWGN, EPA1 | Due to the low mobility of Cat-M UEs, using EPA1 as in RLM requirements |
| Number of TX antennas | 1 |  |
| Number of RX antennas | 2, 4, 8 |  |
| Antenna correlation | Low |  |
| Noise model | AWGN |  |
| Timing offset | Randomly in the range of [0~1]us. | The timing offset base value is set to 50% of Ncs (see Section 8.4.1.4.2 in TS 36.141) |
| Repetition level | Basline: Format 0 and Format 1: {8,32} Format 2 and Format 3: {4,16} Other options are not precluded | May not need to simulate all levels of {1,2,4, 8,16,32,64,128} |
| Number of signatures reserved for Cat-M UEs | 16 | The size of preamble pool has impact on detection performance |
| Frequency offset | 0Hz for AWGN, 270 Hz for EPA1 | More discussion may be needed for frequency offset |
| Frequency hopping | ON/OFF |  |

Table 2. Test preambles for Normal Mode (From Table A.6-1 in TS 36.104 [2])

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 13 | 22 | 32 |
| 1 | 167 | 22 | 2 |
| 2 | 167 | 22 | 0 |
| 3 | 0 | 22 | 0 |

# Simulation Results

Tables 3 and 4 provide the initial idea simulation results for Cat-M PRACH detection.

Table 3 PRACH missed detection requirements for Cat-M1 Mode (Frequency Hopping OFF)

| Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 2 | AWGN | 0 | 4 | - | - | -24.37 | TBD |
|  |  |  | 8 | -24.23 | TBD | - | - |
|  |  |  | 16 | - | - | -28.42 | TBD |
|  |  |  | 32 | -28.24 | TBD | - | - |
|  | EPA1 Low | 270 Hz | 4 | - | - | -15.62 | TBD |
|  |  |  | 8 | -16.04 | TBD | - | - |
|  |  |  | 16 | - | - | -19.98 | TBD |
|  |  |  | 32 | -22.07 | TBD | - | - |
| [4] | AWGN | 0 | 4 | - | - | -26.43 | TBD |
|  |  |  | 8 | -26.20 | TBD | - | - |
|  |  |  | 16 | - | - | -30.10 | TBD |
|  |  |  | 32 | -30.14 | TBD | - | - |
|  | EPA1 Low | 270 Hz | 4 | - | - | -21.24 | TBD |
|  |  |  | 8 | -21.47 | TBD | - | - |
|  |  |  | 16 | - | - | -25.67 | TBD |
|  |  |  | 32 | -26.55 | TBD | - | - |
| [8] | AWGN | 0 | 4 | - | - | -28.40 | TBD |
|  |  |  | 8 | -28.25 | TBD | - | - |
|  |  |  | 16 | - | - | TBD | TBD |
|  |  |  | 32 | TBD | TBD | - | - |
|  | EPA1 Low | 270 Hz | 4 | - | - | -25.35 | TBD |
|  |  |  | 8 | -25.43 | TBD | - | - |
|  |  |  | 16 | - | - | TBD | TBD |
|  |  |  | 32 | TBD | TBD | - | - |
| Note 1: Number of signatures reserved for Cat-M UEs is 16. |  |  |  |  |  |  |  |

Table 4 PRACH missed detection requirements for Cat-M1 Mode (Frequency Hopping Offset 24PRBs)

| Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 2 | AWGN | 0 | 4 | - | - | -24.37 | TBD |
|  |  |  | 8 | -24.24 | TBD | - | - |
|  |  |  | 16 | - | - | -28.36 | TBD |
|  |  |  | 32 | -28.23 | TBD | - | - |
|  | EPA1 Low | 270 Hz | 4 | - | - | -18.07 | TBD |
|  |  |  | 8 | -19.08 | TBD | - | - |
|  |  |  | 16 | - | - | -23.12 | TBD |
|  |  |  | 32 | -24.63 | TBD | - | - |
| [4] | AWGN | 0 | 4 | - | - | -26.43 | TBD |
|  |  |  | 8 | -26.25 | TBD | - | - |
|  |  |  | 16 | - | - | -30.37 | TBD |
|  |  |  | 32 | -30.26 | TBD | - | - |
|  | EPA1 Low | 270 Hz | 4 | - | - | -22.73 | TBD |
|  |  |  | 8 | -23.16 | TBD | - | - |
|  |  |  | 16 | - | - | -26.55 | TBD |
|  |  |  | 32 | -27.42 | TBD | - | - |
| [8] | AWGN | 0 | 4 | - | - | -28.39 | TBD |
|  |  |  | 8 | -28.23 | TBD | - | - |
|  |  |  | 16 | - | - | TBD | TBD |
|  |  |  | 32 | TBD | TBD | - | - |
|  | EPA1 Low | 270 Hz | 4 | - | - | -25.87 | TBD |
|  |  |  | 8 | -26.15 | TBD | - | - |
|  |  |  | 16 | - | - | TBD | TBD |
|  |  |  | 32 | TBD | TBD | - | - |
| Note 1: Number of signatures reserved for Cat-M UEs is 16. Note 2: PRACH frequency hopping is ON. |  |  |  |  |  |  |  |

Table 5 PRACH missed detection requirements for Cat-M1 Mode (Frequency Hopping Offset 48PRBs)

| Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 2 | AWGN | 0 | 4 | - | - | -24.36 | TBD |
|  |  |  | 8 | -24.26 | TBD | - | - |
|  |  |  | 16 | - | - | -28.39 | TBD |
|  |  |  | 32 | -28.23 | TBD | - | - |
|  | EPA1 Low | 270 Hz | 4 | - | - | -19.30 | TBD |
|  |  |  | 8 | -19.46 | TBD | - | - |
|  |  |  | 16 | - | - | -23.59 | TBD |
|  |  |  | 32 | -24.98 | TBD | - | - |
| [4] | AWGN | 0 | 4 | - | - | -26.41 | TBD |
|  |  |  | 8 | -26.25 | TBD | - | - |
|  |  |  | 16 | - | - | -30.36 | TBD |
|  |  |  | 32 | -30.29 | TBD | - | - |
|  | EPA1 Low | 270 Hz | 4 | - | - | -23.24 | TBD |
|  |  |  | 8 | -23.82 | TBD | - | - |
|  |  |  | 16 | - | - | -26.92 | TBD |
|  |  |  | 32 | -27.70 | TBD | - | - |
| [8] | AWGN | 0 | 4 | - | - | -28.38 | TBD |
|  |  |  | 8 | -28.23 | TBD | - | - |
|  |  |  | 16 | - | - | TBD | TBD |
|  |  |  | 32 | TBD | TBD | - | - |
|  | EPA1 Low | 270 Hz | 4 | - | - | -26.34 | TBD |
|  |  |  | 8 | -26.36 | TBD | - | - |
|  |  |  | 16 | - | - | TBD | TBD |
|  |  |  | 32 | TBD | TBD | - | - |
| Note 1: Number of signatures reserved for Cat-M UEs is 16. Note 2: PRACH frequency hopping is ON. |  |  |  |  |  |  |  |

# Summary

In this contribution, we provided the initial ideal simulation results for Cat-M PRACH detection performance. From the simulation results, we observed that

- PRACH detection performance improves with Frequency Hopping. By comparison of the results between 24PRB hopping and no hopping with EPA1 channel, the improvement can be as large as 3dB for 2Rx antenna and 1.5dB for 4Rx antenna.

- Larger frequency hopping offset may also improve the PRACH detection performance, although it may not be significant. By comparison of the results between 24PRB hopping and 40PRBs with EPA1 channel, the improvement is normally smaller than 0.5dB.

# References

- R4-161227, Cat-M PRACH Simulation Assumptions, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Ericsson, Huawei, Hisilicon

- R4-161228	Cat-M PUCCH Simulation Assumptions, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Ericsson, Huawei, Hisilicon

- R4-160350, Work plan for WI on Rel-13 eMTC BS performance requirements Ericsson

- R4-161215, Way forward on eMTC BS demodulation performance requirements, Huawei, HiSilicon, Ericsson, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- R4-160271, Cat-M PUCCH performance requirement, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- R4-161217, Simulation assumptions for PUSCH	, Ericsson, Nokia networks, Alcatel-Lucent, Huawei, HiSilicon

- 3GPP TS 36.104, E-UTRA: Base Station (BS) radio transmission and reception.