3GPP TSG RAN WG4 Meeting #78bis	R4- 161622

SAN JOSE DEL CABO, MEXICO, APRIL 11 – 15, 2016

Agenda Item:	6.5.1

Source: 	Nokia

Title: 	Discussion: Cat-M1 Transmit Timing Accuracy Test Cases

Document for:	Discussion

# Introduction

The RRM requirements related to the Release 13 work item, “Further LTE Physical Layer Enhancements for MTC” [1] were completed in RAN4#78. In RAN4#78, we discussed the list of the potential RRM test cases for verifying the RRM requirements for Cat-M1 UEs in release 13 [2], and a WF was agreed that provided the suggested test cases as well as work plan [3].

The following Cat-M1 RRM test cases are suggested to be considered in Rel-13 for FDD, TDD and HD-FDD

- Intra frequency cell re-selection

- Intra-frequency handover

- Intra-frequency RRC Re-establishment

- Contention Based Random Access

- UE Transmit Timing Accuracy test

- UE Timing Advance Adjustment Accuracy test

- Radio Link Monitoring tests (In-Sync /Out-of-sync with/without DRX)

- Intra-frequency event triggered reporting under fading propagation conditions in synchronous/asynchronous cells with/without DRX

- Intra frequency case RSRP Accuracy

- Tests cases to verify other RRM requirements are not precluded.

Companies are encouraged to check the necessity on above test cases.

Time plan

- Complete test cases in two phases: phase I and phase II

RAN4#78bis

- Agree the complete list of the test cases for Rel-13 with detailed test configurations and parameter settings

- Determine the test cases included in Phase I and Phase II.

RAN4#79

- Provide draft of CRs for phase I test cases

- Agree on phase I test cases

RAN4#80

- Provide draft of CRs of phase II test cases

- Agree on phase II test cases

While Cat-M1 Transmit Timing Accuracy Test Cases were proposed for Cat-M1 UEs in the agreed WF [3], the Transmit Timing Accuracy requirements are actually not clearly defined yet. The discussion of Cat-M1 UE Transmit Timing Accuracy requirements and draft CRs are presented in the companion contributions [15, 16]. In addition, discussion papers and draft CRs for MPDCCH RMCs, PDSCH RMCs and OCGN patterns for supporting Cat-M1 test cases were presented in [17-21]. In this paper, we focus on the configurations and parameter settings for the Transmit Timing Accuracy test cases.

# Discussion

Test Purpose

The purpose of this test is to verify whether a UE is capable of following the frame timing change of the connected eNodeB and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

Test Environment

In legacy Transmit Timing Accuracy Tests, a single cell is used. Under the defined conditions for the strength of the transmitted signals and the propagation condition, the transmit timing is verified by the UE transmitting SRS. For legacy UEs, the Transmit Timing Accuracy test cases are defined in Section A.7.1.1 (FDD) and Section A.7.1.2 (TDD). Cat-M1 Transmit Timing Accuracy test cases should be developed based on these existing test cases with necessary changes for supporting Cat-M1 UEs.

In the following, we present the parameters for Cat-M1 Transmit Timing Accuracy tests with suggested changes in the settings if necessary

Number of carrier and cells

  - As legacy test cases, 1 E-UTRA carrier with 1 serving cell

Channel Bandwidth

  - 10MHz only

Cat-M1 UE narrowband

  - Assigned to any narrowband not overlapping with middle 6PRBs

Number of tests

  - 1 test for no DRX and 1 with DRX cycle = 640ms

PCFICH/PDCCH/PHICH parameters

  - Cat-M1 UE does not monitor PCFICH/PDCCH/PHICH channels

  - Use the same settings as legacy test case

MPDCCH parameters

  - Need to add the new MPDCCH RMCs [16,17]

PDSCH parameters

  - Use new PDSCH RMCs [18,19] to replace legacy PDSCH RMCs

  - UE working narrowband should not be overlapping with the carrier middle 6PRBs in order to test UE’s capability of frequency tuning for cell search

OCNG Patterns

  - Use new OCNG patterns [20,21] to replace legacy ones

SNR levels

  - levels need to be set according to the Cat-M1 requirements

  - For CEModeA, we suggest reusing the SNR levels used in legacy tests.

MIB/SIB1-BR

  - MIB transmission may be repeated in subframe#9 of the previous radio frame for FDD and subframe #5 of the same radio frame for TDD

  - SIB1-BR transmission may be repeated …

  - For CEModeA, no repetition.

SRS configuration

  - CEModeA

    - As defined in 36.213, a UE will transmit SRS on SRS resources based on two trigger types:

      - rigger type 0: higher layer signalling (RRC configuration)

      - trigger type 1: DCI formats 6-0A/6-1A

    - For CEModeA, we suggest sending SRS with type 0 with following SRS configuration parameters

      - srsBandwidthConfiguration (bw5)

      - srsSubframeConfiguration (sc1)

      - ackNackSrsSimultaneousTransmission (FALSE)

      - srsMaxUpPTS (TDD)

      - srsBandwidth (0 No hopping)

      - srsHoppingBandwidth (hbw0)

      - frequencyDomainPosition (0?)

      - duration (true, Indefinite duration)

      - Srs-ConfigurationIndex (2ms non-DRX, 320ms DRX)

      - transmissionComb (0)

    - cyclicShift (cs0 no cyclic shift)

    - SRS-AntennaPort (an1)

  - CEModeB

    - As defined in 36.213, a Cat-M1 UE configured with CEModeB is not expected to be configured with SRS parameters for trigger type 0 and trigger type 1.

    - Therefore, it needs further study on how to conduct UE transmit timing test for a Cat-M1 UE configured with CEModeB.

drx-Configuration Other parameters

  - CEModeA: reuse the configuration for the legacy test with 640ms DRX cycle

  - CEModeB; TBD

# Conclusions

In this paper we discussed the configuration and parameter settings for Cat-M1 Transmit Timing Accuracy tests, particularly for Cat-M1 UEs configured with CEModeA. Based on the discussion, a companion draft CR for CEModeA is also prepared as shown in [22].

It needs further investigation on how to conduct Transmit Timing Accuracy tests for a Cat-M1 UE configured with CEModeB, since a Cet-M1 UE configured with CEModeB is not expected to transmit SRS.

# References

- RP-141660, “New WI proposal: further LTE physical layer enhancements for MTC”, Ericsson, Nokia Networks

- R4-160273, “RRM Test Cases for Cat-M UEs, Nokia Networks”, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- R4-161211, “Way forward on test case list for Cat-M1 RRM, Nokia Networks”, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Ericsson, Huawei, Verizon, Intel

- R4-160514, “RRM requirements for Cat-M1 in IDLE mode in section 4”, Nokia Networks, Alcatel-Lucent, Ericsson, Huawel, HiSilicon, Intel, Verizon

- R4-161301, “Measurement requirements for Rel-13 MTC UE under normal coverage”, Ericsson, Alcatel-Lucent, Intel, CATT, Huawei, HiSilicon,  Verizon, Nokia Networks

- R4-161300, Measurement accuracy requirements for Rel-13 MTC UE,  Ericsson, Ericsson, Alcatel-Lucent, Huawei, HiSilicon, Intel Corporation, CATT, Verizon, Nokia Networks

- R4-161299, Radio link monitoring for Rel-13 MTC UE, Ericsson, Intel Corporation, Alcatel-Lucent, Huawei, HiSilicon, CATT, Verizon, Nokia Networks.

- R4-160976, Reference configuration for Rel-13 MTC,  Ericsson, Intel Corporation, Alcatel-Lucent, CATT, Huawei, HiSilicon, Verizon, Nokia Networks, NTTDOCOMO

- R4-161302, “Measurement requirements for Rel-13 MTC UE under enhanced coverage”, Ericsson, Alcatel-Lucent, Huawei, HiSilicon CATT, Intel Corporation, Verizon, Nokia Networks

- R4-161206, CR on Cat-M1 positioning, Huawei,HiSilicon

- R4-161207, Random access requirements for Cat-M1 UEs,  Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Ericsson

- R4-161210, CR on CGI reading of Cat-M1, Huawei, HiSilicon, Ericsson

- R4-161208, WF on agreements for remaining issues on RLM and SI reading, Ericsson, Intel Corporation, Qualcomm Incorporated, Nokia Networks, Alcatel-Lucent

- R4-161620, “Discussion: UE transmit timing Requirements for Cat-M1 UEs”, Nokia

- R4-161621, “CR: UE transmit timing Requirements for Cat-M1 UEs”, Nokia

- R4-161610, “Discussion: MPDCCH RMCs for Cat-M1 RRM Tests”, Nokia

- R4-161611, “CR: MPDCCH RMCs for Cat-M1 RRM Tests”, Nokia

- R4-161612, “Discussion: PDSCH patterns for Cat-M1 RRM Tests”, Nokia

- R4-161613, “CR: PDSCH patterns for Cat-M1 RRM Tests”, Nokia

- R4-161614, “Discussion: OCNG patterns for Cat-M1 RRM Tests”, Nokia

- R4-161615, “CR: OCNG patterns for Cat-M1 RRM Tests”, Nokia

- R4-161623, “CR: Cat-M1 Transmit Timing Accuracy Test Cases for CEModeA”, Nokia