3GPP TSG-RAN WG4 Meeting #78bis	R4-161623

SAN JOSE DEL CABO, MEXICO, APRIL 11 – 15, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3380 | rev |  | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: Cat-M1 Transmit Timing Accuracy Test Cases for CEModeA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Perf |  |  |  |  |  |  | Date: |  | 2016-03-30 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Cat-M1 Transmit Timing Accuracy Test Cases for CEModeA is not defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define Cat-M1 Transmit Timing Accuracy Test Cases for CEModeA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No Cat-M1 Transmit Timing Accuracy Test Cases for CEModeA |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section A.7.1.x, A.7.1.y, A.7.1.z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

### A.7.1.x	E-UTRAN FD-FDD Cat-M1 UE Transmit Timing Accuracy Tests in CEModeA

#### A.7.1.x.1	Test Purpose and Environment

The purpose of this test is to verify that the UE is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

For this test a single cell is used. Table A.7.1.x.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.x.1-2.

Table A.7.1.x.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN FDD

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  |  | 1 | 1 |  |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 |  |
| Configured UE narrowband |  | 1st |  | 1st |
| DRX cycle |  | ms | N/A | 640Note5 |  |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  |  | R.6 FDD | R.6 FDD |  |
| OCNG PatternNote2 |  |  | MOP.1 FDD | MOP.1 FDD |  |
| PBCH_RA |  | dB | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
|  |  | dBm/15 kHz | -98 | -98 |  |
|  |  | dB | 3 | 3 |  |
|  |  | dB | 3 | 3 |  |
| IoNote4 |  | dBm/9MHz | -65.5 | -65.5 |  |
|  |  | dBm/1.08 MHz | TBD | TBD |  |
| Propagation condition |  | - | AWGN | AWGN |  |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.x.1-3. |  |  |  |  |  |

Table A.7.1.x.1-2: Sounding Reference Symbol Configuration for UE Transmit Timing Accuracy Tests for E-UTRAN FDD

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Test 1 | Test 2 |  |
| srsBandwidthConfiguration | bw5 | bw5 |  |
| srsSubframeConfiguration | sc1 | sc1 |  |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE |  |
| srsMaxUpPTS | N/A | N/A | Not applicable for FDD |
| srsBandwidth | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 |  |
| duration | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 0 | 317 | SRS periodicity of 2ms and 320ms for Test 1 and 2, respectively. |
| transmissionComb | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 |  | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

Table A.7.1.x.1-3: DRX-Configuration for UE Transmit Timing Accuracy Test 2 for E-UTRAN FDD

| Field |  | Value |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Test 2 |  |  |
| onDurationTimer |  | psf1 |  |  |
| drx-InactivityTimer |  | psf1 |  |  |
| drx-RetransmissionTimer |  | psf1 |  |  |
| longDRX-CycleStartOffset |  | sf640 |  |  |
| shortDRX |  | disable |  |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.x.2	Test Requirements

For parameters specified in Tables A.7.1.x.1-1 and A.7.1.x.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.1.2.

The following sequence of events shall be used to verify that the requirements are met.

The test sequence shall be carried out in RRC_CONNECTED for both non-DRX (Test1) and DRX with a cycle length of 640 ms (Tests 2):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +128TS (approximately +4µs) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.1.2 until the UE transmit timing offset is within NTA×TS  ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 24×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.y	E-UTRAN HD-FDD Cat-M1 UE Transmit Timing Accuracy Tests in CEModeA

#### A.7.1.y.1	Test Purpose and Environment

The purpose of this test is to verify that the UE is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

For this test a single cell is used. Table A.7.1.x.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.x.1-2.

#### A.7.1.y.2	Test Requirements

The same as A.7.1.x.2.

### A.7.1.z	E-UTRAN TDD Cat-M1 UE Transmit Timing Accuracy Tests in CEModeA

#### A.7.1.z.1	Test Purpose and Environment

The purpose of this test is to verify that the UE is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

For this test a single cell is used. Table A.7.1.z.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.z.1-2.

Table A.7.1.z.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  |  | 1 | 1 |  |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 |  |
| Configured UE narrowband |  | 1st |  | 1st |
| Special subframe configurationNote6 |  |  | 6 | 6 |  |
| Uplink-downlink configurationNote7 |  |  | 1 | 1 |  |
| DRX cycle |  | ms | N/A | 640Note5 |  |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  |  | R.6 TDD | R.6 TDD |  |
| OCNG PatternNote2 |  |  | MOP.1 TDD | MOP.1 TDD |  |
| PBCH_RA |  | dB | 0 | 0 |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
|  |  | dBm/15 kHz | -98 | -98 |  |
|  |  | dB | 3 | 3 |  |
|  |  | dB | 3 | 3 |  |
| IoNote4 |  | dBm/9MHz | -65.5 | -65.5 |  |
|  |  | dBm/1.08 MHz | TBD | TBD |  |
| Propagation condition |  | - | AWGN | AWGN |  |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.x.1-3. Note 6: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 7: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. |  |  |  |  |  |

Table A.7.1.z.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for E-UTRAN TDD

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Test 1 | Test 2 |  |
| srsBandwidthConfiguration | bw5 | bw5 |  |
| srsSubframeConfiguration | sc3 | sc3 |  |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE |  |
| srsMaxUpPTS | FALSE | FALSE |  |
| srsBandwidth | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 |  |
| duration | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 15 | 325 | SRS periodicity of 10ms and 320ms for Test 1 and 2, respectively. |
| transmissionComb | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 |  | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

Table A.7.1.z.1-3: DRX Configuration in UE Transmit Timing Accuracy Test 2 for E-UTRAN TDD

The same as Table A.7.1.x.1-3.

#### A.7.1.z.2	Test Requirements

The same as A.7.1.x.2.

---END OF CHANGE 1---