3GPP TSG-RAN WG4 Meeting #78BIS	R4-161632

St. San Jose del Cabo, Mexico, 11th – 15th April, 2016

Agenda Item:	7.12.3

Source: 	Intel Corporation

Title: 	SFN channel model in high speed scenarios

Document for:	Discussion

- Introduction

The work item (WI) “Performance enhancements for high speed scenario” was approved at RAN plenary meeting #70 [1]. One objective of this WI is to identify downlink demodulation requirements under the identified high speed scenarios. A non-fading “2-path” high speed train (HST) channel model was provided and agreed in high speed scenario study item (SI) phase for the bidirectional SFN with RRH deployment [2]. In RAN4 #78 Meeting, a “3-path” HST channel model was proposed in [3]. The justification for the new “3-path” HST channel model is that, when the distance from the RRH to the railway, “Dmin”, is large, such as a few hundreds of meters, there would be more than 2 paths of reasonable strength received by HST UE. It is important because multiple paths would bring detrimental impact on demodulation performance of UE, if the receiver is only robust to the “2-path” channel model.

In this contribution, we share our view on the possibility of multiple paths in HST channel model. The multiple paths can be due to the presence of reflection surface in the environment surrounding, such as trees and walls aligned with the railway, which could cause fading effect [4].

- HST SFN channel model with reflection

Fig. 1 Diagram of HST in bidirectional RRH SFN

Fig.1 demonstrates an example of a HST SFN channel with tree reflection along the railway. For simplicity, we consider the reflection channel based on the “2-path” channel model in [2]. Moreover, only the path loss due to the propagation is considered in this contribution, while other impacts of the reflection factor, such as reflection loss, are not considered. Firstly, we define that

- Proposal 1: “Dsrfc” represents distance from reflection surface to railway (as shown in Fig.1).

The received power (normalized to the power when UE is Dmin away from a RRH), delay time and Doppler shift can be similarly calculated as in [2]. As defined by SI Scenario I: let Ds=1000m, Dmin=10m, v=350km/h, fc=2.69GHz, fd,max=872Hz and assume Dsrfc=50m, we obtain Fig.2.

Fig. 2 Tap Power, Delay and Doppler Shift of HST in bidirectional RRH SFN

- Proposal 2: HST channel model should consider multiple paths due to reflection.

This reflection model can be combined with the “3-path” HST channel model [3], as well as unidirectional SFN [5], which would result in a more complex and practical HST channel profile. Moreover, from Fig.2, we observe that:

- Observation 1: when a train is close to blue RRH, the green reflection path would reach UE with a large delay compared to the blue direct path;

- Observation 2: when a train is far from blue RRH, the green reflection path would reach UE at almost the same time as the blue direct path. If the phase shift is considered, the reflection path would cause either constructive or destructive interference.

- Proposal 3: The fading effect in HST channels may be considered when we investigate the receiver’s robustness in high speed scenarios.

- Conclusion

In this contribution, we provide our view on the possibility of multiple paths in HST channel model. The multiple paths can be due to the presence of reflection surface in the environment surrounding, such as trees and walls aligned with the railway, which could cause fading effect.

- Proposal 1: “Dsrfc” is introduced to represent distance from the reflection surface to the railway in HST scenarios.

- Proposal 2: HST channel model should consider multiple paths due to reflection.

- Proposal 3: The fading effect in HST channels may be considered when we investigate the receiver’s robustness in high speed scenarios.

- Reference

[1] RP-152263, “New WI proposal: Performance enhancements for high speed scenario”, NTT DoCoMo Inc., Huawei, HiSilicon, Dec. 2015.

[2] R4-151587, “Channel model for SFN deployment with RRH for HST scenario”, Qualcomm, April 2015.

[3] R4-160992, “UE demodulation performance in high speed scenarios in SFN deployment”, Qualcomm, February 2016.

[4] R. He, B. Ai, Z. Zhong, A. F. Molisch, R. Chen and Y. Yang, “A measurement-based stochastic model for high-speed railway channels”, in IEEE Transactions on Intelligent Transportation System, Vol. 16, No. 3, June 2015.

[5] R4-155748, “Evaluation of Unidirectional RRH arrangement for HST SFN”, Ericsson, October 2015.