3GPP TSG-RAN WG4 Meeting #78bis	R4-161921

San Jose Del Cabo, Mexico, 11 – 15 April, 2016

Source:	Ericsson

Title:	TP to 36.714-04-01 on co-existence analysis for 2A-46D, 7A-46D

Agenda item:				7.4.1

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.714-04-01 for co-existence analysis for 3DL/1UL CA combinations 2A-46D and 7A-46D.

# 2. Text Proposal for TR 36.714-04-01

##### ---Start of changes---

### 6.x		LTE Advanced Carrier Aggregation of Band 2 and Band 46 (4DL/1 UL)

##### ---Text omitted---

##### 6.x.1.2	Co-existence studies for CA_2A-46D (4DL/1UL)

Table 6.x.1.2-1 gives the UL to DL harmonic products for CA band 2 + band 46 with 1UL. It can be seen that 2nd harmonics fall into band 43 and that 3rd harmonics falls into the band 46.

Table 6.x.1.2-1: Band 2 and Band 46 1UL harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 3700 to 3820 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 5550 to 5730 |  | 15450 to 17775 |  |

The 2nd and 3rd order DL to UL harmonics and IMD products caused in the BS by transmitting of band 2 and band 46 DL carriers is calculated and shown in table 6.x.1.2-2 below:

Table 6.x.1.2-2: Band 2 and Band 46 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1930 | 1990 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 3860 to 3980 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 5790 to 5970 |  | 15450 to 17775 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 3160 to 3995 |  | 7080 to 7915 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high & (2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high & (2*f2_low + f1_low) | 2* f2_high – f1_low & (2*f2_high + f1_high) |
| IMD frequency range (MHz) | 1170 to 2065 9010 to 9905 |  | 8310 to 9920 12230 to 13840 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 1910 to 2010 |  | 5130 to 5945 |  |

It can be seen from Table 6.x.1.2-2 that none of 2rd harmonics fall into the any E-UTRA operating BS receive band, and it can be seen that 3rd harmonics fall into the E-UTRA operating BS receive band 46.

It can also be seen that two-tone 3rd order IMD products may fall into Band 22, 42 and 43,  and that the three-tone 3rd order IMD products may fall into Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 33, 34, 35, 36, 37, 39, 65 and 66. With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 33, 34, 35, 36, 37, 39, 65 and 66 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 2 and Band 46 BS transmitters do not share the same antenna with any of these receivers.

##### ---Text omitted---

### 6.y	LTE Advanced Carrier Aggregation of Band 7 and Band 46 (4DL/1 UL)

##### ---Text omitted---

##### 6.y.1.2	Co-existence studies for CA_7A-46D (4DL/1UL)

Table 6.y.1.2-1 gives the UL to DL harmonic products for CA band 7 + band 46 with 1UL. It can be seen that the 2nd or 3rd harmonics don’t fall into any E-UTRA operating band.

Table 6.y.1.2-1: Band 7 and Band 46 1UL harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 5000 to 5140 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 7500 to 7710 |  | 15450 to 17775 |  |

The 2nd and 3rd order DL to UL harmonics and IMD products caused in the BS by transmitting of band 7 and band 46 DL carriers is calculated and shown in table 6.y.1.2-2 below:

Table 6.y.1.2-2: Band 7 and Band 46 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2620 | 2690 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 5240 to 5380 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 7860 to 8070 |  | 15450 to 17775 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 2460 to 3305 |  | 7770 to 8615 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high & (2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high & (2*f2_low + f1_low) | 2* f2_high – f1_low & (2*f2_high + f1_high) |
| IMD frequency range (MHz) | 230 to 685 10390 to 11305 |  | 7610 to 9230 12920 to 14540 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 2600 to 2710 |  | 5130 to 5945 |  |

It can be seen from Table 6.y.1.2-2 that the 2nd harmonics fall into Band 46 of the E-UTRA operating BS receive bands, and that none of the 3rd harmonics fall into anyof the E-UTRA operating BS receive bands

It can also be seen that two-tone 2nd order IMD products may fall into Band 7, 38 and 41, and that the two-tone 3rd order IMD products may fall into Band 31. The three tone 3rd order products may fall into Band 7, 38 and 41.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 7, 31, 38 and 41 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 7 and Band 46 BS transmitters do not share the same antenna with any of these receivers.

##### ---End of changes---