3GPP TSG RAN WG4 Meeting #78bis	R4-161947

San José del Cabo, Mexico, April 11 – 15, 2016

Agenda Item:	6.13.5

Source: 	Ericsson

Title:	Link Simulations for Cell Search in NB-IoT

Document for:	Discussion

# Introduction

During the RAN4#78 meeting on NB-IoT RRM an attempt was made by several companies to align on simulation assumptions for cell search link simulations [1], however due to that synchronization signal design had not been settled by RAN1, the proposal on simulation assumption was not agreed.

Since the RAN4#78 meeting RAN1 has had an ad hoc meeting where the NB-PSS design was agreed while the NB-SSS design still is open. In an attempt to provide input to the upcoming discussions at RAN4#78bis on simulation assumptions we have conducted initial simulations using the agreed NB-PSS design and periodicity, and NB-SSS design based on our previous proposal in RAN1 [3].

In this contribution we provide initial simulation results for PSS detection latency, SSS detection latency, and total cell detection time.

Note: Due to the short time between completion of the RAN1 AH and RAN4#78bis submission deadline the simulation results will be updated in a revision of this document.

# Background

## Synchronization signals

Similar to MBB LTE the synchronization signals consists of a Primary Synchronization Signal (NB-PSS) and a Secondary Synchronization Signal (NB-SSS). The number of versions, the composition, and the periodicity however differs from the legacy LTE system. There is a single PSS common to all 504 cell identities, and there are 504 cell identities represented by SSS. In subframes that carry synchronization signals, NB-PSS and NB-SSS, respectively, occupies the last 11 OFDM symbols, at least for the normal cyclic prefix configuration. The number of subcarriers that are used for carrying synchronization signals is 11 for NB-PSS and 12 for NB-SSS.

The periodicities of NB-PSS and NB-SSS have not yet been determined, but NB-PSS is sent in subframe 5 and NB-SSS in subframe 9. Subframes in which NB-PSS or NB-SSS are transmitted do not carry NB-RS, but may in case of in-band deployment be subjected to puncturing by CRS from the LTE cell in whose downlink bandwidth the NB-IoT cell operates.

The agreed design of NB-PSS is captured in [2], and consists of a Zadoff-Chu sequence of length 11 in frequency domain, generated with root index 5. The same Zadoff–Chu sequence is repeated in 11 subsequent OFDM symbols but with a phase determined by a code cover sequence [1  1  1  1  -1  -1  1  1  1  -1  1].

For the design for NB-SSS it has been agreed that it too shall be based on one or more Zadoff-Chu sequences, but remaining details on number of sequences and potential use of binary scrambling sequences are still open. In the simulations we have used the NB-SSS design in [3].

Regarding periodicities of NB-PSS and NB-SSS we have assumed that the former has a periodicity of 10ms and that the latter has a periodicity of 20ms and is transmitted in every odd radio frame. Moreover, for the latter a binary scrambling code is applied on the four instances transmitted during a 80ms period such that the UE can determine where in the 80ms period it captured the NB-SSS instance. The layout of the synchronization signals in a 80ms period is illustrated in Figure 1.

Figure 1: Assumed periodicities and phases of NB-PSS and NB-SSS in simulations.

## Operation modes

Three operation modes have been agreed for NB-IoT:

- Stand-alone operation, making use of scattered spectrum and e.g. re-farming of GSM spectrum,

- Guard-band operation, making use of the currently unused resource blocks at the edges of the MBB LTE cell, and

- In-band operation, making use of resources that are within the system bandwidth of the MBB LTE cell.

In stand-alone and guard-band operation it is mainly the interference that may put a limit on the performance, whereas in the in-band operation additional puncturing of NB-PSS and NB-SSS by MBB LTE CRSs may have an impact on the performance. For the latter case a boosting of the NB-PSS and NB-SSS by up to 6dB compared to the MBB LTE CRS has been agreed. Particularly for in-band operation the first 3 OFDM symbols in a subframe are reserved for the MBB LTE control region (PDCCH, PHICH, PCFICH) and hence are not used by NB-IoT. Both normal and extended cyclic prefixes are to be supported however the impact on e.g. NB-PSS and NB-SSS when extended cyclic prefix is used is still to be specified by RAN1.

## Coverage scenarios

The NB-IoT device may operate in three coverage scenarios:

- Normal coverage, similar to that of MBB LTE devices

- Extended coverage, similar to that of eMTC devices, and

- Extreme coverage, a new coverage scenario that implies an SINR as low as about -20dB.

# Simulation assumptions

The parameters used in the simulations are shown in Table 1 below. The assumptions for the initial simulations, with purpose of aligning results with interested companies, are based on that the NB-IoT device already has carried out initial cell search, i.e., the device has synchronized its frequency reference to the network. Also the initial cell search has to be investigated in order to provide information to RAN2 on what latency can be expected in case serving cell coverage has been lost during DRX/eDRX inactivity.

The cases considered are a single cell with radio propagation channels AWGN, ETU 1Hz and EPA 1Hz. All three cases aim at modelling a stationary device, with the fading channels modelling that although the device is stationary, the multipath propagation may be influenced by activities in the surrounding (road traffic etc.)

Table 1: Simulation parameters

| Parameter |  | Value(s) | Comment |
| --- | --- | --- | --- |
| Channel model |  | AWGN, ETU 1Hz, EPA 1Hz | Models stationary device |
| System bandwidth |  | 180kHz (1 PRB) |  |
| Antenna configuration | In-band Guard-band | 2TX, 1RX | Puncturing of NB-PSS and NB-SSS in in-band operation when colliding with 2TX CRS pattern |
|  | Stand-alone | 1TX, 1RX |  |
| Power boosting for NB-IoT | In-band Guard-band | 0 dB, 6 dB | NB-IoT is transmitted at 6dB higher power than the MBB LTE cell to which it is associated. |
|  | Stand-alone | N/A |  |
| Cyclic prefix configuration |  | Normal CP | CP lengths 10 and 9 samples for first and following symbols in a slot, respectively, at 1.92MS/s. |
| NB-IoT load condition |  | Fully loaded, modelled by OCNG | OFDM symbols 3-13 |
| MBB LTE cell load condition | In-band | Fully loaded, modelled by OCNG | OFDM symbols 0-2. (Control region) |
|  | Guard-band Stand-alone | N/A |  |
| Frequency offset |  | Static 200Hz frequency offset | Models imperfect AFC tracking. |
| SNR range |  | -20 to 10 dB |  |
| NB-PSS periodicity |  | 10ms |  |
| NB-SSS periodicity |  | 20ms |  |
| Algorithms for cell detection |  | Implementation specific |  |

# Simulation results

## In-band operation

### AWGN

# To be revised

Figure 2: Results for in-band AWGN scenario: (left) 6dB boosting, (right) 0dB boosting.

Table 2: In-band AWGN scenario. Percentiles for total cell detection time and accumulated NB-PSS and NB-SSS instances for successful detection.

|  | Boosting 6dB |  |  |  |  |  |  |  | Boosting 0dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR [dB] | Total time [ms] |  | # PSS |  |  | # SSS |  |  | Total time [ms] |  | # PSS |  |  | # SSS |  |  |
|  | 50pt | 90pt | 50pt | 90pt | 95pt | 50pt | 90pt | 95pt | 50pt | 90pt | 50pt | 90pt | 95pt | 50pt | 90pt | 95pt |
| -20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### ETU 1Hz

# To be revised

Figure 3: Results for in-band ETU 1Hz scenario: (left) 6dB boosting, (right) 0dB boosting.

Table 3: In-band ETU 1Hz scenario. Percentiles for total cell detection time and accumulated NB-PSS and NB-SSS instances for successful detection.

|  | Boosting 6dB |  |  |  |  |  |  |  | Boosting 0dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR [dB] | Total time [ms] |  | # PSS |  |  | # SSS |  |  | Total time [ms] |  | # PSS |  |  | # SSS |  |  |
|  | 50pt | 90pt | 50pt | 90pt | 95pt | 50pt | 90pt | 95pt | 50pt | 90pt | 50pt | 90pt | 95pt | 50pt | 90pt | 95pt |
| -20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### EPA 1Hz

# To be revised

Figure 4: Results for in-band EPA 1Hz scenario: (left) 6dB boosting, (right) 0dB boosting.

Table 4: In-band EPA 1Hz scenario. Percentiles for total cell detection time and accumulated NB-PSS and NB-SSS instances for successful detection.

|  | Boosting 6dB |  |  |  |  |  |  |  | Boosting 0dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR [dB] | Total time [ms] |  | # PSS |  |  | # SSS |  |  | Total time [ms] |  | # PSS |  |  | # SSS |  |  |
|  | 50pt | 90pt | 50pt | 90pt | 95pt | 50pt | 90pt | 95pt | 50pt | 90pt | 50pt | 90pt | 95pt | 50pt | 90pt | 95pt |
| -20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Stand-alone

### AWGN

# To be revised

Figure 5: Results for stand-alone AWGN scenario

Table 5: Stand-alone AWGN scenario. Percentiles for total cell detection time and accumulated NB-PSS and NB-SSS instances for successful detection.

| SNR [dB] | Total time [ms] |  | # PSS |  |  | # SSS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50pt | 90pt | 50pt | 90pt | 95pt | 50pt | 90pt | 95pt |
| -20 |  |  |  |  |  |  |  |  |
| -16 |  |  |  |  |  |  |  |  |
| -12 |  |  |  |  |  |  |  |  |
| -6 |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |

### ETU 1Hz

# To be revised

Figure 6: Results for stand-alone ETU 1Hz scenario

Table 6: Stand-alone ETU 1Hz scenario. Percentiles for total cell detection time and accumulated NB-PSS and NB-SSS instances for successful detection.

| SNR [dB] | Total time [ms] |  | # PSS |  |  | # SSS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50pt | 90pt | 50pt | 90pt | 95pt | 50pt | 90pt | 95pt |
| -20 |  |  |  |  |  |  |  |  |
| -16 |  |  |  |  |  |  |  |  |
| -12 |  |  |  |  |  |  |  |  |
| -6 |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |

### EPA 1Hz

# To be revised

Figure 7: Results for stand-alone EPA 1Hz scenario

Table 7: Stand-alone EPA 1Hz scenario. Percentiles for total cell detection time and accumulated NB-PSS and NB-SSS instances for successful detection.

| SNR [dB] | Total time [ms] |  | # PSS |  |  | # SSS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50pt | 90pt | 50pt | 90pt | 95pt | 50pt | 90pt | 95pt |
| -20 |  |  |  |  |  |  |  |  |
| -16 |  |  |  |  |  |  |  |  |
| -12 |  |  |  |  |  |  |  |  |
| -6 |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |

# Summary

The simulations provide initial results for the amount of accumulation needed for NB-PSS (# PSS) and NB-SSS (# SSS) for correct detection of the timing and the PCI, respectively. The time for RSRP measurements is not covered by the simulations.

The provided cell detection time percentiles (“Total time”) comprise the time needed to detect NB-PSS and NB-SSS. An NB-IoT device implementation that either accurately can determine when a NB-PSS candidate is reliable, or is able to pursue NB-SSS detection for a large number of NB-PSS candidates can meet the cell detection times provided under “Total time” in the tables.

However a NB-IoT device that instead bases cell detection on accumulation of a certain number of NB-PSS instances and NB-SSS instances, respectively, before carrying out detection attempts will require a somewhat longer cell detection time, since NB-PSS and NB-SSS detections each separately will have to fulfil some given probability of success. Assuming independence between NB-PSS and NB-SSS detection, which is true for the AWGN scenario but only approximate for the fading channels, the number of accumulated instances given by the 95th percentiles for NB-PSS and NB-SSS, respectively, are required in order for the cell detection to be successful with 90% probability.

[…]

Observation 1: The achievable cell detection time is highly influenced by which complexity can be accepted for the procedure. A discussion on the acceptable complexity would be in place before settling on cell detection core requirements.

Besides the cell search design as such there is also one factor that will highly influence the cell detection time and the power consumption: whether high or low density of NB-PSS and NB-SSS is used. When low density is used, each accumulated instance of NB-PSS requires 20ms radio time, whereas for high density it only requires 10ms. Moreover the memory for accumulation is doubled.

Observation 2: The density of NB-PSS has an impact on the power consumption and on the required memory for accumulation. The sparser the NB-PSS the more radio time hence power is needed for accumulating a particular number of instances. The sparser the NB-PSS the larger memory is need for accumulated result.

[…]

# Conclusions

We have carried out NB-IoT cell search simulations and have made the following observations:

Observation 1: The achievable cell detection time is highly influenced by which complexity can be accepted for the procedure. A discussion on the acceptable complexity would be in place before settling on cell detection core requirements.

Observation 2: The density of NB-PSS has an impact on the power consumption and on the required memory for accumulation. The sparser the NB-PSS the more radio time hence power is needed for accumulating a particular number of instances. The sparser the NB-PSS the larger memory is need for the accumulation.

[…]

# References

- R4-161292 “Link simulation assumptions for cell search in NB-IoT”, Ericsson, Nokia Networks, Alcatel-Lucent

- R1-161981 “NB-PSS and NB-SSS Design”, Qualcomm Incorporated

- R1-160266 “NB-IoT – Synchronization Channel Design”, Ericsson