3GPP TSG RAN4 Meeting #78bis	R4-162202

San Jose del Cabo, Mexico, 11th – 15th Apr, 2016

Title: 	MSD for 8+41 2UL CA

Source: 	Huawei, HiSilicon

Agenda item:	7.6.1

Document for:	Discussion

# 1	Introduction

This contribution discusses MSD requirements for 8+41 UL CA.

# 2	Discussion

This CA combination has two intermodulation products falling on top of own B8 RX channel, IMD3 and IMD5. IMD3 occurs only in case when B41 allocation is at the very high end of B41 and B8 allocation is at the very low end of B8. IMD5 occurs more often. This combination is specific because it has also 1.4MHz and 3MHz BW’s defined for B8.

The following example test points would be suitable for IMD3 and IMD5 evaluation. In case of IMD3 the intermodulation hits own DL CC only partially when the BW is 5MHz. When B8 BW is 1.4MHz the intermodulation is centred around own DL CC. In case of IMD5 the frequencies can be chosen in a way that the intermodulation is centred around own DL CC.

According to earlier agreement [1], only the intermodulation with highest MSD is specified. In this case it could mean IMD3 with 1.4MHz BW, IMD3 with 5MHz BW, IMD5 with 1.4MHz BW, or IMD with 5MHz BW.

It is not very straightforward to choose the specified IMD for this combination; for instance IMD3 with 5MHz BW does not in the middle of own DL CC leading to larger CF and thus smaller IMD compared with 1.4MHz BW. On the other hand, 1.4MHz BW has not previously been used in UL CA MSD calculations. IMD5 may be larger or smaller than IMD3 depending on BW used in IMD3 analysis.

| CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL | 3rd DL Fc (MHz) | DL BW (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |
| B8+B41 | B8 | IMD3 | fB41 – 2*fB8 | 880.7 | 1.4 | 6 | 925.7 | 1.4 |
|  | B41 |  |  | 2687.1 | 5 | 25 |  |  |
|  | B8 | IMD3 | fB41 – 2*fB8 | 882.5 | 5 | 25 | 927.5 | 5 |
|  | B41 |  |  | 2687.5 | 5 | 25 |  |  |
|  | B8 | IMD5 | 4*fB8-fB42 | 900 | 1.4 | 25 | 945 | 1.4 |
|  | B41 |  |  | 2650 | 5 | 25 |  |  |
|  | B8 | IMD5 | 4*fB8-fB42 - | 900 | 5 | 25 | 945 | 5 |
|  | B41 |  |  | 2650 | 5 | 25 |  |  |

Table 1 Example test points

We use the following RF front-end architectures in our UL CA analysis. We note that harmonic trap is not used even there is 3rd harmonic relation between B8 UL and B41because the trap was not used in respective 1UL/2DL specification.

Figure 1 Common diplexer -architecture

We analyze the amount of IMD using the following component linearity assumptions:

| Component | IP3 (dBm) | IP5 (dBm) |
| --- | --- | --- |
| Antenna switch | 68 | 53 |  |
| Diplexer | 86 | 53 |  |
| Duplexer | 74 | 53 |  |
| PA forward mixing | 30 | 27 |  |
| PA reverse mixing | 28 | 32 |  |
| LNA | -6 | -10 |  |

Table 2 General linearity parameters

B8 duplexer TX rejection at B41	35

B8 duplexer RX rejection at B41	40

B41 filter attenuation at B8 8X		36

B41 filter attenuation at B8 TX		40

Antenna isolation 					10

PCB isolation PA-PA 				60

PCB isolation TX-RX				65

RFIC Pout B8/B41					0

B8 duplexer TX-RX isolation		50

Diplexer isolation						15

# 3	Conclusion

Example test points for B8+B41 UL CA MSD were provided and different alternatives discussed. In addition UE reference architecture was shown and initial set of RF parameters were provided.

# 4	References

[1] R4-144031, “WF on 2UL inter-band CA MSD test configurations”, Mediatek, RAN4#71