3GPP TSG-RAN WG4 Meeting#78bis    						     							  R4-162214

# San Jose del Cabo, Mexico,  11-15 April 2016

Title:	Consideration on V2V candidate band

Source:	CMCC

Agenda Item:	7.13.1

Document for:	Approval

- Introduction

In RAN#71 meeting, WID for Support for V2V services based on LTE sidelink [1] was approved. The RAN4 related objective are :

- To specify a solution/requirement (if needed) for coexistence of PC5-based V2V operation and legacy Uu operation with LTE in the same carrier frequency [RAN1] and in an adjacent carrier frequency [RAN4]

- To specify UE Tx and Rx RF requirement covering operations at up to 6 GHz carrier [RAN4]

- To specify RRM core requirement [RAN4]

In RAN4#78 meeting, A WF on operating band plan for V2V services was approved.

- In licensed bands, RAN WG4 includes specific operating bands based on the current E-UTRA bands.

- RAN4 encourage operators to submit proposals

- RAN WG4 to study two options for V2V service for unlicensed band operation.

  - Reuse Band 46

  - Define a new band

In this paper, we try to make clear of the V2V scenario and proposed candidate bands.

- Discussion

  - PC5 based V2V Scenario

The WI scope mainly focuses on the sidelink enhancement, i.e. PC5-based V2V services. However, if we look at TR 36.885, the network control enhancement of PC5 is also related to Uu.

  - Network control aspect

    - When a UE operates PC5 in a carrier where no cell is detected but it is inside coverage of an eNB in another carrier (i.e., different carriers for Uu and PC5), network may control at least some parameters that affects UE resource selection.

      - At least when the PC5 and Uu carriers are allocated to the same operator, RAN1 assumes that eNB has at least some controls. FFS for the other cases.

Observation#1: V2V UE may have Uu based network control link and PC5 based V2V link in different carriers.

If this multi-carrier V2V UE supports simultaneously Tx and Rx in Uu and PC5 carrier, RF requirement for each band combination (Uu band + PC5 band) should also be specified similar to that we have done for eD2D.

Observation#2:  V2V UE supporting Uu and PC5 in different carriers need new RF requirement for each band combination.

- 2.3	Proposed candidate band for V2V

From operators’ point of view, we think it is better to use dedicate and shared spectrum for PC5 based V2V link, e.g. part of 5GHz band. And for Uu based network control link, licensed band is more appropriate. Take CMCC for example, 5.9GHz (under discussion in China for V2V) could be support for PC5 based V2V link and B41/ B39 for Uu based network control link.

Proposal#1: CMCC proposes 5.9GHz band for V2V link and B41/B39 for network control.

Proposal#2: B41/B39+5.9GHz combination RF requirements need to be specified for V2V UE.

- Conclusion

Observation#1: V2V UE may have Uu based network control link and PC5 based V2V link in different carriers.

Observation#2:  V2V UE supporting Uu and PC5 in different carriers need new RF requirement for each band combination.

Proposal#1: CMCC proposes 5.9GHz band for V2V link and B41/B39 for network control.

Proposal#2: B41/B39+5.9GHz combination RF requirements need to be specified for V2V UE.

- Reference

[1] RP-152293 WID proposal on PC5-based V2V， LG Electronics, Huawei, HiSilicon, CATT, CATR

[2] TR 36.885 Study on LTE-based V2X Services