3GPP TSG-RAN WG4 #78bis	R4-162573

April 11h – 15th, 2016

San Jose del Cabo, Mexico

Agenda item:	8.1.2

Source: 	Qualcomm Incorporated

Title: 	Additional simulation results for HPUE adjacent channel coexistence study

Document for:	Discussion

# Introduction

In this contribution we present additional simulation results for the adjacent channel coexistence study between Band 41 UE power class supporting 26dBm and legacy class 3UEs operating in the same band. The new results are based on the additional simulation assumptions agreed through email discussion after RAN4 #78.

# New simulation assumptions

In RAN4 #78 the following Band 41 HPUE Way Forward was agreed [1]:

- In parallel for the next meeting, companies are requested to

  - Adopt 1 dB ACLR tightening as one option as the basis for continuing work into the next meeting on MPR/A-MPR determination.

  - Companies to provide system coexistence simulation results based on expanded cell in aggressor network vs. conventional cell size in victim network.  China Telecom to provide specific simulation parameters by 03/15/16.

  - Companies are encouraged to provide simulation results with more aggressive power control parameters for information.

  - Companies to evaluate the impact of HPUE to CA with specific examples of B3+B41 with and without Pcell operation in B41.  In CA configuration, Pcell max Tx power is 23 dBm.  Impact to B7 and B38 to be assessed.

Following the above way forward, two new sets of simulation assumptions were agreed through email discussion [2], aiming at:

- Evaluating the impact of larger cell size which can be potentially be available due to the higher Tx power of Class 2 UE

- Evaluating the impact of lower CL-xile, i.e. a situation in which HPUE is considered as a feature for improving the cell edge throughput performance

Based on the two goals above, the new scenarios, namely scenario A1 and A2 were proposed.

## Scenario A1: Expanded cell size in aggressor network vs. conventional cell size in victim network

As mentioned above, the goal of this scenario is to evaluate the impact of larger cell size which can be available when HPUE are deployed. In other words, the aggressor system which deploy class 2 UEs in Band41 will have a larger Inter Site Distance (ISD). The new ISDs were determined with the following procedure [2]:

- With 3dB increase in UE maximal Tx power, the coverage of the HPUE system can be extended. The following criteria is used to set the cell radius of the HPUE system:

  - PL (cell_radius_of_HPUE) = PL (cell_radius_of_normal_UE) + 3dB

    - PL (cell_radius_of_HPUE) is the path-loss when the HPUE and base station separation equals to its cell radius.

    - PL (cell_radius_of_normal_UE) is the path-loss when the normal UE and base station separation equals to its cell radius.

- As a result:

  - For Urban and Suburban environments, the ISD of aggressor system = 1.2 times of the ISD of victim system.

  - For Rural environments, the ISD of aggressor system = 1.22 times of the ISD of victim system.

The new ISD to be used in Scenario A1 are summarized in Table 1.

Table 1. New ISDs for Scenario A1.

| Environment | ISD of victim system (km) | ISD of aggressor system (km) |
| --- | --- | --- |
| Urban | 0.75 | 0.9 |
| Suburban | 2.8 | 3.36 |
| Rural | 6 | 7.32 |
| Rural | 8 | 9.76 |

It is worth noticing that because of the different ISD between aggressor and victim system is not possible to always deploy the aggressor system at cell edge of the victim system. This is shown in Figure 1.

Figure 1. Cellular layout in Urban environment for Scenario A1.

The power control parameters in Scenario A1 are the same as the one already adopted in RAN4 #78 [4]. Only PC Set 1 and 20MHz case will be evaluated.

## Scenario A2: Conventional cell size in aggressor/victim network with modified CLx-ile values

The goal of this scenario is to improve the cell edge throughput performance by modifying the UL power control distribution and increasing the number of UEs which can transmit more than 23dBm. Three new PC Set were agreed, namely Set 4A, 4B, 4C [2]. The old PC set 1 and new PC set 4 are summarized in Table 2.

In this scenario, ISD simulated is the same for both victim and aggressor system and only 20MHz case will be considered.

Table 2. New CL-xile for Scenario A2.

|  |  | CL-xile 23dBm UEs [dB] |  |  |  | CL-xile 26dBm UEs [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Gamma | Set 1 | Set 4A | Set 4B | Set 4C | Set 1 | Set 4A | Set 4B | Set 4C |
| Urban | 1 | 109 | 107 | 103 | 101 | 112 | 110 | 106 | 104 |
| Rural | 1 | 133 | 128 | 125 | 123 | 136 | 131 | 128 | 126 |
| Suburban - ISD=6km | 1 | 117 | 112 | 108 | 106 | 120 | 115 | 111 | 109 |
| Suburban - ISD=8km | 1 | 122 | 116 | 112 | 110 | 125 | 119 | 115 | 113 |

# Simulation Results

In this section we provide a summary of the simulation results obtained with the new simulation assumptions. We first summarize results and conclusions obtained with the old assumptions and then take into account Scenario A1 and A2.

## Summary of previous simulation results

We presented a full set of simulation results in [3]. Table 3 shows the percentage of UEs transmitting above 23dBm, while Table 4 summarizes the ACLR tightening needed in the different scenarios.

Table 3. Percentage of UEs transmitting more than 23dBm.

|  |  | Percentage of UEs transmitting more than 23dBm |  |
| --- | --- | --- | --- |
| Scenario | ISD | 10MHz | 20MHz |
| Urban Set 1 | 750m | 3.47% | 6.78% |
| Urban Set 1p | 750m | 0.37% | 0.90% |
| Urban Set 2 | 750m | negligible | negligible |
| SubUrban Set 1 | 2.8km | 1.84% | 3.91% |
| SubUrban Set 2 | 2.8km | negligible | 0.02% |
| Rural Set1 | 6km | 1.83% | 3.93% |
| Rural Set2 | 6km | negligible | 0.03% |
| Rural Set1 | 8km | 1.95% | 4.18% |
| Rural Set2 | 8km | negligible | 0.03% |

Table 4. ACLR tightening needed by power class 2 UEs.

|  |  | ACLR Tightening Needed |  |
| --- | --- | --- | --- |
| Scenario | ISD | 10MHz | 20MHz |
| Urban Set 1 | 750m | <=1dB | <=0.8dB |
| Urban Set 1p | 750m | <=0.1dB | <=0.1dB |
| Urban Set 2 | 750m | Negligible | Negligible |
| SubUrban Set 1 | 2.8km | Negligible | <=0.1dB |
| SubUrban Set 2 | 2.8km | Negligible | Negligible |
| Rural Set1 | 6km | <=0.1dB | <=0.2dB |
| Rural Set2 | 6km | Negligible | Negligible |
| Rural Set1 | 8km | <=0.1dB | <=0.1dB |
| Rural Set2 | 8km | Negligible | Negligible |

The above results where summarized in the following observation:

Observation 1: ACLR tightening required by class 2 UEs is between 0dB and 1dB (from [3]).

This conclusion were consistent with results presented by other companies and capture in the RAN4 #78 way forward [1]:

- Adopt 1 dB ACLR tightening as one option as the basis for continuing work into the next meeting on MPR/A-MPR determination.

## Scenario A1

In order to understand the different impact of Scenario A1 we will first analyse the new UL power distributions available with the updated ISD for the aggressor system.

Figure 2. UE Tx power distributions for Scenario A1.

Figure 2 shows the UE tx power distribution considering the new ISD proposed for Scenario A1 for all the simulated environments. As it can be observed, and as expected, with the ISD the UL power for the aggressor system is around 3dB higher (mean value). However it is worth noticing that the interferer level at the base station victim due to adjacent channel interference is not exactly 3dB higher compared to the case in which ISD is the same for aggressor and victim system. This is simply due to the fact that the aggressor UE - victim BS distance distribution is not the same in the two cases.

A summary of the simulation results obtained is shown in Table 5. As it can be observed the ACLR tightening needed is always less than 2dB. A collection of all the detailed simulation results for scenario A1 is presented in Appendix A.

Table 5. ACLR tightening needed by power class 2 UEs in Scenario A1.

|  |  | ACLR Tightening Needed - PC Set 1 - 20MHz |  |
| --- | --- | --- | --- |
| Scenario | ISD victim | ISD aggressor=ISD victim | ISD aggressor from Scenario A1 |
| Urban | 750m | <=0.8dB | <=2dB |
| SubUrban | 2.8km | <=0.1dB | <=1.3dB |
| Rural | 6km | <=0.1dB | <=2dB |
| Rural | 8km | <=0.1dB | <=1.9dB |

## Scenario A2

This scenario is different compared to the one analysed in RAN4 #78 because of the more aggressive power control settings. The UE tx power distributions for scenario A2 are presented in Figure 3.

Figure 3. UE Tx power distributions for Scenario A2.

Table 6 summarizes the percentage of UEs transmitting more than 23dBm in Scenario A2. The table also shows a comparison with the PC set 1 adopted in the previous simulations. As it can be observed, the number of UEs exploiting power higher than 23dBm can go up to about 29%.

Table 6. Percentage of UEs transmitting more than 23dBm in Scenario A2.

|  |  | Percentage of UEs transmitting more than 23dBm |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Scenario | ISD victim | Set 1 | Set 4A | Set 4B | Set 4C |
| Urban | 750m | 6.78% | 9.99% | 19.32% | 25.32% |
| SubUrban | 2.8km | 3.91% | 10.97% | 18.03% | 23.76% |
| Rural | 6km | 3.93% | 11.28% | 21.42% | 27.90% |
| Rural | 8km | 4.18% | 11.77% | 22.21% | 28.72% |

A summary of the simulation results obtained is shown in Table 7. As it can be observed the ACLR tightening needed is always less than 2dB. A collection of all the detailed simulation results for scenario A2 is presented in Appendix B.

Table 7. ACLR tightening needed by power class 2 UEs in Scenario A2.

|  |  | ACLR Tightening Needed in Scenario A2 - 20MHz |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Scenario | ISD victim | Set 1 | Set 4A | Set 4B | Set 4C |
| Urban | 750m | <=0.8dB | <=1dB | <=1.9dB | <=2dB |
| SubUrban | 2.8km | <=0.1dB | <=0.5dB | <=1.1dB | <=1.3dB |
| Rural | 6km | <=0.1dB | <=1dB | <=1.5dB | <=2dB |
| Rural | 8km | <=0.1dB | <=1dB | <=1.3dB | <=1.6dB |

# Conclusion

In this contribution, we provided further simulation results for the coexistence analysis between HPUE operating in Band 41 and class 3 UEs operating in adjacent channel. The new results are obtained considering the new assumptions agreed through email discussion. By taking into account the collected data, we observed the following:

Observation 1: ACLR tightening required by class 2 UEs in scenario A1 and A2 is <=2dB.

# Reference

- R4-161488, “Band 41 HPUE Way Forward”, Qualcomm Incorporated, Sprint, China Telecom, China Unicom, CMCC.

- R4-161533, “Additional simulation assumptions for coexistence study on Band 41 HPUE”, China Telecom, Sprint, Nokia Networks.

- R4-161048, “Adjacent channel coexistence for High Power UE”, Qualcomm Incorporated.

- R4-156846, “Way Forward on System level simulation methodology and assumptions for coexistence study on new Band 41 UE power class supporting +26 dBm”, Sprint, Alcatel-Lucent.

- 3GPP TR 36.942 v13.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”.

- R4-157922, 3GPP TR 36.886, “3rd Generation Partnership Project; Technical Specification Group RAN; High Power UE for Band 41”.

- R4-158342, “HPUE simulation anomalies”, Sprint, Intel, CMCC, Qualcomm, Ericsson, Alcatel-Lucent.

# Throughput loss curves for Scenario A1

In the following sections we present a set of curves showing the mean and 5%-tile throughput degradation due to ACI obtained for Scenario A1. The detailed methodology adopted for this scenario is described in [2].

## Urban – ISD = 750m (26dBm aggressor ISD = 900m)

Figure 4. Throughput degradation due to ACI. Urban scenario – ISD=750m – PC set 1. ISD in the case of 26dBm aggressor system is 900m.

## SubUrban – ISD = 2.8km (26dBm aggressor ISD = 3.36km)

Figure 5. Throughput degradation due to ACI. SubUrban scenario – ISD=2.8km – PC set 1. ISD in the case of 26dBm aggressor system is 3.36km.

## Rural – ISD = 6km (26dBm aggressor ISD = 7.32km)

Figure 6. Throughput degradation due to ACI. SubUrban scenario – ISD=6km – PC set 1. ISD in the case of 26dBm aggressor system is 7.32m.

## Rural – ISD = 8km (26dBm aggressor ISD = 9.76km)

Figure 7. Throughput degradation due to ACI. SubUrban scenario – ISD=8km – PC set 1. ISD in the case of 26dBm aggressor system is 9.76m.

# Throughput loss curves for Scenario A2

In the following sections we present a set of curves showing the mean and 5%-tile throughput degradation due to ACI obtained for Scenario A2. The detailed methodology adopted for this scenario is described in [2].

## PC Set 4A

### Urban

Figure 8. Throughput degradation due to ACI. Urban scenario – ISD=750m – PC set 4A.

### SubUrban

Figure 9. Throughput degradation due to ACI. SubUrban scenario – ISD=2.8km – PC set 4A.

### Rural – ISD=6km

Figure 10. Throughput degradation due to ACI. Rural scenario – ISD=6km – PC set 4A.

### Rural – ISD=8km

Figure 11. Throughput degradation due to ACI. Rural scenario – ISD=8km – PC set 4A.

## PC Set 4B

### Urban

Figure 12. Throughput degradation due to ACI. Rural scenario – ISD=750m – PC set 4B.

### SubUrban

Figure 13. Throughput degradation due to ACI. SubUrban scenario – ISD=2.8km – PC set 4B.

### Rural – ISD=6km

Figure 14. Throughput degradation due to ACI. Rural scenario – ISD=6km – PC set 4B.

### Rural – ISD=8km

Figure 15. Throughput degradation due to ACI. Rural scenario – ISD=8km – PC set 4B.

## PC Set 4C

### Urban

Figure 16. Throughput degradation due to ACI. Rural scenario – ISD=750m – PC set 4C.

### SubUrban

Figure 17. Throughput degradation due to ACI. Rural scenario – ISD=2.8km – PC set 4C.

### Rural – ISD=6km

Figure 18. Throughput degradation due to ACI. Rural scenario – ISD=6km – PC set 4C.

### Rural – ISD=8km

Figure 19. Throughput degradation due to ACI. Rural scenario – ISD=8km – PC set 4C.