3GPP TSG-RAN WG4 Meeting #78bis 	R4-162591

# San José Del Cabo, México, April 9th –17th, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3549 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean up for CRS-IM related requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CRSIM-Perf |  |  |  |  |  |  | Date: |  | 2016-4-1 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The requirements for CRS-IM related test cases are still with bracket and some parameters setup is still with bracket or TBD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the bracket for the CRS-IM related requirements, and use the right number to replace TBD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CRS-IM related requirements will not be stable |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS36.521 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

##### 8.2.1.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information

The requirements are specified in Table 8.2.1.4.1E-2, with the addition of parameters in Table 8.2.1.4.1E-1. The purpose is to verify the closed loop rank-one performance with wideband precoding when CRS assistance information [7] is configured. In Table 8.2.1.4.1E-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.2.1.4.1E-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  |  | dB | 0 | 0 | 0 |
| at antenna port |  |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  |  | dB | Reference Value in Table 8.2.1.4.1E-2 | 10.45 | 4.6 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  |  | 0 | 1 | 128 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 |  |  |
| Number of control OFDM symbols |  |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  |  | 4 | N/A | N/A |
| Precoding granularity |  |  | PRB | 50 | N/A | N/A |
| PMI delay (Note 2) |  |  | ms | 8 | N/A | N/A |
| Reporting interval |  |  | ms | 1 | N/A | N/A |
| Peporting mode |  |  |  | PUCCH 1-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A | N/A |
| Cyclic prefix |  |  |  | Normal | Normal | Normal |
| Interference model |  |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | 80 | 80 |
|  | Rank 2 |  | % | N/A | 20 | 20 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). |  |  |  |  |  |  |

Table 8.2.1.4.1E-2: Minimum Performance Single-Layer Spatial Multiplexing (FRC)

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.10-3 FDD | OP.1FDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 2x2 low | 70 | 10.8 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

##### 8.2.2.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information

The requirements are specified in Table 8.2.2.4.1E-2, with the addition of parameters in Table 8.2.2.4.1E-1. The purpose is to verify the closed loop rank-one performance with wideband precoding when CRS assistance information [7] is configured. In Table 8.2.2.4.1E-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1, Cell 2 and Cell 3, respectively. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.2.2.4.1E-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC)

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink configuration |  |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  |  | 4 | 4 | 4 |
| Downlink power allocation |  |  | dB | -3 | -3 | -3 |
|  |  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  |  | dB | 0 | 0 | 0 |
| at antenna port |  |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  |  | dB | Reference Value in Table 8.2.2.4.1E-2 | 10.45 | 4.6 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  |  | 0 | 1 | 128 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 |  |  |
| Number of control OFDM symbols |  |  |  | 2 | 2 | 2 |
| Interference model |  |  |  | N/A | As specified in clause B.5.3 | As specified in clause B.5.3 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | 80 | 80 |
|  | Rank 2 |  | % | N/A | 20 | 20 |
| ACK/NACK feeback mode |  |  |  | Multiplexing | N/A | N/A |
| PDSCH transmission mode |  |  |  | 4 | N/A | N/A |
| Precoding granularity |  |  | PRB | 50 | N/A | N/A |
| PMI delay (Note 2) |  |  | ms | 10 or 11 | N/A | N/A |
| Reporting interval |  |  | ms | 1 or 4 (Note 3) | N/A | N/A |
| Peporting mode |  |  |  | PUCCH 1-1 | N/A | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A | N/A |
| Cyclic prefix |  |  |  | Normal | Normal | Normal |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. |  |  |  |  |  |  |

Table 8.2.2.4.1E-2: Minimum Performance Single-Layer Spatial Multiplexing (FRC)

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.10-3 TDD | OP.1TDD | N/A | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 11.2 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

#### 8.3.1.1G	Single-layer Spatial Multiplexing (CRS assistance information is configured)

The requirements are specified in Table 8.3.1.1G-2, with the addition of parameters in Table 8.3.1.1G-1. The purpose is to verify the performance of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell with CRS assistance information. In Table 8.3.1.1G-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1, Cell2 and Cell 3 is according to Annex C.3.2. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.3.1.1G-1: Test parameters of TM9-Single-Layer (2 CSI-RS ports)

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB | 0 | 0 | 0 |  |
|  |  |  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |  |
|  |  |  |  | dB | -3 | -3 | -3 |  |
| at antenna port |  |  |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  |  |  | dB | Reference Value in Table 8.3.1.1G-2 | 10.45 | 4.6 |
| BWChannel |  |  |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  |  |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  |  |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  |  |  | 0 | 1 | 128 |
| Cell-specific reference signals |  |  |  |  | Antenna ports 0,1 |  |  |
| CSI reference signals |  |  |  |  | Antenna ports 15,16 | N/A | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  |  | Subframes | 5 / 2 | N/A | N/A |
| CSI reference signal configuration |  |  |  |  | 8 | N/A | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  |  | Subframes / bitmap | 3 / 0010000000000000 | N/A | N/A |
| Number of control OFDM symbols |  |  |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  |  |  | TM9-1layer | N/A | N/A |
| Precoding granularity |  |  |  |  | Frequency domain: 1 PRG Time domain: 1 ms | N/A | N/A |
| Beamforming model |  |  |  |  | Annex B.4.1 | N/A | N/A |
| Interference model |  |  |  |  | N/A | As specified in clause B.5.4 | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  |  |  | % |  | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  | % | N/A | 80 | 80 |
|  |  | Rank 2 |  | % | N/A | 20 | 20 |
| Cyclic prefix |  |  |  |  | Normal | Normal | Normal |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 4: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. Note 5: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. |  |  |  |  |  |  |  |

Table 8.3.1.1G-2: Minimum Performance of TM9-Single-Layer (2 CSI-RS ports)

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.51-1 FDD | OP.1FDD | OP.1FDD | OP.1FDD | EVA5 |  |  | 2x2 Low | 70 | 11.6 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

##### 8.3.1.3.4	Minimum requirement with Different Cell ID and non-colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)

The requirements are specified in Table 8.3.1.3.4-3, with the additional parameters in Table 8.3.1.3.4-1 and Table 8.3.1.3.4-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where three transmission points have different Cell ID and non-colliding CRS. In particular the test verifies that the UE, configured with quasi co-location type B,  performs correct tracking and compensation of the frequency difference between two transmission points, channel parameters estimation and rate matching behaviour according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ signalling defined in [6]. Further, the test verifies that the UE, configured with the CRS assistance information [7], can mitigate interference from CRS for demodulation.  The CRS assistance information [7] includes TP 3. In Table 8.3.1.3.4-1, transmission point 1 (TP 1) is serving cell transmitting PDCCH, synchronization signals and PBCH, transmission point 2 (TP 2) transmits PDSCH with different Cell ID, and Transmission point 3 (TP 3) is the aggressor transmission point. The downlink physical channel setup for TP 1 is according to Table C.3.4-1, for TP 2 is according to Table C.3.4-2, and for TP 3 is according to Annex C.3.2.

Table 8.3.1.3.4-1: Test Parameters for quasi co-location type B with different Cell ID and non-Colliding CRS when CRS assistance information is configured

| parameter |  |  | Unit | TP 1 | TP 2 | TP 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Beamforming model |  |  |  | N/A | Port 7 as specified in clause B.4.1 | N/A |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  |  | N/A | Antenna ports {15,16} | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | N/A | 5 / 2 | N/A |
| CSI reference signal 0 configuration |  |  |  | N/A | 0 | N/A |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  |  | Subframes/bitmap | N/A | 2/ 0010000000000000 | N/A |
|  |  |  | dB | 10.45 | Reference Value in Table 8.3.1.3.4-3 | 8.45 |
| at antenna port |  |  | dBm/15kHz | -98 | -98 | N/A |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 128 |
| Number of control OFDM symbols |  |  |  | 1 | 2 | 2 |
| Timing offset between TPs |  |  | us | N/A | -0.5 | 3 |
| Frequency offset between TPs |  |  | Hz | N/A | 200 | -100 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  |  | Type B, ‘00’ |  | N/A |
| PDSCH transmission mode |  |  |  | Blank | 10 | 9 |
| Number of allocated resource block |  |  |  | N/A | 50 | N/A |
| Symbols for unused PRBs |  |  |  | N/A | OCNG(Note2) | N/A |
| Interference model |  |  |  | N/A | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | N/A | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | N/A | 80 |
|  | Rank 2 |  | % | N/A | N/A | 20 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |  |  |

Table 8.3.1.3.4-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | Blanked | PDSCH |

Table 8.3.1.3.4-3: Performance Requirements for quasi co-location type B with different Cell ID and non-Colliding CRS when CRS assistance information is configured

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP 1 | TP 2 | TP3 | TP 1 | TP 2 | TP3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.52-1 FDD | N/A | OP.1FDD | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 10.8 | ≥2 |
| Note 1: The propagation conditions for TP.1, TP.2 and TP.3 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP.1, TP.2 and TP.3. Note 3: SNR corresponds to of TP.2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.3.1.3.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)

The requirements are specified in Table 8.3.1.3.5-3, with the additional parameters in Tables 8.3.1.3.5-1 and 8.3.1.3.5-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where three transmission points  have different Cell ID and non-colliding CRS. In particular the test verifies that the UE, configured with quasi co-location type B,  performs correct tracking and compensation of the timing difference between two transmission points, channel parameters estimation and rate matching according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ (PQI) signalling defined in [6]. Further, the test verifies that the UE, configured with the CRS assistance information [7], can mitigate interference from CRS for demodulation. The CRS assistance information [7] includes TP 3. In Tables 8.3.1.3.5-1 and 8.3.1.3.5-2, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals and PBCH, Transmission point 2 (TP 2) has different Cell ID as TP 1, and Transmission point 3 (TP 3) is the aggressor transmission point. Multiple NZP CSI-RS resources and ZP CSI-RS resources are configured. In each sub-frame, DL PDSCH transmission is dynamically switched between TP 1 and TP 2 with multiple PDSCH RE Mapping and Quasi-Co-Location Indicator configuration (PQI). Configurations of PDSCH RE Mapping and Quasi-Co-Location Indicator and downlink transmission hypothesis are defined in Table 8.3.1.3.5-2. The downlink physical channel setup for TP 1 is according to Table C.3.4-1,  for TP 2 is according to Table C.3.4-2, and for TP 3 is according to Annex C.3.2.

Table 8.3.1.3.5-1: Test Parameters DPS transmission with CRS assistance information

| parameter |  |  | Unit | TP 1 | TP 2 | TP 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Beamforming model |  |  |  | As specified in clause B.4.1 | As specified in clause B.4.1 | N/A |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  |  | Antenna ports {15,16} | N/A | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 5 / 2 | N/A | N/A |
| CSI reference signal 0 configuration |  |  |  | 0 | N/A | N/A |
| CSI reference signals 1 |  |  |  | N/A | Antenna ports {15,16} | N/A |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | N/A | 5 / 2 | N/A |
| CSI reference signal 1 configuration |  |  |  | N/A | 8 | N/A |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  |  | Subframes/bitmap | 2/ 0010000000000000 | N/A | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmapS |  |  | Subframes/bitmap | N/A | 2/ 0000010000000000 | N/A |
| (Note 2) |  |  | dB | Reference Value in Table 8.3.1.3.5-3 | Reference Value in Table 8.3.1.3.5-3 | 8.45 |
| at antenna port |  |  | dBm/15kHz | -98 | -98 | N/A |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 128 |
| Number of control OFDM symbols |  |  |  | 2 | 2 | 2 |
| Timing offset between TPs |  |  |  | N/A | -0.5 | 3 |
| Frequency offset between TPs |  |  | Hz | N/A | 200 | -100 |
| Number of allocated resource blocks |  |  | PRB | 50 | 50 | N/A |
| PDSCH transmission mode |  |  |  | 10 | 10 | 9 |
| Probability of occurrence of PDSCH transmission(Note 3) |  |  | % | 30 | 70 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 4) | OCNG (Note 4) | N/A |
| Interference model |  |  |  | N/A | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | N/A | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | N/A | 80 |
|  | Rank 2 |  | % | N/A | N/A | 20 |
| Note 1: Note 2: of TP1 is set the same as that of TP2 Note 3: PDSCH transmission from TPs shall be randomly determined independently for each subframe. Probabilities of occurrence of PDSCH transmission from TPs are specified. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |  |  |

Table 8.3.1.3.5-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blanked |
| PQI set 1 | CSI-RS 1 | ZP CSI-RS 1 | Blanked | PDSCH |

Table 8.3.1.3.5-3: Performance Requirements DPS transmission with CRS assistance information

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP 1 | TP 2 | TP3 | TP 1 | TP 2 | TP3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.52-1 FDD | OP.1FDD | OP.1FDD | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 10.7 | ≥2 |
| Note 1: The propagation conditions for TP.1, TP.2 and TP.3 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP.1, TP.2 and TP.3. Note 3: SNR corresponds to of TP.2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

#### 8.3.2.1H	Single-layer Spatial Multiplexing (CRS assistance information is configured)

The requirements are specified in Table 8.3.2.1H-2, with the addition of parameters in Table 8.3.2.1H-1. The purpose is to verify the performance of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell with CRS assistance information. In Table 8.3.2.1H-1, Cell 1 is the serving cell, and Cell 2 and Cell 3 are the aggressor cells. The downlink physical channel setup for Cell 1, Cell 2 and Cell 3 is according to Annex C.3.2. The CRS assistance information [7] includes Cell 2 and Cell 3.

Table 8.3.2.1H-1: Test parameters of TM9-Single-Layer (2 CSI-RS ports)

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Uplink downlink Configuration |  |  |  | 1 | 1 | 1 |
| Special subframe configuration |  |  |  | 4 | 4 | 4 |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
|  |  |  | dB | -3 | -3 | -3 |
| at antenna port |  |  | dBm/15kHz | -98 | N/A | N/A |
| Es/Noc |  |  | dB | Reference Value in Table 8.3.2.1H-2 | 10.45 | 4.6 |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Subframe Configuration |  |  |  | Non-MBSFN | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  |  | s | N/A | 3 | -1 |
| Frequency shift between Cells |  |  | Hz | N/A | 300 | -100 |
| Cell Id |  |  |  | 0 | 1 | 126 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 |  |  |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 5 / 4 | N/A | N/A |
| CSI reference signal configuration |  |  |  | 8 | N/A | N/A |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  |  | Subframes / bitmap | 4 / 0010000000000000 | N/A | N/A |
| Number of control OFDM symbols |  |  |  | 2 | 2 | 2 |
| PDSCH transmission mode |  |  |  | TM9-1layer | N/A | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  |  | % |  | 20 | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | 80 | 80 |
|  | Rank 2 |  | % | N/A | 20 | 20 |
| Precoding granularity |  |  |  | Frequency domain: 1 PRG Time domain: 1 ms | N/A | N/A |
| Beamforming model |  |  |  | Annex B.4.1 | N/A | N/A |
| Cyclic prefix |  |  |  | Normal | Normal | Normal |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: The number of the CRS ports in Cell 1, Cell 2 and Cell 3 is the same. Note 5: SIB-1 will not be transmitted in Cell 2 and Cell 3 in this test. Note 6: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. |  |  |  |  |  |  |

Table 8.3.2.1H-2: Minimum Performance of TM9-Single-Layer (2 CSI-RS ports)

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.51-1 TDD | OP.1TDD | OP.1TDD | OP.1TDD | EVA5 |  |  | 2x2 Low | 70 | 11.9 | ≥2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to Es/Noc of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

##### 8.3.2.4.4	Minimum requirement with Different Cell ID and non-Colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)

The requirements are specified in Table 8.3.2.4.4-3, with the additional parameters in Table 8.3.2.4.4-1 and Table 8.3.2.4.4-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where three transmission points have different Cell ID and non-colliding CRS. In particular the test verifies that the UE, configured with quasi co-location type B,  performs correct tracking and compensation of the frequency difference between two transmission points, channel parameters estimation and rate matching behaviour according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ signalling defined in [6]. Further, the test verifies that the UE, configured with the CRS assistance information [7], can mitigate interference from CRS for demodulation. The CRS assistance information [7] includes TP 3. In Table 8.3.2.4.4-1, transmission point 1 (TP 1) is serving cell transmitting PDCCH, synchronization signals and PBCH, transmission point 2 (TP 2) transmits PDSCH with different Cell ID, and Transmission point 3 (TP 3) is the aggressor transmission point. The downlink physical channel setup for TP 1 is according to Table C.3.4-1, for TP 2 is according to Table C.3.4-2, and for TP 3 is according to Annex C.3.2.

Table 8.3.2.4.4-1: Test Parameters for quasi co-location type B with different Cell ID and non-colliding CRS when CRS assistance information is configured

| Parameter |  |  | Unit | TP 1 | TP 2 | TP 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Beamforming model |  |  |  | N/A | Port 7 as specified in clause B.4.1 | N/A |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  |  | N/A | Antenna ports {15,16} | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | N/A | 5 / 4 | N/A |
| CSI reference signal 0 configuration |  |  |  | N/A | 0 | N/A |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  |  | Subframes/bitmap | N/A | 4/ 0010000000000000 | N/A |
|  |  |  | dB | 10.45 | Reference Value in Table 8.3.2.4.4-3 | 8.45 |
| at antenna port |  |  | dBm/15kHz | -98 | -98 | N/A |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 128 |
| Number of control OFDM symbols |  |  |  | 1 | 2 | 2 |
| Timing offset between TPs |  |  | us | N/A | -0.5 | 3 |
| Frequency offset between TPs |  |  | Hz | N/A | 200 | -100 |
| qcl-Operation, ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ |  |  |  | Type B, ‘00’ |  | N/A |
| PDSCH transmission mode |  |  |  | Blank | 10 | 9 |
| Number of allocated resource block |  |  |  | N/A | 50 | N/A |
| Symbols for unused PRBs |  |  |  | N/A | OCNG(Note2) | N/A |
| Interference model |  |  |  | N/A | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | N/A | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | N/A | 80 |
|  | Rank 2 |  | % | N/A | N/A | 20 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |  |  |

Table 8.3.2.4.4-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | Blanked | PDSCH |

Table 8.3.2.4.4-3: Performance Requirements for quasi co-location type B with different Cell ID and non-Colliding CRS when CRS assistance information is configured

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP 1 | TP 2 | TP3 | TP 1 | TP 2 | TP3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.52-1 TDD | N/A | OP.1TDD | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 11.1 | ≥2 |
| Note 1: The propagation conditions for TP.1, TP.2 and TP.3 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP.1, TP.2 and TP.3. Note 3: SNR corresponds to of TP.2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |

##### 8.3.2.4.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)

The requirements are specified in Table 8.3.2.4.5-3, with the additional parameters in Tables 8.3.2.4.5-1 and 8.3.2.4.5-2. The purpose of this test is to verify the UE capability of supporting non quasi-colocated antenna ports when the UE receives DCI format 2D in a scenario where three transmission point have the different Cell ID and non-colliding CRS. In particular the test verifies that the UE, configured with quasi co-location type B, performs correct tracking and compensation of the timing difference between two transmission points, channel parameters estimation and rate matching according to the ‘PDSCH RE Mapping and Quasi-Co-Location Indicator’ (PQI) signalling defined in [6]. Further, the test verifies that the UE, configured with the CRS assistance information [7], can mitigate interference from CRS for demodulation. The CRS assistance information [7] includes TP 3. In Tables 8.3.2.4.5-1 and 8.3.2.4.5-2, transmission point 1 (TP 1) is the serving cell transmitting PDCCH, synchronization signals and PBCH, Transmission point 2 (TP 2) has different Cell ID as TP 1, and Transmission point 3 (TP3) is the aggressor transmission point. Multiple NZP CSI-RS resources and ZP CSI-RS resources are configured. In each sub-frame, DL PDSCH transmission is dynamically switched between TP 1 and TP 2 with multiple PDSCH RE Mapping and Quasi-Co-Location Indicator configuration (PQI). Configurations of PDSCH RE Mapping and Quasi-Co-Location Indicator and downlink transmission hypothesis are defined in Table 8.3.2.4.5-2. The downlink physical channel setup for TP 1 is according to Table C.3.4-1,  for TP 2  is according to Table C.3.4-2, and for TP 3 is according to Annex C.3.2

Table 8.3.2.4.5-1: Test Parameters for DPS transmission with CRS assistance information

| parameter |  |  | Unit | TP 1 | TP 2 | TP 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 | 0 |
|  |  |  | dB | -3 | -3 | -3 |
| Beamforming model |  |  |  | As specified in clause B.4.1 | As specified in clause B.4.1 | N/A |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals 0 |  |  |  | Antenna ports {15,16} | N/A | N/A |
| CSI-RS 0 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 5 / 4 | N/A | N/A |
| CSI reference signal 0 configuration |  |  |  | 0 | N/A | N/A |
| CSI reference signals 1 |  |  |  | N/A | Antenna ports {15,16} | N/A |
| CSI-RS 1 periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | N/A | 5 / 4 | N/A |
| CSI reference signal 1 configuration |  |  |  | N/A | 8 | N/A |
| Zero-power CSI-RS 0 configuration ICSI-RS / ZeroPower CSI-RS bitmap |  |  | Subframes/bitmap | 4/ 0010000000000000 | N/A | N/A |
| Zero-power CSI-RS1 configuration ICSI-RS / ZeroPower CSI-RS bitmapS |  |  | Subframes/bitmap | N/A | 4/ 0000010000000000 | N/A |
| (Note 2) |  |  | dB | Reference Value in Table 8.3.2.4.5-3 | Reference Value in Table 8.3.2.4.5-3 | 8.45 |
| at antenna port |  |  | dBm/15kHz | -98 | -98 | N/A |
| BWChannel |  |  | MHz | 10 | 10 | 10 |
| Cyclic Prefix |  |  |  | Normal | Normal | Normal |
| Cell Id |  |  |  | 0 | 1 | 128 |
| Number of control OFDM symbols |  |  |  | 2 | 2 | 2 |
| Timing offset between TPs |  |  |  | N/A | -0.5 | 3 |
| Frequency offset between TPs |  |  | Hz | N/A | 200 | -100 |
| Number of allocated resource blocks |  |  | PRB | 50 | 50 | N/A |
| PDSCH transmission mode |  |  |  | 10 | 10 | 9 |
| Probability of occurrence of PDSCH transmission(Note 3) |  |  | % | 30 | 70 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 4) | OCNG (Note 4) | N/A |
| Interference model |  |  |  | N/A | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission in interference cells |  |  | % | N/A | N/A | 20 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | N/A | 80 |
|  | Rank 2 |  | % | N/A | N/A | 20 |
| Note 1: Note 2: of TP 1 is set the same as that of TP 2 Note 3: PDSCH transmission from TPs shall be randomly determined independently for each subframe. Probabilities of occurrence of PDSCH transmission from TPs are specified. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |  |  |

Table 8.3.2.4.5-2: Configurations of PQI and DL transmission hypothesis for each PQI set

| PQI set index | Parameters in each PQI set |  | DL transmission hypothesis for each PQI Set |  |
| --- | --- | --- | --- | --- |
|  | NZP CSI-RS Index (For quasi co-location) | ZP CSI-RS configuration | TP 1 | TP 2 |
| PQI set 0 | CSI-RS 0 | ZP CSI-RS 0 | PDSCH | Blanked |
| PQI set 1 | CSI-RS 1 | ZP CSI-RS 1 | Blanked | PDSCH |

Table 8.3.2.4.5-3: Performance Requirements for DPS transmission with CRS assistance information

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TP 1 | TP 2 | TP3 | TP 1 | TP 2 | TP3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.52-1 TDD | OP.1TDD | OP.1TDD | N/A | EVA5 | EVA5 | EVA5 | 2x2 Low | 70 | 11.2 | ≥2 |
| Note 1: The propagation conditions for TP.1, TP.2 and TP.3 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of TP.1, TP.2 and TP.3. Note 3: SNR corresponds to of TP.2 as defined in clause 8.1.1. |  |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------------------End of Change ------------------------------------------------------------