TSG-RAN Working Group 4 (Radio) meeting #78bis		R4-162995

San Jose del Cabo, Mexico, 11 - 15 April, 2016

Source: 	Ericsson

Title:	Regulatory requirements for V2V in unlicensed bands

Agenda Item:	7.13.1

Document for:	Discussion

# Introduction

One of the objectives of the V2V sidelink WI in [1] is

- To specify UE Tx and Rx RF requirement covering operations at up to 6 GHz carrier [RAN4]

Two different scenarios are under consideration for V2V, licensed and unlicensed spectrum. In this contribution, we look at the RF regulatory requirements for ITS in the unlicensed bands. In particular, we look at the ETSI harmonized standard for ITS in Europe. The summary below is based on a draft version of  ETSI EN 302 571 V2.0.0 (2016-03) [2]. This is currently under approval procedure and will be the next version of the ETSI HS by the end of 2016.

# Disscussion

In Europe, Intelligent Transport Systems (ITS) is specified in the ETSI HS between 5 855 MHz to 5 925 MHz, see Table 1. ECC Decision (08)01 [3] defines the spectrum usage conditions in the frequency range 5 875 GHz to 5 905 MHz for non-safety ITS and proposes CEPT administrations to consider within a future review of the Decision to include the designation of the frequency sub-band 5905-5925 MHz for an extension of ITS spectrum. ECC Recommendation (08)01 [4] recommends the spectrum usage of the frequency range 5 855 MHz to 5 875 MHz for non-asfety ITS. The Commission Decision 2008/671/EC [5] mandates a harmonized use of the frequency band 5 875 MHz to 5 905 MHz dedicated to safety-related ITS applications throughout the member states of the European Union.

Table 1 5 GHz ITS frequency band segmentation in Europe

| Frequency band | Frequency range | Usage | Regulation |
| --- | --- | --- | --- |
| ITS-G5B | 5 855 MHz to 5 875 MHz | ITS non-safety applications | ECC Recommendation (08)01 [4] |
| ITS-G5A | 5 875 MHz to 5 905 MHz | ITS road safety | Commission Decision 2008/671/EC [5], ECC Decision (08)01 [3] |
| ITS-G5D | 5 905 MHz to 5 925 MHz | Future ITS applications | ECC Decision (08)01 [3] |

In the following, we review the regulatory comformance requirements specified in the harmonized standard EN 302 571 [2], which applies to radio transmitters and receivers for ITS operating in the frequency range 5 855 MHz to 5 925 MHz in Europe.

## TX requirements

### RF output power

The radio frequency (RF) output power is the mean equivalent isotropically radiated power (e.i.r.p.) during transmission bursts.

The maximum RF output power shall not exceed 33 dBm e.i.r.p.

### Power spectral density

The power spectral density (PSD) is the mean e.i.r.p. spectral density during transmission bursts.

The maximum power spectral density shall not exceed 23 dBm/MHz e.i.r.p.

### Transmit power control

The TPC range shall at least be 3 dBm up to the maximum specified RF output power e.i.r.p of the equipment.

### Transmitter frequency stability

The nominal carrier frequencies, fc , for ITS channels in the frequency range 5 855 MHz to 5 925 MHz are 5860 MHz, 5870 MHz, 5880 MHz, 5890 MHz, 5900 MHz, 5910 MHz and 5920 MHz. The maximum channel bandwidth is 10 MHz

The equipment is required to operate on the applicable specific carrier centre frequencies and the actual carrier centre frequency for any given channel shall be maintained within the range fc ± 20 ppm.

### Transmitter unwanted emissions

Unwanted emissions refer to radio frequency emissions outside the 5 GHz ITS frequency band, i.e. outside of 5 855 MHz to 5 925 MHz.

Table 2 and Table 3 tabulate transmitter unwanted emission limits in the spurious domain below 1 GHz and above 1 GHz, respectively.

Table 2. Transmitter unwanted emission limits in the spurious domain below 1 GHz

| Frequency range | Maximum power, (e.r.p.) (dBm) | Reference bandwidth |
| --- | --- | --- |
| 30 MHz f 1 GHz | -36 | 100 kHz |

Table 3. Transmitter unwanted emissions in the spurious domain above 1 GHz

| Frequency range | Maximum power, (e.i.r.p.) (dBm) | Bandwidth |
| --- | --- | --- |
| 1 GHz to 5,795 GHz | -30 dBm | 1 MHz |
| 5,950 GHz to 18 GHz | -30 dBm | 1 MHz |

Table 5 tabluates transmitter unwanted emission limits in the out-of-band domain of the 5 GHz ITS frequency band.

The out-of-band domain is defined as ±250 % of the channel bandwidth. and then the out-of-band (OOB) domain for the 5 GHz ITS frequency band is 5,830 GHz to 5,855 GHz at the lower part and 5,925 GHz to 5,950 GHz at the higher part of the frequency band. However, the out-of-band domain is larger for the 5 GHz ITS frequency band and is including parts of the spurious domain (i.e. 5795-5830MHz), see Table 4.

Table 4. Transmitter unwanted emission limits in the out-of-band domain of the 5 GHz ITS frequency band

| Frequency range | Maximum power, (e.i.r.p.) (dBm) | Reference bandwidth |
| --- | --- | --- |
| 5,795 GHz to 5,815 GHz | -65 ([7], Clause 4.3) | 1 MHz |
| 5,815 GHz to 5,855 GHz | -30 | 1 MHz |
| 5,925 GHz to 5,950 GHz | -30 | 1 MHz |

### Transmitter spectrum mask within the 5 GHz ITS frequency band for 10 MHz channels

The spectrum mask in Table 5 applies within 5 855 MHz to 5 950 MHz according to ETSI EN 302 571 [2]. .We note that there may be a typo in the applicable range and this may be 855 MHz to 5 925 MHz

Table 5. Transmitter spectrum mask for 10 MHz channel bandwidth

| Power Spectral Density at the carrier centre frequency fc (dBm/100 kHz) | ± 4,5 MHz offset (dBm/100 kHz) | ± 5,0 MHz offset (dBm/100 kHz) | ± 5,5 MHz offset (dBm/100 kHz) | ± 10 MHz offset (dBm/100 kHz) | ± 15 MHz offset (dBm/100 kHz) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | -26 | -32 | -40 | -50 |

Conclusion 1: the TX requirements defined by ETSI need to be taken into account when specifying the 3GPP requirements for the ITS spectrum band

## RX requirements

### Receiver spurious emissions

Receiver spurious emissions are emissions at any frequency when the equipment is in receive mode.

Table 6 tablulates the limits that the receiver shall not exceed.

Table 6. Spurious radiated emission limits

| Frequency range | Maximum power | Measurement bandwidth | Special requirement |
| --- | --- | --- | --- |
| 30 MHz f 1 GHz | -57 dBm (e.r.p.) | 100 kHz | n/a |
| 1 GHz f 5,795 GHz | -47 dBm (e.i.r.p.) | 1 MHz | n/a |
| 5,795 GHz f 5,815 GHz | -60 dBm (e.i.r.p.) | 1 MHz | Applicable for an intended antenna installation ≤ 2 meters above the ground level. |
|  | -65 dBm (e.i.r.p.) | 1 MHz | Applicable for an intended antenna installation > 2 meters above the ground level. |
| 5,815 GHz f 18 GHz | -47 dBm (e.i.r.p.) | 1 MHz | n/a |

### Receiver selectivity

#### Adjacent channel rejection

The adjacent channel rejection is a measure of the capability of the receiver to operate satisfactorily in the presence of a signal in the adjacent channel, which differs in frequency from the wanted signal by ± 10 MHz.

The harmonized standard EN 302 571 [2] only provides the minimum required adjacent channel rejection parameters under specified conditions for receivers using the OFDM PHY specified in [6]

#### Alternate channel rejection

The alternate channel rejection is a measure of the capability of the receiver to operate satisfactorily in the presence of a signal in the alternate adjacent channel, which differs in frequency from the wanted signal by ± 20 MHz.

The harmonized standard EN 302 571 [2] only provides the minimum required alternate channel rejection parameters under specified conditions for receivers using the OFDM PHY specified in [6],

#### Blocking

Blocking is a measure of the capability of the receiver to operate satisfactorily in the presence of a signal in frequency band further away and it shall be tested at ± 50 MHz, ± 100 MHz, and ± 200 MHz. Blocking testing shall be performed at least at 6 different frequency offset positions. The manufacturer of the equipment can add additional frequency offsets positions.

The blocking level shall not be less than -30 dBm.

### Receiver sensitivity

The receiver sensitivity is defined as the minimum receive signal level at the antenna connector required for a given packet error rate and modulation scheme (noise factor of 10 dB and 5 dB implementation margins are assumed).

The harmonized standard EN 302 571 [2] only provides the receiver sensitivity requirements for receivers using the OFDM PHY specified in [6]

Conclusion 2: The RX selectivity and sensitivity as defined by the ETSI ITS HS considers OFDM PHY receivers. RAN4 needs to analyse and compare these with the corresponding LTE-based ITS requirements.

# Conclusion

In this contribution, we present a summary of the regulatory requirements for the 5.9GHz ITS band in Europe defined in Draft ETSI EN 302 571 V2.0.2 (2016-01). This is currently under approval and is expected to be made available publicly by the end of 2016

Conclusion 1: the TX requirements defined by ETSI need to be taken into account when specifying the 3GPP requirements for the ITS spectrum band

Conclusion 2: The RX selectivity and sensitivity as defined by the ETSI ITS HS considers OFDM PHY receivers. RAN4 needs to analyse and compare these with the corresponding LTE-based ITS requirements.

# References

- RP-152293, “New WI proposal: Support for V2V services based on LTE sidelink”, LG Electronics, Huawei, HiSilicon, CATT, CATR

- Draft ETSI EN 302 571 V2.0.0 (2016-03): Intelligent Transport Systems (ITS); Radiocommunications equipment operating  in the 5 855 MHz to 5 925 MHz frequency band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU

- ECC Decision (08)01: "ECC Decision of 14 March 2008 on the harmonised use of the 5875-5925 frequency band for Intelligent Transport Systems (ITS)", approved 14 March 2008 and amended 3 July 2015.

- ECC Recommendation (08)01:"Use of band 5855-5875 MHz for Intelligent Transport Systems (ITS)," approved 21 February 2008 and amended 3 July 2015.

- Commission Decision 2008/671/EC of 5 August on the harmonised use of radio spectrum in the 5875-5905 MHz frequency band for safety related application of Intelligent Transport Systems (ITS).

- IEEE 802.11™-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

- ETSI TS 102 792 (V1.2.1) (06-2015): "Intelligent Transport Systems (ITS); Mitigation techniques to avoid interference between European CEN Dedicated Short Range Communication (CEN DSRC) equipment and Intelligent Transport Systems (ITS) operating in the 5 GHz frequency range".

- ETSI TS 102 687 (V1.1.1) (07-2011): "Intelligent Transport Systems (ITS); Decentralized Congestion Control Mechanisms for Intelligent Transport Systems operating in the 5 GHz range; Access layer part".

- ETSI EN 300 674 (parts 1, 2-1 and 2-2): "Electromagnetic compatibility and Radio spectrum Matters (ERM); Road Transport and Traffic Telematics (RTTT); Dedicated Short Range Communication (DSRC) transmission equipment (500 kbit/s / 250 kbit/s) operating in the 5,8 GHz Industrial, Scientific and Medical (ISM) band".

- ETSI ES 200 674-1 (V2.4.1): "Intelligent Transport Systems (ITS); Road Transport and Traffic Telematics (RTTT); Dedicated Short Range Communications (DSRC); Part 1: Technical characteristics and test methods for High Data Rate (HDR) data transmission equipment operating in the 5,8 GHz Industrial, Scientific and Medical (ISM) band".

- ETSI EN 302 637-2: "Intelligent Transport Systems (ITS); Vehicular Communications; Basic Set of Applications; Part 2: Specification of Cooperative Awareness Basic Service"

- ETSI TS 102 687 (V1.1.1) (07-2011): "Intelligent Transport Systems (ITS); Decentralized Congestion Control Mechanisms for Intelligent Transport Systems operating in the 5 GHz range; Access layer part".

- ETSI TS 103 175 (V1.1.1) (06-2015): "Intelligent Transport Systems (ITS); Cross Layer DCC Management Entity for operation in the ITS G5A and ITS G5B medium".