3GPP TSG-RAN WG4 Meeting #79	R4-163184

Nanjing, China, 23 - 27 May 2016

Agenda item:			6.5.4

Source:	Nokia

Title:	Cat-M1 PUCCH Practical Simulation Results

Document for:		Discussion

# Introduction

The PUCCH simulation assumptions for supporting Cat-M UEs were agreed in RAN4#78 [1]. In RAN4#78bis, we provided the ideal Cat-M PUCCH link level simulation results based on the agreed PUCCH simulation assumptions. In this paper, we present the practical Cat-M PUCCH link level simulation results.

# Simulation Assumptions

The simulation assumptions for Cat-M PUCCH requirements were provided in [1] (as shown in Table 1) for

- ACK missed detection requirements for single user PUCCH format 1a

- CQI performance requirements for PUCCH format 2

For the simulation of the ACK missed detection requirements, the false alarm (DTXACK) rate shall not exceed 1% and the ACK missed detection (ACKNAK, DTX) probability shall not exceed 1%. For the simulation of the CQI performance requirements, the CQI block error probability (BLER) is defined as the conditional probability of incorrectly decoding the CQI information when the CQI information is sent. All CQI information shall be decoded (no exclusion due to DTX). The CQI information bit payload per sub-frame is equal to 4 bits. The CQI block error probability shall not exceed 1% at the given SNR.

Table 1.	Cat-M PUCCH simulation assumptions [1]

| Parameters | Values |
| --- | --- |
| PUCCH Format | PUCCH Format 1a, Format 2 |
| Propagation conditions | EPA5 (PUCCH Format 1a) EVA5 (Format 2) |
| Number of TX antennas | 1 |
| Number of RX antennas | 2 (Format 1a) 2(Format 2) |
| Antenna correlation | Low |
| Noise model | AWGN |
| Channel bandwidth (MHz) | 3, 5, 10, 15, 20 |
| Number of PRBs for PUCCH | 1 (edge PRB of the channel bandwidth) |
| Cyclic prefix | Normal (PUCCH Format 1a, Format 2) |
| DM RS sequence group hopping | Disabled |
| Channel estimation | Practical channel estimation and noise estimation |
| Cyclic shift separation | 2 |
| Number of users | single user PUCCH format 1a and PUCCH format 2 |
| Payload per PUCCH transmission | 4 bits (PUCCH format 2) |
| Number of PUCCH repetitions | {4,8,32} |
| Frequency Hopping Intervals | FDD: 4 TDD: 5 |
| Note 1: Assume no scheduling request for PUCCH format 1a |  |

# Simulation Results

Table 2 provides a template for the simulation results for Cat-M PUCCH format 1a.

Table 2 Minimum requirements for single user PUCCH format 1a, 1Tx

| Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Repetitions | Channel Bandwidth / SNR [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 2 | normal | EPA5 Low | 4 | -7.64 | -7.66 | -7.98 | -7.57 | -7.94 |
|  |  |  | 8 | -12.23 | -13.10 | -13.20 | -13.09 | -13.25 |
|  |  |  | 32 | -15.48 | -17.22 | -17.33 | -17.30 | -17.48 |

Table 3 Minimum requirements for single user PUCCH format 2, 1Tx

| Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Repetitions | Channel Bandwidth / SNR [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 2 | normal | EVA5 Low | 4 | -6.85 | -6.50 | -6.97 | -6.94 | -6.62 |
|  |  |  | 8 | -11.22 | -11.57 | -11.62 | -11.53 | -11.45 |
|  |  |  | 32 | -14.94 | -15.47 | -15.28 | -15.34 | -15.23 |

# Summary

In this contribution, we provided the practical simulation results for Cat-M PUCCH demodulation performance. We suggest these results be considered in defining the Cat-M1 PUSCH performance requirements.

# References

- R4-161228	Cat-M PUCCH Simulation Assumptions, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Ericsson, Huawei, Hisilicon

- R4-162705, Initial Cat-M PUCCH Simulation Results, Nokia

- R4-160350, Work plan for WI on Rel-13 eMTC BS performance requirements Ericsson

- R4-161215, Way forward on eMTC BS demodulation performance requirements, Huawei, HiSilicon, Ericsson, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- R4-160271, Cat-M PUCCH performance requirement, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- 3GPP TS 36.104, E-UTRA: Base Station (BS) radio transmission and reception.