3GPP TSG-RAN WG4 Meeting #79	R4-163185

Nanjing, China, 23 - 27 May 2016

Agenda item:			6.5.4

Source:	Nokia

Title:	Cat-M1 PUSCH Practical Simulation Results

Document for:		Discussion

# Introduction

The PUSCH simulation assumptions for supporting Cat-M UEs were agreed in RAN4#78 [1]. In this paper, we provided the updated ideal Cat-M PUSCH link level simulation results based on the agreed PUSCH simulation assumptions [2] as well as the practical Cat-M PUSCH link level simulation results.

# Simulation Assumptions

The simulation assumptions for Cat-M PUSCH requirements were provided in [1] (as shown in Table 1) for

Table 1	Parameter for PUSCH.

| Parameters | Values |  |
| --- | --- | --- |
|  | CE Mode A | CE Mode B |
| Maximum number of HARQ transmissions | 4 | 2 |
| RV sequences | {0, 2, 3, 1, 0, 2, 3, 1,…} | {0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1,…} |
| Uplink-downlink allocation for TDD | Configuration 1 (2:2) | Configuration 1 (2:2) |
| Propagation conditions | EPA5 | ETU1 |
| Number of Tx antennas | 1 | 1 |
| Number of Rx antennas | 2 | 2 |
| Antenna correlation | Low | Low |
| Channel bandwidth (MHz) | 3, 5, 10, 15, 20 | 3, 5, 10, 15, 20 |
| Cyclic prefix | Normal | Normal |
| Number of PUSCH repetitions | 8 | 256 |
| Frequency hopping interval | 4: FDD 5: TDD | 4: FDD 5: TDD |
| MCS | 1/3 QPSK (See Table 2) | 1/3 QPSK (See Table 2) |

Table 2	FRCs for PUSCH.

| Reference channel | 1/3 QSPK for CE Mode A | 1/3 QPSK for CE Mode B |
| --- | --- | --- |
| Allocated resource blocks | 6 | 1 |
| DFT-OFDM Symbols per subframe | 12 | 12 |
| Modulation | QPSK | QPSK |
| Code rate | 1/3 | 1/3 |
| Payload size (bits) | 600 | 104 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Coded block size including 12bits trellis termination (bits) | 1884 | 296 |
| Total number of bits per sub-frame | 1728 | 288 |
| Total symbols per sub-frame | 864 | 144 |

# Ideal Simulation Results

Table 3-1 Minimum requirements for PUSCH, 3 MHz Channel Bandwidth for CEModeA, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | EPA 5Hz Low | Ax-1 | 70% | -8.5 |

Table 3-2 Minimum requirements for PUSCH, 5 MHz Channel Bandwidth for CEModeA, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | EPA 5Hz Low | Ax-1 | 70% | -8.6 |

Table 3-3 Minimum requirements for PUSCH, 10 MHz Channel Bandwidth for CEModeA, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | EPA 5Hz Low | Ax-1 | 70% | -9.3 |

Table 3-4 Minimum requirements for PUSCH, 15 MHz Channel Bandwidth for CEModeA, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | EPA 5Hz Low | Ax-1 | 70% | -9.4 |

Table 3-5 Minimum requirements for PUSCH, 20 MHz Channel Bandwidth for CEModeA, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | EPA 5Hz Low | Ax-1 | 70% | -9.5 |

Table 3-6 Minimum requirements for PUSCH, 3 MHz Channel Bandwidth for CEModeB, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | ETU 1Hz Low | Ax-2 | 70% | -19.9 |

Table 3-7 Minimum requirements for PUSCH, 5 MHz Channel Bandwidth for CEModeB, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | ETU 1Hz Low | Ax-2 | 70% | -19.9 |

Table 3-8 Minimum requirements for PUSCH, 10 MHz Channel Bandwidth for CEModeB, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | ETU 1Hz Low | Ax-2 | 70% | -19.9 |

Table 3-9 Minimum requirements for PUSCH, 15 MHz Channel Bandwidth for CEModeB, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | ETU 1Hz Low | Ax-2 | 70% | -19.9 |

Table 3-10 Minimum requirements for PUSCH, 20 MHz Channel Bandwidth for CEModeB, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | ETU 1Hz Low | Ax-2 | 70% | -20.0 |

# Practical Simulation Results

Table 4-1 Minimum requirements for PUSCH, 3 MHz Channel Bandwidth for CEModeA, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | EPA 5Hz Low | Ax-1 | 70% | -6.5 |

Table 4-2 Minimum requirements for PUSCH, 5 MHz Channel Bandwidth for CEModeA, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | EPA 5Hz Low | Ax-1 | 70% | -6.6 |

Table 4-3 Minimum requirements for PUSCH, 10 MHz Channel Bandwidth for CEModeA, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | EPA 5Hz Low | Ax-1 | 70% | -7.3 |

Table 4-4 Minimum requirements for PUSCH, 15 MHz Channel Bandwidth for CEModeA, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | EPA 5Hz Low | Ax-1 | 70% | -7.4 |

Table 4-5 Minimum requirements for PUSCH, 20 MHz Channel Bandwidth for CEModeA, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | EPA 5Hz Low | Ax-1 | 70% | -7.5 |

Table 4-6 Minimum requirements for PUSCH, 3 MHz Channel Bandwidth for CEModeB, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | ETU 1Hz Low | Ax-2 | 70% | -17.9 |

Table 4-7 Minimum requirements for PUSCH, 5 MHz Channel Bandwidth for CEModeB, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | ETU 1Hz Low | Ax-2 | 70% | -17.9 |

Table 4-8 Minimum requirements for PUSCH, 10 MHz Channel Bandwidth for CEModeB, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | ETU 1Hz Low | Ax-2 | 70% | -17.9 |

Table 4-9 Minimum requirements for PUSCH, 15 MHz Channel Bandwidth for CEModeB, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | ETU 1Hz Low | Ax-2 | 70% | -17.9 |

Table 4-10 Minimum requirements for PUSCH, 20 MHz Channel Bandwidth for CEModeB, 1Tx

| Number of RX antennas | Cyclic prefix | Frequency Hopping | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Normal | 1 | ETU 1Hz Low | Ax-2 | 70% | -18.0 |

# Summary

In this contribution, we provided the updated ideal and also the practical level simulation assumptions for Cat-M PUSCH performance requirements. We suggest these results be considered in defining the Cat-M1 PUSCH performance requirements.

# References

- R4-161217, Simulation assumptions for PUSCH	, Ericsson, Nokia networks, Alcatel-Lucent, Huawei, HiSilicon

- R4-162706, “Cat-M1 PUSCH Initial Simulation Results”, Nokia

- R4-160350, Work plan for WI on Rel-13 eMTC BS performance requirements Ericsson

- R4-161215, Way forward on eMTC BS demodulation performance requirements, Huawei, HiSilicon, Ericsson, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

- 3GPP TS 36.104, E-UTRA: Base Station (BS) radio transmission and reception.