3GPP TSG-RAN WG4 Meeting #79	                                           R4-163494

# Nanjing, China, 23- 27 May, 2016

Agenda Item:	6.14.3.1

Source: 	Samsung

Title: 	Test case design for Class A PMI test cases

Document for:	Discussion

Introduction

In last RAN4 meeting, such agreements were achieved for Class A PMI test cases in [1]:

- Test Metric

  - Reusing existing 8Tx PMI test metric:   ,    is [TBD%] of the maximum throughput obtained  at  using the precoders configured according to the UE reports,  and   is the throughput measured at   with random precoding.

- Test applicability for CSS configuration:

  - Defining performance requirements which applicable for all the CSS configurations. For each test case, select corresponding CSS configuration based on UE capability (baseline)

    - FFS for detailed selection rule

    - Bring simulation results for both single PMI and multiple PMI test case with  different Codebook configuration 1,2,3,4 in next RAN4 meeting

- Test parameters

  - Introduce Single PMI test with 12 ports, (N1, N2) = (2, 3), (O1, O2) = (8, 4), CDM2, codebook configuration =1,2,3,4, PUSCH 3-1, EPA 5Hz, 64QAM ½ with rank1

  - Introduce Multiple PMI test with 16 ports, (N1, N2) = (2, 4), (O1, O2) = (8, 8), CDM4, codebook configuration =1,2,3,4, PUSCH 1-2, EVA 5Hz, 16QAM ½ with rank2

In this contribution, we would like to give more detailed simulation results in terms of different test parameters and provide our preference for Class A PMI test cases design.

Simulation assumption

Detailed simulation assumptions for single and multiple PMI test cases were given in table below.

| Parameter | Unit | Test 1(Single PMI test) | Test 2 (Multiple PMI test) |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 | 10 |
| Transmission mode |  | 9 | 9 |
| Propagation channel |  | EPA5 | EVA5 |
| Precoding granularity (only for reporting and following PMI) |  | 50 | 6 |
| Correlation and antenna configuration |  | 2D High XP 12 x 2 (N1,N2,P) = (2,3,2) | 2D High XP 16 x 2 (N1,N2,P) =(2,4,2) |
| Beamforming model |  | [Annex B.4.3] | [Annex B.4.3] |
| Cell-specific reference signals |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  | Antenna ports | Antenna ports |
|  |  | 15,…,26 | 15,…,30 |
| Number of CSI-RS ports |  | 12 | 16 |
| CDM Type |  | CDM2 | CDM4 |
| CSI-RS periodicity and subframe offset TCSI-RS / ICSI-RS |  | 5/1 | 5/1 |
| NZP-CSI-RS-Configuration-List |  | {0,1,2} | {0,1} |
| eMIMO-Type |  | Class A | Class A |
| codebookConfig-N1 |  | 2 | 2 |
| codebookConfig-N2 |  | 3 | 4 |
| codebook-Over-Sampling-RateConfig-O1 |  | 8 | 8 |
| codebook-Over-Sampling-RateConfig-O2 |  | 4 | 8 |
| Codebook-Config |  | 1,2,3,4 | 1,2,3,4 |
| codebookSubsetRestriction-1 |  | 0x01 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF | 0x02 FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
| codebookSubsetRestriction-2 |  | (Codebook-Config 1): 0000 0000 1111 (Codebook-Config 2,3,4): 0x 00 000000 0000 FFFF | (Codebook-Config 1): 0000 1111 0000 (Codebook-Config 1): 0x 00 000000 FFFF 0000 |
| Reporting mode |  | PUSCH 3-1 | PUSCH 1-2 |
| Reporting interval | ms | 5 | 5 |
| PMI delay | ms | 8 | 8 |
| Measurement channel |  | 64QAM 1/2 (R.XX-1) | 16QAM 1/2 (R.XX-2) |
| Rank Number of PDSCH |  | 1 | 2 |
| Max number of HARQ transmissions |  | 4 | 4 |
| Redundancy version coding sequence |  | {0,1,2,3} | {0,1,2,3} |

Performance evaluation

Absolute throughput performance with following PMI and relative throughput ratio under different codebook configurations (1, 2, 3 and 4) for single PMI test are given in table 1 and table 2 below. For multiple PMI test, figure 3 and figure 4 show absolute throughput performance and relative throughput ratio between following PMI and random PMI.

Reference test point

As agreed in last meeting, existing test metric for 8Tx XP PMI test cases was reused for Class A PMI test cases. The reference test point as the number of percentile of the maximum throughput with following PMI is still FFS. The required SNR points with following PMI and corresponding throughput ratios for different percentiles of maximum throughput (70%,75%,80% and 85%) were summarized in table 1 and table 2 below for single PMI test and multiple PMI test separately.

We can observe that:

- For single PMI test, at SNR around 2.5dB, absolute throughputs achieve 80% of maximum throughout and corresponding throughput ratios are around 6.0.

- For multiple PMI test, at SNR around 3.6dB, absolute throughputs achieve 80% of maximum throughout and corresponding throughput ratios are around 6.2~7.0.

Considering proper test SNR points and feasible test requirements, we propose:

Proposal1: Taking [80%] of the maximum throughput as reference test point.

Test applicability for Codebook configuration

Following RAN1/RAN2 agreements, UE can indicate its codebook-Config capability with having separated field for each configuration.

MIMO-NonPrecodedCapabilities-r13 ::=	SEQUENCE {

config1-r13								ENUMERATED {supported}			OPTIONAL,

config2-r13								ENUMERATED {supported}			OPTIONAL,

config3-r13								ENUMERATED {supported}			OPTIONAL,

config4-r13								ENUMERATED {supported}			OPTIONAL

}

As agreed in last RAN4 meeting, we will define performance requirements which applicable for all the CSS configurations. For each test case, select corresponding CSS configuration based on UE capability and detailed selection rule is FFS.

Based on simulation results, we can observe that there are no big differences for both absolute throughput performance and relative throughput ratio between different codebook configurations (1, 2, 3 and 4).

Considering this, we proposed such selection rule:

Proposal2: For each test case, random select one of codebook configuration value from UE supported codebook configurations.

Figure 1 Absolute throughput with following PMI vs. SNR for single PMI test

Figure 2 Throughput ratio with following PMI vs. SNR for single PMI test

Figure 3  Absolute throughput with following PMI vs. SNR for multiple PMI test

Figure 4  Throughput ratio with following PMI vs. SNR for multiple PMI test

Table 1 Reference SNR point /TP ratio for single PMI test cases

| Reference point (Percentile of maximum throughput ) | Required SNR(dB) (following PMI ) |  |  |  | TP ratio (Following PMI/Random) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Codebook configuration | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 |
| 70% | 1.6 | 1.7 | 1.7 | 1.6 | 7.3 | 7.1 | 7.0 | 7.1 |
| 75% | 2.0 | 2.1 | 2.1 | 2.0 | 6.6 | 6.4 | 6.6 | 6.5 |
| 80% | 2.5 | 2.6 | 2.6 | 2.6 | 6.2 | 6.0 | 6.1 | 6.1 |
| 85% | 3.1 | 3.2 | 3.2 | 3.1 | 5.7 | 5.7 | 5.6 | 5.7 |

Table 2 Reference SNR point /TP ratio for Multiple PMI test cases

| Reference point (Percentile of maximum throughput ) | Required SNR(dB) (following PMI ) |  |  |  | TP ratio (Following PMI/Random) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Codebook configuration | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 |
| 70% | 2.1 | 2.3 | 2.2 | 2.2 | 8.9 | 8.0 | 8.5 | 8.5 |
| 75% | 2.8 | 3.0 | 3.0 | 2.9 | 7.8 | 6.9 | 7.2 | 7.5 |
| 80% | 3.5 | 3.8 | 3.7 | 3.6 | 7.0 | 6.2 | 6.6 | 6.8 |
| 85% | 4.2 | 4.6 | 4.4 | 4.4 | 6.3 | 5.7 | 6.0 | 6.0 |

Conclusion

In this contribution, simulation results were given for CSI Class A PMI test cases. Based on the simulation results, we propose:

Proposal1: Taking [80%] of the maximum throughput as reference test point.

Proposal2:  For each test case, random select one of codebook configuration value from UE supported codebook configurations.

Reference

[1] R4-163030,” Way forward for performance requirements under FD-MIMO”, Samsung, Qualcomm, MediaTek, Ericsson, Intel, LGE