3GPP TSG-RAN WG4 Meeting #79	R4-163632

# Nanjing, China, 23 – 27 May, 2016

Agenda item:	6.6.3

Source: 	Qualcomm Incorporated

Title: 	Simulation results for reference receivers for Control Channel Interference Mitigation

Document for:	Discussion

# Introduction

In work item RP-151107 [1], it was proposed to use advanced receiver architectures for control channel interference mitigation. In RAN4 #78bis, RAN4 trimmed down the test case list and arrived at a consensus on interference models for sync network scenarios. In this contribution, we provide simulation results based on the agreed test case parameters agreed in [2].

# Simulation results

We provide simulation results with the following reference receivers

- MMSE-MRC (baseline receiver for Rel12)

- MMSE-IRC

- MMSE-IRC + CRS-IC

Table 1 provides common simulation parameters used for simulations.

Table 1: Common simulation parameters

| Parameters | Value |
| --- | --- |
| System bandwidth | 10 MHz for the serving and interfering cells |
| Duplexing mode | FDD |
| Cyclic prefix | Normal |
| Number of interference cells | 2 interfering cells |
| Interference power profile | High INR: I1/Noc = 13.91 dB, I2/Noc = 3.34 dB |
| Time-frequency offset modelling for synchronous network | Time offset: Interference cell #1 – 2us, Interference cell #2 – 3us Frequency offset: Interference cell #1 – 200Hz, Interference cell #2 – 300Hz |
| Cell ID | Serving cell: 0 Colliding CRS: Interferer cell #1 - 6, Interferer cell #2 - 1 Non-Colliding CRS: Interferer cell #1 - 1, Interferer cell #2 – 6 |
| CRS ports | Port 0 and 1 |
| Antenna configuration | 2x2, Low correlation |
| Tx EVM | 6% |
| Unused Serving cell RE-s and PRB-s | OCNG |

## PDCCH/PCFICH simulation results

Table 2 provides simulation parameters used for PDCCH/PCFICH agreed in [2]. Table 3 provides a list of test cases (from which down-selection of final test cases need to be done) for which results for PDCCH/PCFICH performance are provided.

Table 2. Simulation parameters for PDCCH/PCFICH

| Parameters | Value |
| --- | --- |
| Serving cell PDCCH | DCI Format 2 (43 bits – FDD, 46 bits – TDD) |
| CFI | Case 1: CFIS = 1, CFII = 1 Case 2: CFIS = 3, CFII = 3 |
| PHICH Ng | Ng = 1/6 (i.e. number of PHICH groups = 2 for 10 MHz BW) |
| PHICH duration | Normal |
| Interference model | PDCCH/PHICH interference signals are emulated using random QPSK-modulated symbols with the SFBC-based precoding: - Per-CCE signal transmission granularity - N = 50% interference loading for PDCCH/PHICH - Guaranteed interference loading on a subframe basis (N% of all available CCEs are chosen to be active in each subframe) - In each subframe, randomly pick one CCE from the set of empty CCEs. Repeat the procedure till the loading is equal N %. - Interference PDCCH/PHICH power boosting has uniform distribution in log domain in the range from -6dB to 6dB - Interference PDCCH/PHICH power normalization is used - Apply single power normalization factor [-1.3] dB. |
| CRS assistance information | Provided |

Table 3. PDCCH/PCFICH test cases

| Test number | Network Type | PDCCH AL | Propagation Condition | Antenna configuration | CRS pattern | CFI |
| --- | --- | --- | --- | --- | --- | --- |
| PDCCH #1a | Synchronous | 2 CCE | EPA5 for all cells | 2 x 2 Low | Colliding | CFIS = 1, CFII = 1 |
| PDCCH #1b | Synchronous | 2 CCE | EPA5 for all cells | 2 x 2 Low | Colliding | CFIS = 3, CFII = 3 |
| PDCCH #2a | Synchronous | 4 CCE | EPA5 for all cells | 2 x 2 Low | Non-colliding | CFIS = 1, CFII = 1 |
| PDCCH #2b | Synchronous | 4 CCE | EPA5 for all cells | 2 x 2 Low | Non-colliding | CFIS = 3, CFII = 3 |

Results for PDCCH/PCFICH are provided below.

PDCCH #1a: Serving cell PDCCH AL 2 Interference PDCCH Model 50% loading + Non-Uniform power, High INR CFIS = 1, CSII = 1 Colliding CRS.

PDCCH #1b: Serving cell PDCCH AL 2 Interference PDCCH Model 50% loading + Non-Uniform power, High INR CFIS = 3, CSII = 3 Colliding CRS

PDCCH #2a: Serving cell PDCCH AL 4 Interference PDCCH Model 50% loading + Non-Uniform power, High INR CFIS = 1, CSII = 1 Non-colliding CRS.

PDCCH #2b: Serving cell PDCCH AL 4 Interference PDCCH Model 50% loading + Non-Uniform power, High INR CFIS = 3, CSII = 3 Non-colliding CRS

## EPDCCH simulation results

For EPDCCH, simulation parameters are defined in Table 4 and scenarios for which simulation results are provided are given in Table 4.

Table 4. EPDCCH simulation parameters

| Parameters | Value |
| --- | --- |
| Serving cell EPDCCH | FRC #1: AL 2, Localized EPDCCH FRC #2: AL 4, Distributed EPDCCH DCI Format 2C (44 bits – FDD, 47 bits – TDD) |
| Number of PDCCH symbols and EPDCCH start | Distributed EPDCCH: CFI=2, EPDCCH starting symbols is derived from CFI Localized EPDCCH: The EPDCCH starting symbol is 2. CFI = 1. EPDCCH starting symbol is RRC configured. Aligned control regions and EPDCCH starting symbols in the serving and interference cells |
| EPDCCH parameters | Number of EPDCCH Sets Configured = 1 Distributed EPDCCH set PRBs {3, 17, 31, 45} Localized EPDCCH set PRBs {0, 7, 14, 21, 28, 35, 42, 49} EPDCCH is transmitted in all subframes EPDCCH precoding model is in accordance to TS 36.101 B.4.4. and B.4.5 |
| Interference model | Model #1: No PDSCH interference Model #2: Rel-11 Type A receiver asynchronous interference model (TS 36.101 B.5.2, 8.2.1.2.4) Model #3: Full PDSCH interference model. Reuse TM9 interference model from Rel-11 Type A receiver requirements (TS 36.101 8.3.1.1A, B.5.4). |
| Performance metrics | Pm-dsg vs SINR |

Table 5. EPDCCH test cases

| Test number | FRC | Interference model | Propagation Condition | Antenna configuration | CRS pattern | NW type |
| --- | --- | --- | --- | --- | --- | --- |
| EPDCCH 1 FDD | FRC #1 | Model #1 | EPA5 for all cells | 2 x 2 Low | Non-colliding | Sync |
| EPDCCH #2 FDD | FRC #2 | Model #2 | EVA70 for all cells | 2 x 2 Low | NA | Async |

Results for EPDCCH are provided below:

EPDCCH #1 FDD: EPDCCH AL 2, Localized, No PDSCH interference, High INR, Asynchronous NW, Non-Colliding CRS pattern..

EPDCCH #2 FDD: EPDCCH AL 4, Distributed, Rel11 Type A receiver async network, High INR, Async NW..

<PHICH simulation results will be added later>

# References

- RP-151107, “New WI proposal: Interference mitigation for downlink control channels of LTE”, Intel Corporation, ZTE, RAN #68, June 2015

- R4-162759, “WF on DL Control Channel IM UE demodulation test cases”, ZTE, Intel Corporation, RAN#78bis, April 2016