3GPP TSG RAN4 Meeting #79	R4-163636

Nanjing, China, 23 – 27 May, 2016

Title: 	MSD definition for 2UL Inter-band CA combinations that have multiple IMD’s for one band

Source: 	Huawei, HiSilicon

Agenda item:	7.6.1

Document for:	Other [approval]

# 1	Introduction

This contribution proposes how to handle the case when there are more than one intermodulations landing on top of own DL channel for one band in 2UL Inter-band CA.

# 2	Discussion

According to [1] the following has been agreed for MSD definition in 2UL Inter-band CA.

- Choose a specific carrier frequency allocation such that IMD is center-aligned with victim DL carrier to attest the worst-case self-desensitization condition.

- Choose 5-MHz bandwidth with full RB for all carriers (if supported in a CA combination) to essentially comply with the Rel-8 UL configurations for REFSENS tests and approach the worst-case self-desensitization condition.

- If 5-MHz bandwidth is not supported in certain CA combinations, then choose 10 MHz with full RB for one or both carriers.

- For B2_B4, IMD3 can overlap with DL carrier only when B2 carrier bandwidth is at 20 MHz [2].

- A correction factor (CF) is calculated to address the power difference between the total IMD power and the power within DL carrier modulation bandwidth, as illustrated in Figure 2-1.

Figure 2-1 Definition of correction factor (CF)

According to condition 1, only the worst case MSD is specified for each band. In practice it means that if certain band in 2UL Inter-band CA configuration has more than one IMD hitting on top of own DL channel, only the highest MSD in dB’s will be specified. This has been the practice so far. Please note that for instance in 2UL/2DL Inter-band CA, there are in some cases MSD defined for both bands; this has been totally according to the agreement above. Below is an example excerpt from 36.101 2UL/2DL MSD table.

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| CA_2A-4A | 2 | 1860 | 20 | 502 | 1940 | 5 | FDD |
|  | 4 | 1752.5 | 5 | 25 | 2152.5 | N/A |  |
| CA_2A-4A | 2 | 1868.3 | 5 | 25 | 1948.3 | N/A | FDD |
|  | 4 | 1735 | 5 | 25 | 2135 | 5 |  |
| CA_3A-5A | 3 | 1771 | 10 | 50 | 1866 | 4 | FDD |
|  | 5 | 838 | 5 | 25 | 883 | N/A |  |
| CA_3A-5A | 3 | 1721 | 10 | 50 | 1816 | N/A | FDD |
|  | 5 | 838 | 5 | 25 | 883 | 24 |  |
| CA_3A-19A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD |
|  | 19 | 838 | 5 | 25 | 883 | N/A |  |
| CA_3A-19A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD |
|  | 19 | 838 | 5 | 25 | 883 | 27 |  |
| CA-3A-20A | 3 | 1775 | 5 | 25 | 1870 | 4 | FDD |
|  | 20 | 840 | 5 | 25 | 799 | N/A |  |
| CA-3A-20A | 3 | 1735 | 5 | 25 | 1830 | N/A | FDD |
|  | 20 | 847 | 5 | 25 | 806 | 9 |  |
| CA_3A-26A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD |
|  | 26 | 838 | 5 | 25 | 883 | N/A |  |
| CA_3A-26A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD |
|  | 26 | 838 | 5 | 25 | 883 | 26 |  |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 |  |  |  |  |  |  |  |

Table 1 Example of 2UL/2DL Inter-band CA MSD

Recently there has been some discussion about the case when there are more than one IMD hitting on top of own DL channel on one band in 2UL/2DL Inter-band CA configuration. A good example of this case is 2UL Inter-band CA B3+B42, where B3 is subject to three different IMD’s: IMD2, IMD4, and IMD5. The agreed table below is from [2].

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| CA_3A-42A (for IM2) | 3 | 1740 | 5 | 25 | 1835 | 29.8 | FDD |
|  | 42 | 3575 | 5 | 25 | 3575 | N/A | TDD |
| CA_3A-42A (for IM4) | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD |
|  | 42 | 3435 | 5 | 25 | 3435 | N/A | TDD |
| CA_3A-42A (for IM5) | 3 | 1740 | 5 | 25 | 1835 | 0.2 | FDD |
|  | 42 | 3575 | 5 | 25 | 3575 | N/A | TDD |

Table 2 MSD summary for dual uplink B3+B42

WF with the proposal below to further discuss this issue was agreed in previous meeting [3].

Proposal: For CA_3-42, interested companies are encouraged to investigate if MSD requirement for IM2 can also verify that of IM4. Once identified, the studies will be captured into TR 36.714-02-02. If not identified, other approaches to verify MSD for IM4 needs to be determined in RAN4#79.

The idea behind the sentence “For CA_3-42, interested companies are encouraged to investigate if MSD requirement for IM2 can also verify that of IM4.” is that if the same IMD mechanism (switch, PA forward mixing etc.) dominates both IMD4 and IMD2, maybe the smaller IMD (in this case IMD4) could be indirectly verified by testing only IMD2. By that way RAN4 could specify only the largest IMD (in this case IMD2) according to agreement in [1].

In [4], [5], and [6] the dominating factor for both IMD2 and IMD4 main branch was B3 forward mixing. For the diversity branch B3 forward mixing is amongst the dominating factors but not clearly the most dominant one for IMD2. For IMD4 in diversity path B3 forward mixing is the most dominant factor. Thus, in our opinion, the situation is not clear enough to claim that by verifying IMD2, the IMD4 requirement could be guaranteed. As a consequence, we believe that other approaches to verify MSD for IM4 needs to be considered. The following part of this contribution is generic, and not specific only to tackle issues with B3+B42.

We well understand the issue from operator perspective; naturally each operator is interested in verifying the IMD or even IMD’s that exist with their spectrum holding. However, it this case there might be operators whose spectrum is subject only to IMD2, only to IMD4, only to IMD5 or even all IMD2, IMD4, and IMD5. Thus, in principle, all IMD’s should be specified to have fully “water-proof” solution.

We foresee four options to go forward:

1) Remain status quo (define only the highest MSD)

2) Specify all MSD’s

3) Specify highest MSD, and lower ones based on operator request

4) Specify highest MSD, and lower ones if MSD exceeds certain threshold value

Each of these options have pro’s and con’s. On vendors side the issue with defining more than one MSD is seen burdensome from standardization and testing perspective. We believe that existing specification should not be modified, i.e. the changes in the process, if any, should apply only for new combinations.

To give broader background for this issue we did a survey amongst the existing 2UL Inter-band CA combinations to check how many bands in current combinations currently have more than one IMD.

Table 7.3.1A-0f: 2 UL and 2 DL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Other MSD exists? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_1A_3A | 1 | 1950 | 5 | 25 | 2140 | 23 | FDD | NO |
|  | 3 | 1760 | 5 | 25 | 1855 | N/A |  | N/A |
| CA_1A_8A | 1 | 1965 | 5 | 25 | 2155 | 6 | FDD | NO |
|  | 8 | 887.5 | 5 | 25 | 932.5 | N/A |  | N/A |
| CA_2A-4A | 2 | 1860 | 20 | 502 | 1940 | 5 | FDD | NO |
|  | 4 | 1752.5 | 5 | 25 | 2152.5 | N/A |  | N/A |
| CA_2A-4A | 2 | 1868.3 | 5 | 25 | 1948.3 | N/A | FDD | N/A |
|  | 4 | 1735 | 5 | 25 | 2135 | 5 |  | NO |
| CA_3A-5A | 3 | 1771 | 10 | 50 | 1866 | 4 | FDD | NO |
|  | 5 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-5A | 3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 24 |  | YES, IMD5 |
| CA_3A-7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | 7 | 2535 | 10 | 50 | 2655 | 13 |  | NO |
| CA_3A-8A | 3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A [IMD5] |
|  | 8 | 900 | 5 | 25 | 945 | 8 |  | YES, IMD5 |
| CA_3A-19A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD | NO |
|  | 19 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-19A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | 19 | 838 | 5 | 25 | 883 | 27 |  | YES, IMD5 |
| CA-3A-20A | 3 | 1775 | 5 | 25 | 1870 | 4 | FDD | NO |
|  | 20 | 840 | 5 | 25 | 799 | N/A |  | N/A |
| CA-3A-20A | 3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | 20 | 847 | 5 | 25 | 806 | 9 |  | NO |
| CA_3A-26A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD | NO |
|  | 26 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-26A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | 26 | 838 | 5 | 25 | 883 | 26 |  | YES, IMD5 |
| CA_4A-5A | 4 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 26 |  | YES, IMD5 |
| CA_4A-7A | 4 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | 7 | 2535 | 5 | 25 | 2655 | 15 |  | NO |
| CA_5A-7A | 5 | 834 | 5 | 25 | 879 | 12 | FDD | YES, IMD5 |
|  | 7 | 2547 | 10 | 50 | 2667 | N/A |  | N/A |
| CA_7A-20A | 7 | 2512 | 10 | 50 | 2632 | N/A | FDD | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 12 |  | YES, IMD5 |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 |  |  |  |  |  |  |  |  |

Table 7.3.1A-0g: 2 UL and 3 DL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Other MSD exists? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc (MHz) | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| CA_1A-5A-7A | CA_1A-7A | 1 | 1968 | 5 | 25 | 2158 | 5 | NA | FDD | NA |
|  |  | 7 | 2512 | 10 | 50 | 2632 | 10 | NA |  | NA |
|  |  | 5 | 835 | 5 | 25 | 880 | 5 | 1.0 |  | NO |
| CA_3A-7A-20A | CA_3A-7A | 3 | 1737 | 5 | 25 | 1832 | 5 | NA | FDD | NA |
|  |  | 7 | 2543 | 10 | 50 | 2663 | 10 | NA |  | NA |
|  |  | 20 | 847 | 10 | 20 | 806 | 10 | 10.5 |  | NO |
|  | CA_3A-20A | 3 | 1775 | 10 | 50 | 1870 | 10 | NA | FDD | NA |
|  |  | 20 | 855 | 5 | 25 | 896 | 5 | NA |  | NA |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | 26.0 |  | YES, IMD3 |
| CA_3A-7A-28A | CA_3A-7A | 3 | 1747 | 5 | 25 | 1842 | 5 | NA | FDD | NA |
|  |  | 7 | 2543 | 5 | 25 | 2663 | 5 | NA |  | NA |
|  |  | 28 | 741 | 5 | 25 | 796.0 | 5 | 20 |  | NO |
|  | CA_7A-28A | 7 | 2543 | 5 | 25 | 2663 | 5 | NA | FDD | NA |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | NA |  | NA |
|  |  | 3 | 1737.5 | 5 | 25 | 1832.5 | 5 | 26 |  | NO |

It can be seen from the tables above that currently there are 8 UL Inter-band CA combinations out of 16 that have more than one intermodulation on top of own DL channel. However, 7 of these 8 cases relate to IMD5 which is often very small or even zero in some cases.

To address this issue in general, we propose to add a note to table 7.3.1A-0f and table 7.3.1A-0g that contains information on if there is another IMD that falls on top of own DL CC than the one that is specified. The purpose of this note is to give additional information for UL Inter-band CA use scenarios etc. As the IMD under this note is only informative, it should not be inside the table.

What to do with for instance B3+B42 is a tough call. If the current process were followed, only IMD2 (29.8dB) would be specified. However, in that case IMD4 that occurs at least with one operator’s spectrum owning would be left unspecified even the MSD is fairly significant (8dB). We propose to choose option 4 to go forward, i.e to define a threshold for additional IMD(s) to be defined. In choosing the proposed threshold value we sought balance between the workload, specification complexity, and the value that we believe is fair accounting that in any case for each band there is at least one higher MSD specified. Based on these considerations we propose 3dB threshold value.

Proposal1: For each UL Inter-band CA only the highest IMD is specified automatically. Each IMD that falls on top of own DL channel and has ≥3dB MSD will be specified as well. This applies from Rel14 onwards.

Proposal2: An informative note to UL Inter-band CA MSD tables shall be added in Rel14 to indicate if there are other IMD’s than the specified one(s) for each band.

# 3	Conclusion

Case when more than one intermodulation falls on top of own DL channel was discussed. While we find this issue pretty challenging to solve in manner that would be suitable for all stakeholders, we propose the following to tackle this issue.

Proposal1: For each UL Inter-band CA only the highest IMD is specified automatically. Each IMD that falls on top of own DL channel and has ≥3dB MSD will be specified as well. This applies from Rel14 onwards.

Proposal2: An informative note to UL Inter-band CA MSD tables shall be added in Rel14 to indicate if there are other IMD’s than the specified one(s) for each band.

# 4	References

[1] R4-144031, “WF on 2UL inter-band CA MSD test configurations”, Mediatek, RAN4#71

[2] R4-161856, “How to specify MSD requirements for CA_3A-42A (2DL2UL)”, NTT DOCOMO Inc.

[3] R4-163087, “How to specify MSD requirements for CA_3A-42A(2DL/2UL)”, NTT DOCOMO Inc.

[4]	R4-160119, “MSD for 3+42”, Qualcomm Incorporated

[5]	R4-160396, “MSD analysis for 2UL CA_3A_42A”, MediaTek Inc.

[6]	R4-161398, “MSD for 3+42 2UL CA”, Huawei, HiSilicon