nrameter in  SNR due to Baseband
nrameter in  SNR due to Baseband3GPP TSG-RAN WG4 Meeting #79                           R4-163806

# Nanjing, China, 23rd -27th May, 2016

Source:	CMCC

Title:			  Discussion on Spectrum for New Radio

Agenda Item:	9.4

Document for:	Approval

- Introduction

In RAN4#78bis, selecting candidate bands for NR was discussed. A WF [1] was approved for NR in 24.25-86GHz in the following.

- Study RF parameters based on the following sub-frequency ranges.

  - 24.25-33.4, 37-43.5, 45.5-52.6, 66-86

- It is for further study which specific frequencies to select for time consuming studies, such as co-existence simulations. The number of specific frequencies chosen may be less than the number of frequency ranges.

This contribution further discusses this issue.

- Discussion

Base on the SID [2] of New Radio Access Technology, RAN4 should focus on RF parameters and co-existence study. As we all know, these two aspects highly depend on frequency bands. However, in our opinion, it is not in a hurry to select candidate bands for NR.

For NR in 24.25 – 86 GHz, four sub-frequency ranges have been identified in the last meeting. It is suppose each sub-frequency range has similar RF performance. RF parameters and co-existence can be conducted for each sub-frequency range instead of particular frequency band.

Observation#1: RF parameters and co-existence study can be conducted based on each sub-frequency range.

Similar method should also be applied to frequency below 6GHz. Low frequency range is supposed to support large coverage scenarios for NR, and it is important for 5G stand-alone deployment. Although RAN4 haven’t identify how to group lower frequency for sub-frequency ranges, several ranges was already proposed in NR deployment scenario in TR 38.913 [3]. “Around 4GHz” is mentioned in several scenarios, and it could cover 3.3 – 4.99 GHz in which several frequency range was identified for IMT in WRC-15 such as 3.3 – 3.4 GHz, 3.4- 3.6 GHz, 4.8 - 4.99GHz and etc. “Around 700MHz” is also mentioned in several scenarios, and it could cover IMT frequency under 1GHz, e.g. 450 – 960MHz.

Proposal#1: NR frequency below 6GHz can be grouped into two sub-frequency ranges of 3.3 – 4.99 GHz and 450 – 960 MHz.

- Conclusion

Observation#1: RF parameters and co-existence study can be conducted based on each sub-frequency range.

Proposal#1: NR frequency below 6GHz can be grouped into two sub-frequency ranges of 3.3 – 4.99 GHz and 450 – 960 MHz.

- Reference

[1] R4-162879, WF on RF parameters requested by WP 5D, NTT DOCOMO, INC., Qualcomm, CMCC, Nokia, Huawei, Ericsson, KT, Intel, Samsung

[2] RP-160671, New SID Proposal: Study on New Radio Access Technology, NTT DOCOMO

[3] TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies