nrameter in  SNR due to Baseband
nrameter in  SNR due to Baseband3GPP TSG-RAN WG4 Meeting #79                           R4-163807

# Nanjing, China, 23rd -27th May, 2016

Source:	CMCC

Title:			  Discussion on RF requirement for New Radio in 24.25 - 86 GHz

Agenda Item:	9.4

Document for:	Discussion

- Introduction

In RAN4#78bis a WF [1] was approved for NR in 24.25-86GHz in the following.

- Study RF parameters based on the following sub-frequency ranges.

  - 24.25-33.4, 37-43.5, 45.5-52.6, 66-86

- It is for further study which specific frequencies to select for time consuming studies, such as co-existence simulations. The number of specific frequencies chosen may be less than the number of frequency ranges.

This contribution firstly shows some survey on RF performance for millimeter wave frequency. Secondly, Impact on RF parameters based on the former survey is discussed. Finally, some considerations on promoting the study of NR RF parameters in 24.25-86 GHz are addressed.

- Preliminary RF performance review in 24.25 - 86GHz

We have investigated the industry's mainstream base stations, terminals, instrumentation and front-end device vendor’s production capacity in 24.25 - 86GHz. At present, most of the components in this frequency are used for the 802.11ad or microwave system. Although it is not designed for IMT, to a certain extent it reflects the ability of the current industry.

2.1 Power Amplifier

Table 2.1-1 Parameters for PA in different frequency bands

| Frequency | P1dB (dBm) | OIP3 (dB) | Efficiency |
| --- | --- | --- | --- |
| 2GHz | 56 | 10-15dB larger than P1dB | 5%-6% |
| 24.25-33.4 GHz | 24 – 36 | 10-15dB larger than P1dB | 5%-6% |
| 37-43.5 GHz, | 18 – 33 | 10-15dB larger than P1dB | 2%-3% |
| 45.5‑52.6 GHz | 15 -24 | 10-15dB larger than P1dB | 2%-3% |
| 66-86 GHz | 15 - 24 | 10-15dB larger than P1dB | 2%-3% |

Current millimetre wave devices usually use GaAs-based or GaN-based PA whose cost is relatively high. As we can see in Table 2.1-1, P1dB decreases dramatically as frequency increases, and the power efficiency is extremely worse than that for lower frequency bands. High output power high efficient PA is still unavailable in this frequency range.

2.2 Performance of Low Noise Amplifier

Table 2.2-1 Parameters for LNA in different frequency bands

| Frequency | Noise figure (dB) |
| --- | --- |
| 2GHz | 0.9 - 1.5 |
| 24.25-33.4 GHz | 2.2 - 2.8 |
| 37-43.5 GHz, | 3 - 4 |
| 45.5‑52.6 GHz | 4 - 5 |
| 66-86 GHz | 4 - 5 |

Current millimetre wave LNA is usually GaAs-based or SiGe-based. SiGe-based LNA has a lower cost and a higher degree of integration. As we can see in Table 2.2-1, the noise figure increases a lot as the frequency band increases.

2.3 Filter

Table 2.3-1 Parameters for filter in different frequency bands

| Frequency | Insertion Loss (dB) |
| --- | --- |
| 2GHz | 0.8 – 1.5 |
| 24.25-33.4 GHz | 0.8 - 1.5 |
| 37-43.5 GHz, | 1 - 1.5 |
| 45.5‑52.6 GHz | 1 – 1.5 |
| 66-86 GHz | 1 - 1.5 |

Cavity filter and dielectric filter can be used for frequency under about 40 GHz, while waveguide filter shall be used for higher frequency band. As we can see in Table 2.3-1, the insertion loss based on current product is comparable to that for lower bands. That’s mainly because no stringent rejection requirement is needed till now. Attenuation could be in the order of 40-50dB and transient bandwidth is much be larger than that for lower band. If the co-existence requirement is relatively loose after future co-existence study, low cost microstrip filter may also be used for millimetre wave devices. What’s more, at present some of the millimetre wave transmitters do not even use RF filter at all.

2.4 Switch

Table 2.4-1 Parameters for switch in different frequency bands

| Frequency | Insertion Loss (dB) |
| --- | --- |
| 2GHz | around 0.5 |
| 24.25-33.4 GHz | around 2 |
| 37-43.5 GHz, | around 2 |
| 45.5‑52.6 GHz | around 3 |
| 66-86 GHz | around 3 |

In order to achieve potential shorter TTI for NR, switching time is supposed to be shorter than 100ns, while current industry capability cannot satisfy. Current millimetre wave switch’s IL is sensitive to frequency, and is about 2dB or even larger when frequency above 24GHz based on our survey. That may be guidance for NR RF design – avoid cascading multiple switches in millimetre wave RF chain,.

2.5 Circulator

Table 2.5-1 Parameters for circulator in different frequency bands

| Frequency | Insertion Loss (dB) |
| --- | --- |
| 2GHz | 0.2 – 0.3 |
| 24.25-33.4 GHz | 0.25 - 0.4 |
| 37-43.5 GHz, | 0.5 |
| 45.5‑52.6 GHz | 0.6 |
| 66-86 GHz | 0.7 |

Circulator is used in E-TURA BS. The insertion loss also increases the as frequency increases. However, the absolute value is relatively small compared to that other component contributes. As PA output power at millimetre wave is low, circulator may be replaced by switch in RF design.

2.6 Phase shifter

Table 2.6-1 Parameters for phase shifter in different frequency bands

| Frequency | Insertion Loss (dB) | Power tolerance (dBm) |
| --- | --- | --- |
| 2GHz | 0.2 – 0.3 | > 55 |
| 24.25-33.4 GHz | 5 – 10 | >20 |
| 37-43.5 GHz, | 5 – 10 | >20 |
| 45.5‑52.6 GHz | Integrated in VGA chip |  |
| 66-86 GHz | Integrated in VGA chip |  |

The industry of millimetre wave phase shifter is relatively mature, but the current product has the problems of high insertion loss and long switching time. In the future, it is necessary to promote the research and development of higher power tolerance, lower insertion loss, shorter switching time and higher accuracy phase shifter, and promote its integration with VGA chip.

2.7 Mixer

Table 2.7-1 Parameters for mixer (passive) in different frequency bands

| Frequency | Insertion Loss (dB) | Conversion Loss (dB) |
| --- | --- | --- |
| 2GHz | 6-10 | 6-10 |
| 24.25-33.4 GHz | 10 | 10 |
| 37-43.5 GHz, | 10 | 10 |
| 45.5‑52.6 GHz | 12 | 12 |
| 66-86 GHz | 12 | 12 |

The industry of millimetre wave phase shifter is relatively mature. For future NR device, mixer is supposed to be integrated in transceiver.

2.8 Antenna

Table 2.8-1 Parameters for mixer in different frequency bands

| Frequency | Single Antenna Element Gain (dBi) | 3dB Beamwidth (deg) |
| --- | --- | --- |
| 2GHz | 7-8 | 65 – 90 |
| 24.25-33.4 GHz | 5-8 | 80 - 100 |
| 37-43.5 GHz, | 4-8 | 80 – 100 |
| 45.5‑52.6 GHz | 4-8 | 80 – 100 |
| 66-86 GHz | 4-8 | 80 - 100 |

The industry of high frequency antenna is relatively mature. Due to the small size of the antenna, it is required of higher degree of manufacture arts and higher degree of integration (AiP, Antenna in Package).

2.9 Transceiver

Larger phase noise of PLL is the major problem of RF circuit in millimeter wave transceiver. And it is a challenge for ADC/DAC with higher sampling rate, high resolution and low power consumption. To achieve massive active antenna array, transceiver need higher degree of integration, 8 -16 paths may needed to be integrated in one chip in the future.

- Impact on RF parameter in 24.25 – 86 GHz

Considering the component performance above, a preliminary analysis is done for the impact on 24.25 – 86 GHz frequency band RF parameters.

3.1 Impact on maximum output power

Table 3.1-1 Impact on maximum output power

|  | PA P1dB (dBm) | RF filter IL (dB) | RF switch IL (dB) | Circulator IL (dB) |
| --- | --- | --- | --- | --- |
| 2GHz | 56 | 0.8 – 1.5 | 0.4-0.5dB | 0.2 – 0.3 |
| 24.25-33.4 GHz | 24 - 36 | 0.8 - 1.5 | 2 | 0.25 - 0.4 |
| 37-43.5 GHz, | 18 - 33 | 1 - 1.5 | 2 | 0.5 |
| 45.5‑52.6 GHz | 15 - 24 | 1 - 1.5 | 3 | 0.6 |
| 66-86 GHz | 15 - 24 | 1 - 1.5 | 3 | 0.7 |

The maximum output power depends on P1dB of PA, RF filter IL, RF switch IL and/or circulator IL (depends on RF reference architecture). As shown in Table 3.1-1, the output power of PA is the bottleneck, and the total degradation of maximum output power is about 20 – 45 dB compared to the lower band. The degradation needs to be compensated by antenna array gain.

Observation#1: the maximum output power for a single transmitter in 24.25 – 86 GHz is much lower than traditional one.

3.2 Impact on ACLR and spectrum mask

ACLR and spectrum mask highly depends on the linearity of PA. As we can see in able 2.2-1, for millimetre wave band, the OIP3 is still 10 – 15 dB larger than P1dB, i.e. the linearity of PA is similar among different frequency. However, since channel bandwidth for NR in millimetre wave frequency will be larger, the frequency separation between spurious domain and channel edge shall be extended. Moreover, the bandwidth of adjacent channel shall also be enlarged correspondingly.

Observation#2: Range of spurious domain and bandwidth of adjacent channel need to be revisited for 24.25 – 86 GHz.

DPD technology is widely used in E-UTRA transmitter. For millimetre wave, the NR devices may not be able to handle the DPD complexity caused by lager channel bandwidth. If not, the ACLR and spectrum mask may have a certain degradation compared to current E-UTRA system. Otherwise, power back-off will be needed.

Observation#3: DPD applicability for wide bandwidth need be studied for 24.25 – 86 GHz.

3.3 Impact on spurious emission

The spurious emission requirement mainly depends on the performance of RF filter. As millimetre wave devices have already been widely used for microwave system and 802.11ad, which approve that general spurious emission requirement (regulation) can be met. If additional spurious emission requirement is needed, RF filter can be design for that; however the IL will be increased correspondingly.

Observation#4: filter may be redesigned if additional spurious emission requirement is needed in 24.25 – 86 GHz.

3.4 Impact on noise figure

Table 3.4-1 Impact on noise figure

|  | LNA NF (dB) | RF switch IL (dB) | Circulator IL (dB) | RF filter IL (dB) |
| --- | --- | --- | --- | --- |
| 2GHz | 0.9 - 1.5 | 0.4 - 0.5 | 0.2 | 0.8 – 1.5 |
| 24.25-33.4 GHz | 2.2 - 2.8 | 2 | 0.25 - 0.4 | 0.8 - 1.5 |
| 37-43.5 GHz, | 3 - 4 | 2 | 0.5 | 1 - 1.5 |
| 45.5‑52.6 GHz | NA | NA | NA | 1 - 1.5 |
| 66-86 GHz | 4 - 5 | 3 | 0.7 | 1 - 1.5 |

The total noise figure mainly depends on LNA NF, RF filter IL, RF switch IL and/or circulator IL (depends on RF reference architecture). As shown in table 3.4-1, the LNA NF and RF switch IL contribute the most degradation, and the degradation is very sensitive to frequency bands. Additional about 3 – 6 dB NF will be introduced to 24.25 - 86 GHz devices.

Observation#5: NF increases by about 3 - 6 dB in 24.25 - 86 GHz.

3.5 Impact on ACS and in-band blocking

ACS and in-band blocking performance mainly depends on the SFDR of ADC. The ADC should support much higher sampling rate due to large channel bandwidth. Based on our survey, ADC with sampling rate larger than 1GHz and even 3GHz is available, but it is not cost-efficient and with poor performance of power consumption.

Observation#6: ACS and in-band blocking depends on the performance of higher sampling rate ADC.

3.6 Impact on out-of-band blocking

The performance of out-of-band blocking highly depends on the rejection of filter. Frequency separation between the channel edge and out-of-band blocking range shall be enlarged for wide channel bandwidth. Blocking interference level may also be further decreased as RAN4 already discussed for frequency above 2800MHz.

Observation#7: out-of-band blocking performance may degrade due to the high frequency and large bandwidth in 24.25 – 86 GHz.

- Considerations on promoting the study of NR RF parameters in 24.25 - 86 GHz

4.1 Potential Study procedure

Figure 4.1-1 overview of working procedure for NR RF requirements above in 24.25-86 GHz

In order to promote the study of NR RF parameters in 24.25-86GHz, we suggest that the group shall agree on a study procedure, and Figure 4.1-1 is an example.

For BS side, maximum output power (EIRP) and noise figure can be estimate as in section 3.1 and 3.4 base on current component performance. However, UE is more sensitive to cost and complexity, so poorer maximum output power and noise figure may be applicable to UE as long as the link budget can be satisfied.

Once maximum output power and noise figure is agreed, system simulation for co-existence could be done for ACIR and in-band blocking requirements. ACLR and ACS can be trade-off between BS and UE. Since no E-UTRA/UTRA/GSM system is no these frequency, only NR to NR co-existence is considered. RF requirements such as additional spurious emission, co-location spurious emission, out-of-band blocking and co-location blocking can be determined after the MCL study.

Observation#8: RAN4 should agree on a study procedure for RF parameter in 24.25 – 86 GHz, Figure 4.1-1 is an example.

4.2 Applicable scenario for 24.25 - 86GHz

In TS 38.913 [2], ten deployment scenarios for NR were approved. However, only four of them are applicable to 24.26-86GHz. We list these four scenarios in Table 4.2-1, and note that “around 30 GHz” includes frequency range from 24 – 40GHz, and “around 70 GHz” includes the range from 66 GHz – 86 GHz.

Table 4.2-1 overview of working procedure for NR RF requirements above in 24.25-86 GHz

| Scenarios | Carrier Frequency among 24.25-86GHz | Corresponding sub-frequency ranges (GHz) |
| --- | --- | --- |
| Indoor Hotspot | Around 30 GHz or around 70 GHz | 24.25-33.4, 37-43.5, 45.5-52.6, 66-86 |
| Dense Urban | Around 30 GHz | 24.25-33.4, 37-43.5 45.5-52.6 is FFS |
| Urban Macro | Around 30 GHz | 24.25-33.4, 37-43.5, 45.5-52.6 is FFS |
| High Speed | For BS to relay: around 30 GHz | 24.25-33.4, 37-43.5, 45.5-52.6 is FFS |
|  | For relay to UE: around 30 GHz or around 70 GHz | 24.25-33.4, 37-43.5, 45.5-52.6, 66-86 |

The note for “around 30 GHz” and “around 70GHz” only cover 3 out of 4 sub-frequency ranges in 24.25 - 86GHz. We believe that the scenario including “around 70GHz” should also be applicable to 45.5-52.6GHz, while whether the scenario only including “around 30GHz” is applicable to 45.5-52.6GHz is FFS.

Observation#9: in order to reduce the work load, RF parameter for NR in 24.25 – 86 GHz should be conducted only based on the agreed four scenarios in Table 4.2-1 as a baseline.

- Conclusion

Observation#1: the maximum output power for a single transmitter in 24.25 – 86 GHz is much lower than traditional one.

Observation#2: Range of spurious domain and bandwidth of adjacent channel need to be revisited for 24.25 – 86 GHz.

Observation#3: DPD applicability for wide bandwidth need be studied for 24.25 – 86 GHz.

Observation#4: filter may be redesigned if additional spurious emission requirement is needed in 24.25 – 86 GHz.

Observation#5: NF increases by about 3 - 6 dB in 24.25 - 86 GHz.

Observation#6: ACS and in-band blocking depends on the performance of higher sampling rate ADC.

Observation#7: out-of-band blocking performance may degrade due to the high frequency and large bandwidth in 24.25 – 86 GHz.

Observation#8: RAN4 should agree on a study procedure for RF parameter in 24.25 – 86 GHz, Figure 4.1-1 is an example.

Observation#9: in order to reduce the work load, RF parameter for NR in 24.25 – 86 GHz should be conducted only based on the agreed four scenarios in Table 4.2-1 as a baseline.

- Reference

- R4-162879, WF on RF parameters requested by WP 5D, NTT DOCOMO, INC., Qualcomm, CMCC, Nokia, Huawei, Ericsson, KT, Intel, Samsung

- TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies

- TR 38.900, hannel model for frequency spectrum above 6 GHz