3GPP TSG-RAN WG4 #79	R4-163926

Nanjing, China, May.23~May.27, 2016

Source: 	Ericsson

Title:	Discussion on LAA CSI test

Agenda Item:	6.10.4

Document for:	Discussion

# Introduction

In RAN4#78, the test purposes were agreed and how to setup CSI test is preliminary discussed. In RAN4#78bis, more details are agreed. In this paper, we further share our view on the CSI test case list and how to setup each CSI test.

# Test case list discussion

According to the [4], we have the following agreements:

- CQI feedback mode:

  - Aperiodic CQI test

  - Periodic CQI test is FFS

- One CSI test for CRS based transmission scheme and one CSI test for DMRS based transmission scheme for the corresponding CQI feedback mode

- Transmission mode

  - Baseline: TM3 (CRS based CSI measurement) and TM9 (CSI-RS based)

    - If any issues is identified for TM3, TM2 may be selected

- Test methodology

  - Two power levels with interference level constant

For CQI feedback mode, aperiodic CQI test had been agreed to be introduced. One issue for further study is about periodic CQI test. For periodic CQI, it is quite useful for broader scenarios, and especially efficient for small TXOPs. For aperiodic CQI, it is quite useful for large TXOPs and burst transmission. In order to employ the benefit of both periodic CQI and aperiodic CQI, we propose to introduce both aperiodic and periodic CSI test.

Proposal 1: Both aperiodic CSI and periodic CSI test will be introduced in LAA CSI test.

# LAA CSI test purpose

Based on the CSI reference resource thereof and the agreed test purposes in [3]:

- UE should not average CRS/CSI-RS measurements across transmission bursts.

- UE should find the right CSI reference resource in FS3

- Other purposes are not precluded

For periodic and aperiodic CSI, one main difference is the definition of the CSI reference resource. According, the test method may be different. In the following sections, periodic CSI test will be introduced in Section 4 and aperiodic CSI test will be introduce in Section 5.

# Test method for periodic CSI test

## Background for reference resource

Based on the above agreements, for LAA, the following new UE behaviors are defined (Note: it is based on RAN1 agreements in [1] ):

- UE should not average CRS/CSI-RS measurements across transmission bursts

  - It is up to UE implementation whether to average CRS/CSI-RS measurements across subframes or not

- A subframe is not valid subframe if it is not part of a transmission burst

- For a UE configured with TM9 or TM10, a subframe is not valid subframe if it doesn’t consist of CSI-RS configured for CSI measurement

- Initial partial subframe and end partial subframe are not valid subframe

- Outside a configured DMTC, a subframe where the CRS scrambling sequence doesn’t follow corresponding subframe index is not valid subframe

- Within a configured DMTC, a subframe where the CRS scrambling sequence doesn’t follow corresponding subframe index is not valid subframe

For behavior b), it is crucial for good LAA performance. The main characteristic of LAA is the uncertainty of availability of subframes due to LBT. UE needs implement new functionality to detect the presence of the subframe before uses the subframe for CSI measurements. Otherwise, the reported CSI may not indicate the actual channel condition. Thus, it is an essential functionality for unlicensed carrier where any transmission shall be subjected to LBT.

Further, the uncertainty for the availability of subframes due to LBT may lead to UE have to implement new function block to buffer CSI until the last subframe of the potential reference subframes. In license carrier, the CSI reference resource is pre-known in advance. More specifically, The subframe is defined as CSI reference resource in time domain when the subframe n-nCQI_ref is the latest valid subframe which satisfies nCQI_ref  X (X follows existing specifications).  Hence, UE can start to buffer CSI in the n-nCQI_ref  subframe in theory, as shown in Figure 1. However, for unlicensed carrier, the availability of each subframe is not pre-known for UE when the carrier is unlicensed carrier. Hence, UE need to start buffer CSI very earlier, as shown in Figure 2. In this example, a lot of subframes may be not available due to LBT.

Figure 1: One example for CSI buffering in licensed carrier

Figure 2: One example for CSI buffering in unlicensed carrier

For behavior c), e) and f), It is necessary for UE to have new functionality to detect the presence of valid reference signal in the potential reference resource. Due to the LBT uncertainty of the transmission subframes, the availability of the reference signal for CSI may be randomized. In order to get the right CSI report, the reference signal presence detection is a pre-condition for the accurate channel estimation.

Furthermore, UE needs to have functionality to check the validation of the reference signal. In LAA, there are two types of CRS. One type CRS is legacy CRS which is the same as CRS in license carrier, one type CRS is CRS which is part of DRS. For the first type CRS, the CRS scrambling sequence follows corresponding subframe index. For the second type CRS, the CRS scrambling sequence will be generated using subframe index 0 when transmitted in subframe 0~4, and using subframe index 5 when transmitted in subframe 5~9. Without this functionality, UE may get wrong channel estimation in a configured DMTC.

For these new UE expected behavior, it is better to use limit test cases to cover all of these purposes.

## CSI test for CRS-based transmission scheme in LAA

In order to verify these new UE expected behavior, the signal/channel transmission pattern for CRS-based transmission scheme and DMRS-based transmission scheme are proposed in Figure 3 and Figure 4, respectively.

During the transmission, two power levels are configured for the burst transmission, one is P_higher and one is P_lower. P_higher denotes higher power will be used for the burst transmission and P_lower denotes the lower power will be used for the burst transmission. In the figure, “red” block denotes the bursts with P_higher, and “green” block denotes the bursts with P_lower.

For the CRS report, periodic CSI report is configured. The periodicity of CSI report is 5 ms. In order to have the same CSI pattern for TDD Pcell and FDD Pcell, subframe 2 and 7 are used for the CSI reports. We can define the CSI report in subframe 7 as CSI set 0 (denoted as CSI_0) and define the CSI report in subframe 2 as CSI set 1 (denoted as CSI_1).

Two sets of subframes are defined, one set includes subframe 1, 2, 3 and the second set includes subframe 4, 5, 6, 7, 8, 9. For the first set of subframes, P_higher is applied. For the second set of subframes, P_Lower is applied. For the subframes belonging to subframe set 1, the transmission probability is 1. For the subframes belonging to subframe set 2, the transmission probability can have two options:

- Option 1: Assume all subframes belonging to the second set are in one burst, the burst transmission probability is 0.5

- Option 2: The transmission probability for each subframe is independent and the transmission probability is 0.1591

Figure 3: Signal/channel transmission pattern for CRS-based transmission scheme

In the pattern, DRS is configured and is transmitted in subframe 3. The periodicity of DRS is 40 ms. Assume the median CQI value corresponding to P_higher is CQI_higher, and the median CQI value corresponding to P_Lower is CQI_lower. In Table 3, the basic setup parameters are given.

Based on this setup, the median value of CQI report in CSI set 0 shall be CQI_higher. In the CSI set 1, in case there are at least one transmission in subframe 5, 6, 7, 8, the median value of CQI report is CQI_lower, and in case there are no any transmission in subframe 5, 6, 7, 8, the median value of CQI report shall be CQI_higher. In the CSI set 1, there are about 50% CQI value is about CQI_higher and another 50% CQI value is CQI_lower.

In the test, the following test metric can be used:

- Metric 1:  BLER for CSI_0 can be applied

- Metric 2: The Delta CQI between CQI_higher and CQI_lower

- Metric 3: The feedback shall be within CQI_lower +/-1 if there is at least one transmission in subframe 5, 6, 7 and 8, and within CQI_higher +/-1 if there is no any transmission in subframe 5, 6, 7 and 8 in CSI_1.

- Metric 4:  The percentage of CQI_higher shall be larger than X percent and the percentage of CQI_lower shall be larger than Y percent (X and Y could be 40% for the implementation margin) in CSI_1.

Table 1: Test setup for LAA CRS-based transmission scheme

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |  |
| PDSCH transmission mode |  |  | 2 |  |
| Downlink power allocation |  | dB | -3 |  |
|  |  | dB | -3 |  |
|  |  | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (2 x 2) |  |
| SNR (Note 2) |  | dB | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 3 |  |
| PUCCH Report Type |  |  | TBD |  |
| Reporting periodicity |  | ms | Npd = 5 |  |
| cqi-pmi-ConfigurationIndex |  |  | 4 |  |
| Transmission power for subframe 1,2,3 |  |  | P_higher |  |
| Transmission probability for subframe 1,2,3 |  |  | 1 |  |
| Transmission power for subframe 4,5,6,7,8,9 |  |  | P_lower |  |
| Transmission probability for subframe 4,5,6,7,8,9 |  |  | Option 1: Assume all subframes belonging to the second set are in one burst, the burst transmission probability is 0.5 Option 2: The transmission probability for each subframe is independent and the transmission probability is 0.1591 |  |
| Start symbol |  |  | 7 for subframe 4, and 0 for all other subframes (it is subjected to UE capability) |  |
| End symbol |  |  | 10 for subframe 0 and 14 for all other subframes (It is subjected to UE capability) |  |
| DRS periodicity |  | ms | 40 |  |
| DRS configuration |  |  | Transmit in subframe 3 |  |

With the above metric, if the UE averages CRS measurements across transmission burst, the feedback value in the CSI_0 and CSI_1 will be very similar. Thus, it is difficult to meet the BLER requirements given in metric 1. The delta value of the feedback in the two CSI sets will be close to zero. Therefore, it will fail metric 2.

With the above setup, if UE uses subframe which is not part of a transmission burst, it will fail metric 3 and metric 4 if the transmission probability for the subframes belonging to subframe set 2 follows option 2. In this case, in the CSI set 1, there will 84% ( It is derived from (1-0.1591) chance with very low CQI value. In case subframe 8 is not transmitted, the feedback will be not within CQI_lower +/- 1.

Further, with the above setup, it can even verify whether UE can have the right understanding for the initial partial subframe. If UE make mistake to identify the subframe 4 is initial partial subframe, the probability of CQI_lower may be higher than 50%. It is about 58%  (it is derived from 1-(1-0.1591)5).

Moreover, the above setup can verify whether UE have the capability to perform the detection of the presence of valid reference signal in the potential reference resource. Assume UE doesn’t differentiate the CRS scrambling with or without subframe index and always assume CRS is scrambled with subframe index, it will lead to very lower CQI feedback in subframe 7 of the first radio frame. Since the DRS periodicity is 40 ms and there are 4 CQI report for CSI_0 during 40 ms. If the UE fail to have this functionality, it will have 25% probability cannot meet metric 1.

Due to the uncertainty of the subframe 4, 5, 6, 7, 8 and 9, to get the right feedback, UE have to have the right buffer strategy. Hence, the proper buffer strategy can be verified.

Proposal 2: Taking into account the setup in Table 1 as one reference for the CSI test setup for CRS-based transmission scheme in the periodic CSI test

Proposal 3: The following metric can be considered for the periodic CRS-based CSI test:

- Metric 1:  BLER for CSI_0 can be applied

- Metric 2: The Delta CQI between CQI_higher and CQI_lower

- Metric 3: The feedback shall be within CQI_lower +/-1 if there is at least one transmission in subframe 5, 6, 7 and 8, and within CQI_higher +/-1 if there is no any transmission in subframe 5, 6, 7 and 8.

- Metric 4:  The percentage of CQI_higher shall be larger than X percent and the percentage of CQI_lower shall be larger than Y percent (X and Y could be 40% for the implementation margin)

## CSI test for DMRS-based transmission scheme

For DMRS-based transmission scheme, the signal/channel transmission pattern for DMRS-based transmission scheme is shown in Figure 6. The CSI-RS periodicity is 5ms and it is transmitted in subframe 3 and 8.

As the CRS-based transmission scheme, two sets of subframes are defined, one set includes subframe 1, 2 and 3, another set includes subframe 4, 5, 6, 7, 8 and 9. Two CSI report sets are defined, one CSI report set is collected from subframe 7 (denoted as CSI_0) and one CSI report set is collected from subframe 5 (denoted as CSI_1). In the first subframe set, P_higher power is applied and in the second subframe set, P_lower power is applied. In the figure, “red” block denotes the bursts with P_higher, and “green” block denotes the bursts with P_lower. For the subframes belong to subframe set 1, the transmission probability is 1. For the subframes belonging to subframe set 2, the transmission probability can have two options:

- Option 1: The transmission probability for each subframe is independent and the transmission probability is 0.5

- Option 2: Assume all subframes belonging to the second set are in one burst, the burst transmission probability is 0.5

In the pattern, DRS is configured and is transmitted in subframe 8. The periodicity of DRS is 40 ms. Assume the median CQI value corresponding to P_higher is CQI_higher, and the median CQI value corresponding to P_Lower is CQI_lower. The basic setup parameters are given in Table 4.

Based on this setup, the median value of CQI report in CSI set 0 shall be CQI_higher. In the CSI set 1, in case there is transmission in subframe 8 and the CSI-RS is scramble with the subframe index, the median value of CQI report is CQI_lower. Otherwise, the median value of CQI report shall be CQI_higher. Since the transmission probability of subframe 8 is 0.5, Hence, the CQI_lower probability shall be about 50%, and CQI_higher probability shall be about 50% in if DRS impact is not considered. If DRS impact is considered, since the subframe 8 in DMTC is not a valid reference subframe, hence, there are about 62.5% probability with CQI_higer and 37.5% probability with CQI_lower. Here, 62.5% is derived as 0.5*0.75 + 0.25 for CQI_higher, since in the first report at subframe 2 after the DMTC window, the CQI report shall be always CQI_higher and this event happen with probability 0.25 (once every four report). For other CQI_lower, the probability is about 37.5%.

Based on this statistics, in the test, the following test metric can be used:

- Metric 1:  BLER for CSI_0 can be applied

- Metric 2: The Delta CQI between CQI_higher and CQI_lower

- Metric 3: The feedback shall be within CQI_lower +/-1 if there is transmission in subframe 8, otherwise, the feedback shall be within CQI_higher +/-1 in CSI_1

- Metric 4:  The percentage of CQI_higher in CSI_1 shall be larger than X percent and the percentage of CQI_lower in CSI_1 shall be larger than Y percent (X could be 60% and Y could be 30% with implementation margin) in CSI_1

Similar like CRS-based transmission scheme, if the UE averages CRS measurements across transmission burst, the feedback value in the CSI_0 and CSI_1 will be very similar. Thus, it is difficult to meet the BLER requirements given in metric 1. The delta value of the feedback in the two CSI sets will be close to zero. As a result, it will fail metric 2.

Table 2: Test setup for LAA DMRS-based transmission scheme

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |  |
| PDSCH transmission mode |  |  | 2 |  |
| Downlink power allocation |  | dB | -3 |  |
|  |  | dB | -3 |  |
|  |  | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (2 x 2) |  |
| SNR (Note 2) |  | dB | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 3 |  |
| PUCCH Report Type |  |  | TBD |  |
| Reporting periodicity |  | ms | Npd = 5 |  |
| cqi-pmi-ConfigurationIndex |  |  | 4 |  |
| Transmission power for subframe 1,2,3 |  |  | P_higher |  |
| Transmission probability for subframe 1,2,3 |  |  | 1 |  |
| Transmission power for subframe 4,5,6,7,8,9 |  |  | P_lower |  |
| Transmission probability for subframe 4,5,6,7,8,9 |  |  | Option 1: The transmission probability for each subframe is independent and the transmission probability is 0.5 Option 2: Assume all subframes belonging to the second set are in one burst, the burst transmission probability is 0.5 |  |
| Start symbol |  |  | 7 for subframe 4, and 0 for all other subframes (It is subjected to UE capability) |  |
| End symbol |  |  | 10 for subframe 0 and 14 for all other subframes (It is subjected to UE capability) |  |
| DRS periodicity |  | ms | 40 |  |
| DRS configuration |  |  | Transmit in subframe 3 |  |
| CSI reference signals |  |  | Antenna ports 15,16 |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/3 |  |
| CSI reference signal configuration |  |  | 0 |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0100 0000 |  |

With the above setup, if UE have no capability to perform the detection of the presence of valid reference signal in the potential reference resource, it will fail to meet the Metric 3 and Metric 4.

Assume option 1 is used for the transmission probability for subframe 4, 5, 6, 7, 8 and 9. In these subframes, subframe 8 consists CSI-RS and other subframes does not consist CSI-RS. For the UE followed specification, a subframe is not a valid subframe if it doesn’t consist of CSI-RS configured for CSI-measurement. In the CSI_1, there will be 62.5% median CQI_value is CQI_higher, and there will be 37.5% median CQI_value is CQI_lower. Otherwise, the percentage of CQI_lower value in the CSI_1 is much higher than 50%, i.e., 93.5%. Obviously, these UE will violate the specification.

In case option 2 is used for the transmission probability for subframe 4, 5, 6, 7, 8 and 9, Regardless UE follows this specification or not, the percentage of CQI_lower value in CSI_1 is 37.5%. From this sense, option 1 is more preferable.

Figure 4: Signal/channel transmission pattern for DMRS-based transmission scheme

Proposal 4: Taking into account the setup in Table 2 as one reference for the CSI test setup for DMRS-based transmission scheme in the periodic CSI test

Proposal 5: The following metric can be considered for the periodic DMRS-based CSI test

- Metric 1:  BLER for CSI_0 can be applied

- Metric 2: The Delta CQI between CQI_higher and CQI_lower

- Metric 3: The feedback shall be within CQI_lower +/-1 if there is transmission in subframe 8, otherwise, the feedback shall be within CQI_higher +/-1 in CSI_1

- Metric 4:  The percentage of CQI_higher in CSI_1 shall be larger than X percent and the percentage of CQI_lower in CSI_1 shall be larger than Y percent (X could be 60% and Y could be 30% with implementation margin) in CSI_1

# Test method for aperiodic CSI test

## Background for aperiodic CSI reference resource

In additional to the valid subframe definition for periodic CSI definition, For aperiodic CSI test, the CSI reference resource is defined as:

In the time domain, for a UE configured in transmission mode 1-9 or transmission mode 10 with a single configured CSI process for the serving cell, the CSI reference resource is defined by a single downlink or special subframe n-nCQI_ref, where for aperiodic CSI reporting, if the UE is not configured with the higher layer parameter csi-SubframePatternConfig-r12, nCQI_ref is such that the reference resource is in the same valid downlink or valid special subframe as the corresponding CSI request in an uplink DCI format.

In the following sections, the CSI test for aperiodic is discussed.

## CSI test for CRS-based transmission scheme in LAA

The signal/channel transmission pattern for CRS-based transmission scheme and DMRS-based transmission scheme are proposed in Figure 5 and Figure 6, respectively.

Similar like the setup in periodic CSI test, during the transmission, two power levels are configured for the burst transmission, one is P_higher and one is P_lower. P_higher denotes higher power will be used for the burst transmission and P_lower denotes the lower power will be used for the burst transmission. In the figure, “red” block denotes the bursts with P_higher, and “green” block denotes the bursts with P_lower.

In the test, PUSCH 3-0 is configured. For the CSI request, one option is to transmit the CSI request in the predefined fixed subframes. For example, the CSI request is send in subframe 8 and 3 and its corresponding CSI report is performed in subframe 7 and subframe 2. We can define the CSI report in subframe 7 as CSI set 0 (denoted as CSI_0) and define the CSI report in subframe 2 as CSI set 1 (denoted as CSI_1). Another option is to transmit the CSI request in any randomly selected subframes.

Two sets of subframes are defined and used for burst transmission, one set includes subframe 1, 2, 3 and the second set includes subframe  5, 6, 7, 8, 9. As one option, for the first set of subframes, P_higher is applied. For the second set of subframes, P_Lower is applied. As another option, for each burst, the transmission power is randomly selected from {P_higher, P_lower} with equal probability for each burst. Both options are fine.

The transmission probability for each subframe is 0.5. In the pattern, DRS is configured and is transmitted in subframe 3. The periodicity of DRS is 40 ms. Assume the median wideband CQI value corresponding to P_higher is CQI_higher, and the median wideband CQI value corresponding to P_Lower is CQI_lower. In Table 3, the basic setup parameters are given.

Figure 5: Signal/channel transmission pattern for CRS-based transmission scheme

Based on this setup, the median value of wideband CQI report in CSI set 0 shall be CQI_higher. In the CSI set 1, the median value of wideband CQI report is CQI_lower.

In the test, the following test metric can be used:

- Metric 1:  BLER for CSI_0 and CSI_1 can be applied

- Metric 2: The Delta CQI between CQI_higher and CQI_lower

Table 3: Test setup for LAA CRS-based transmission scheme

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |  |
| PDSCH transmission mode |  |  | 3 |  |
| Downlink power allocation |  | dB | -3 |  |
|  |  | dB | -3 |  |
|  |  | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (2 x 2) |  |
| SNR (Note 2) |  | dB | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| PUSCH Report Type |  |  | PUSCH 3-0 |  |
| CSI request transmission |  |  | Option 1: CSI request is transmitted in the fixed subframes Option 2: CSI request is transmitted in any random selected subframes |  |
| Transmission power for subframe 1,2,3 |  |  | Option 1: P_higher Option 2: randomly selected from {P_higher, P_lower} with equal probability for each burst |  |
| Transmission power for subframe 5,6,7,8,9 |  |  | Option 1: P_lower Option 2: randomly selected from {P_higher, P_lower} with equal probability for each burst |  |
| Transmission probability for subframe 1, 2, 3 and 5,6,7,8,9 |  |  | 0.5 |  |
| Start symbol |  |  | 0 for all other subframes |  |
| End symbol |  |  | TDB |  |
| DRS periodicity |  | ms | 40 |  |
| DRS configuration |  |  | Transmit in subframe 3, it is subjected to the transmission probability. |  |

With the above metric, if the UE averages CRS measurements across transmission burst, the feedback value in the CSI_0 and CSI_1 will be very similar. Thus, it is difficult to meet the BLER requirements given in metric 1. The delta value of the feedback in the two CSI sets will be close to zero. Therefore, it will fail metric 2. With the above setup, if UE uses subframe which is not part of a transmission burst, it will fail metric 1.

Proposal 6: Taking into account the setup in Table 3 as one reference for the CSI test setup for CRS-based transmission scheme in the aperiodic CSI test.

Proposal 7: The following metric can be considered for the aperiodic CRS-based CSI test:

- Metric 1:  BLER for CSI_0 and CSI_1 can be applied

- Metric 2: The Delta CQI between CQI_higher and CQI_lower

## CSI test for DMRS-based transmission scheme

For DMRS-based transmission scheme, the signal/channel transmission pattern for DMRS-based transmission scheme is shown in Figure 6. The CSI-RS periodicity is 5ms and it is transmitted in subframe 3 and 8.

As the CRS-based transmission scheme, two sets of subframes are defined, one set includes subframe 1, 2 and 3, another set includes subframe 5, 6, 7, 8 and 9. Two CSI report sets are defined, one CSI report set is collected from subframe 7 (denoted as CSI_0) and one CSI report set is collected from subframe 2 (denoted as CSI_1).

For the power allocation for each subframe set, we can have two options. In the first option, in the first subframe set, P_higher power is applied and in the second subframe set, P_lower power is applied. In the figure, “red” block denotes the bursts with P_higher, and “green” block denotes the bursts with P_lower. In the second options, the power is randomly selected from {P_higher, P_lower} with equal probability for each transmission burst. Both options are fine.

The CSI request is send on subframe 8 and subframe 3. The transmission probability for each subframe is 0.5. In the pattern, DRS is configured and is transmitted in subframe 8. The periodicity of DRS is 40 ms. Assume the median CQI value corresponding to P_higher is CQI_higher, and the median CQI value corresponding to P_Lower is CQI_lower. The basic setup parameters are given in Table 4.

Based on this setup, the median value of CQI report in CSI set 0 shall be CQI_higher and the median value of CQI report in CSI set 1 shall be CQI_lower. In the test, the following test metric can be used:

- Metric 1:  BLER for CSI_0 and CSI_1 can be applied

- Metric 2: The Delta CQI between CQI_higher and CQI_lower

Similar like CRS-based transmission scheme, if the UE averages CRS measurements across transmission burst, the feedback value in the CSI_0 and CSI_1 will be very similar. Thus, it is difficult to meet the BLER requirements given in metric 1. The delta value of the feedback in the two CSI sets will be close to zero. As a result, it will fail metric 2.

Table 4: Test setup for LAA DMRS-based transmission scheme

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |  |
| PDSCH transmission mode |  |  | 3 |  |
| Downlink power allocation |  | dB | -3 |  |
|  |  | dB | -3 |  |
|  |  | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (2 x 2) |  |
| SNR (Note 2) |  | dB | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |
| CSI Report Type |  |  | PUSCH 3-0 |  |
| Transmission power for subframe 1,2,3 |  |  | Option 1: P_higher Option 2: randomly select the power from { P_higher, P_lower} with equal probability for each transmission burst |  |
| Transmission power for subframe 4,5,6,7,8,9 |  |  | Option 1: P_lower Option 2: randomly select the power from { P_higher, P_lower} with equal probability for each transmission burst |  |
| Transmission probability for subframe 1, 2, 3 and 5,6,7,8,9 |  |  | 0.5 |  |
| Start symbol |  |  | 0 for all other subframes |  |
| End symbol |  |  | TBD |  |
| DRS periodicity |  | ms | 40 |  |
| DRS configuration |  |  | Transmit in subframe 3 |  |
| CSI reference signals |  |  | Antenna ports 15,16 |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/3 |  |
| CSI reference signal configuration |  |  | 0 |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0100 0000 |  |

With the above setup, if UE fail to identify the right CSI reference resource, it will fail both metric 1 and metric 2.

Figure 6: Signal/channel transmission pattern for DMRS-based transmission scheme

Proposal 8: Taking into account the setup in Table 4 as one reference for the CSI test setup for DMRS-based transmission scheme in aperiodic CSI test.

Proposal 9: The following metric can be considered for the aperiodic DMRS-based CSI test

- Metric 1:  BLER for CSI_0 and CSI_1 can be applied

- Metric 2: The Delta CQI between CQI_higher and CQI_lower

# Conclusion

In this paper, we propose the CSI test method for aperiodic CSI test and the corresponding test metric. We have the following proposals:

Proposal 1: Both aperiodic CSI and periodic CSI test will be introduced in LAA CSI test.

Proposal 2: Taking into account the setup in Table 1 as one reference for the CSI test setup for CRS-based transmission scheme in the periodic CSI test

Proposal 3: The following metric can be considered for the periodic CRS-based CSI test:

- Metric 1:  BLER for CSI_0 can be applied

- Metric 2: The Delta CQI between CQI_higher and CQI_lower

- Metric 3: The feedback shall be within CQI_lower +/-1 if there is at least one transmission in subframe 5, 6, 7 and 8, and within CQI_higher +/-1 if there is no any transmission in subframe 5, 6, 7 and 8.

- Metric 4:  The percentage of CQI_higher shall be larger than X percent and the percentage of CQI_lower shall be larger than Y percent (X and Y could be 40% for the implementation margin)

Proposal 4: Taking into account the setup in Table 2 as one reference for the CSI test setup for DMRS-based transmission scheme in the periodic CSI test

Proposal 5: The following metric can be considered for the periodic DMRS-based CSI test

- Metric 1:  BLER for CSI_0 can be applied

- Metric 2: The Delta CQI between CQI_higher and CQI_lower

- Metric 3: The feedback shall be within CQI_lower +/-1 if there is transmission in subframe 8, otherwise, the feedback shall be within CQI_higher +/-1 in CSI_1

- Metric 4:  The percentage of CQI_higher in CSI_1 shall be larger than X percent and the percentage of CQI_lower in CSI_1 shall be larger than Y percent (X could be 60% and Y could be 30% with implementation margin) in CSI_1

Proposal 6: Taking into account the setup in Table 3 as one reference for the CSI test setup for CRS-based transmission scheme in the aperiodic CSI test.

Proposal 7: The following metric can be considered for the aperiodic CRS-based CSI test:

- Metric 1:  BLER for CSI_0 and CSI_1 can be applied

- Metric 2: The Delta CQI between CQI_higher and CQI_lower

Proposal 8: Taking into account the setup in Table 4 as one reference for the CSI test setup for DMRS-based transmission scheme in aperiodic CSI test.

Proposal 9: The following metric can be considered for the aperiodic DMRS-based CSI test

- Metric 1:  BLER for CSI_0 and CSI_1 can be applied

- Metric 2: The Delta CQI between CQI_higher and CQI_lower

We hope the group can consider these test setups in the CSI tests.

# Reference

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #83, 15th-22th Nov. 2015.

- R1-157893, Introduction of LAA (PHY layer aspects), Motorola Mobility, Nov. 2015

- R4-161182, Way forward on demodulation performance and CSI requirements for LAA, Huawei, HiSilicon, Feb. 2016.

- R4-162753, Way forward on demodulation performance and CSI requirements for LAA, Huawei, HiSilicon, Ericsson, Qualcomm, Intel, LG electronics, April 2016.