3GPP TSG-RAN WG4 Meeting #79	R4-164188

Nanjing, China, 23 – 27 May 2016

Agenda Item:	7.3.2

Source: 	Nokia, Verizon

Title: 	TP for TR 36.714-03-01: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination CA_13A-66A-66A

Document for:	Approval

# 1.	Introduction

The revised basket WI proposal including support of the LTE Advanced Carrier Aggregation (CA) of Band Combination CA_13A-66A-66A was approved in RAN#71 [1]. One objective of the WI is to specify the band-combinations specific Radio Frequency (RF) requirements for inter-band CA of this combination.

In this paper, we investigate the impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting CA of this band combination to the receiver of own or different BS, and provide a text proposal to record the findings in the 3DL/1UL TR 36.714-03-01 [2].

# 2.	Discussion

For LTE Advanced BS supporting inter-band CA, the transceivers of the two CA bands would typically share a common antenna system. When BS for two bands share the same antenna, operators should ensure the following:

- One band BS transmitter Out-Of-Band Emissions (OOBE) including harmonics and IMD falling into the other BS receive band do not cause BS receiver desensitization; and one band BS transmit carrier powers do not result in the other band BS receiver blocking.

- Third order Passive Intermodulations (PIM) generated by BS antenna path (including antennas, cables and connectors) and caused by multiple transmit band carriers do not fall into one of multiple BS receive bands and result in BS receiver desensitization.

## 2.1	Harmonics and IMD Products

Harmonics are produced by the BS oscillator or generated in non-linear circuit elements; while IMD products are caused by the mixing of multiple frequencies in non-linear circuit elements. Wireless systems producing transmit signals within multiple frequency bands must consider harmonics and IMD generation as BS receiver desensitization may occur when harmonics or IMD frequencies fall within the receive band of own or different BS. Usually the non-linear element is an amplifier or mixer, but RF connectors and BS antenna systems can also generate harmonics and IMDs due to imperfect physical contact or dissimilar metals used in their construction. Wireless BS are particularly sensitive to harmonics and IMD interference which fall upon uplink receiver channels that are in use by the BS. To limit the receiver desensitization to 0.4 dB, in-band harmonics and IMD interference must be limited to less than 10 dB below the receiver noise floor.

Harmonics and IMD are measured as the ratio in power levels between the distortion signal and the input signal in units of dBc. The magnitudes of the harmonics and IMD depend on the characteristics of the non-linear element, and the magnitude normally decreases as the order of the harmonics and IMD increases. For this reason, lower order harmonics and IMD products are considered the most damaging to wireless systems, while the higher order harmonics and IMD products are generally less damaging. Therefore, we will focus our investigation on the 2nd and 3rd order harmonics and IMD products caused by transmitting of Band 13 and Band 66 carriers in the BS.

## 2.2	BS Harmonics and IMD for CA

The LTE operating bands currently defined in the 3GPP standards are provided in Table 1 below [3]:

Table 1: LTE frequency bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
| 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
| 6 (NOTE 1) | 830 MHz | – | 840 MHz | 875 MHz | – | 885 MHz | FDD |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 9 | 1749.9 MHz | – | 1784.9 MHz | 1844.9 MHz | – | 1879.9 MHz | FDD |
| 10 | 1710 MHz | – | 1770 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
| 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
| 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
| 15 | Reserved |  |  | Reserved |  |  | FDD |
| 16 | Reserved |  |  | Reserved |  |  | FDD |
| 17 | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz | FDD |
| 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
| 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  |
| 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
| 22 | 3410 MHz | – | 3490 MHz | 3510 MHz | – | 3590 MHz | FDD |
| 23 | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | FDD |
| 24 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz | FDD |
| 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD (NOTE 2) |
| 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
| 31 | 452.5 MHz | – | 457.5 MHz | 462.5 MHz | – | 467.5 MHz | FDD |
| 32 | N/A |  |  | 1452 MHz | – | 1496 MHz | FDD (NOTE 2) |
| 33 | 1900 MHz | – | 1920 MHz | 1900 MHz | – | 1920 MHz | TDD |
| 34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
| 35 | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | TDD |
| 36 | 1930 MHz | – | 1990 MHz | 1930 MHz | – | 1990 MHz | TDD |
| 37 | 1910 MHz | – | 1930 MHz | 1910 MHz | – | 1930 MHz | TDD |
| 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
| 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |
| 44 | 703 MHz | – | 803 MHz | 703 MHz | – | 803 MHz | TDD |
| 45 | 1447 MHz | – | 1467 MHz | 1447 MHz | – | 1467 MHz | TDD |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD (NOTE 3, NOTE 4) |
| 65 | 1920 MHz | – | 2010 MHz | 2110 MHz | – | 2200 MHz | FDD |
| 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD (NOTE 5) |
| 67 | N/A |  |  | 738 MHz | – | 758 MHz | FDD (NOTE 2) |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | FDD |
| NOTE 1: Band 6 is not applicable. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3. NOTE 4: In this version of the specification, restricted to E-UTRA DL operation when carrier aggregation is configured. Band 46 is divided into four sub-bands as in Table 5.5-1A. NOTE 5: The range 2180 – 2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. |  |  |  |  |  |  |  |

It can be seen from Table 1 that Band 13 is operating at 777 – 787 MHz for UL and 746 – 756 MHz for DL, and Band 66 is operating at 1710 – 1780 MHz for UL and 2110 – 2200 MHz for DL. Therefore, the 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of CA_13A-66A-66A DL carriers can be calculated as shown in Table 2 below:

Table 2: CA_13A-66A-66A DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 746 | 756 | 2110 | 2200 | 2110 | 2200 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 656 |  |  | 846 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 656 |  |  | 846 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 3464 |  |  | 3654 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 4966 |  |  | 5156 |  |  |

It can be seen from Table 2 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA_13A-66A-66A may fall into the BS receive band of Bands 5, 6, 12, 13, 14, 17, 18, 19, 20, 22, 26, 27, 28, 42, 43, 44, 46 and 68.

Note that the 3rd order IMD products may fall into the BS own receive block within Band 13. Assume that the paired spectrum within Band 66 is used, and the 3 carriers transmitted by the CA_13A-66A-66A BS are a, b and c MHz in channel bandwidth and d, e and f MHz from the lower edges of Band 13 and Band 66 DL frequency bands as shown in Figure 1 below:

a MHz

b MHz

3rd CC

2nd CC

c MHz

1st CC

2110

746

d MHz

MHz

e MHz

f MHz

f1-low

f1-high

f2-low

f2-high

f3-low

f3-high

Figure 1: 3 carriers transmitted by the CA_13A-66A-66A BS using paired spectrum within Band 66

And the corresponding BS receive blocks are as shown in Figure 2 below:

a MHz

b MHz

3rd CC

2nd CC

c MHz

1st CC

1710

777

d MHz

MHz

e MHz

f MHz

f1-low

f1-high

f2-low

f2-high

f3-low

f3-high

Figure 2: 3 carriers received by the CA_13A-66A-66A BS using paired spectrum within Band 66

Then the 3rd order IMD products caused in the BS by transmitting of the 3 CC can be calculated as shown in Table 3 below:

Table 3: CA_13A-66A-66A BS DL 3rd order IMD products using paired spectrum within Band 66

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (f1-low + f2-low – f3-high) | 746 + d + e – f – c |
| (f1-high + f2-high – f3-low) | 746 + d + a + e + b – f |
| (f1-low + f3-low – f2-high) | 746 + d + f – e – b |
| (f1-high + f3-high – f2-low) | 746 + d + a + f + c – e |
| (f2-low + f3-low – f1-high) | 3474 + e + f – d – a |
| (f2-high + f3-high – f1-low) | 3474 + e + b + f + c – d |
| (f1-low + f2-low + f3-low) | 4966 + d + e + f |
| (f1-high + f2-high + f3-high) | 4966 + d + a + e + b + f + c |

Comparing the IMD frequency limits in Table 3 with the BS receive blocks in Figure 2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of the 3 CC will only fall into the BS own receive block of Band 13 under one of the transmit configurations shown in Table 4 below:

Table 4: CA_13A-66A-66A BS transmit configurations using paired spectrum within Band 66 with 3rd IMD within Band 13 BS own receive blocks

| 1st CC channel bandwidth (a MHz) | 2nd CC channel bandwidth (b MHz) | 3rd CC channel bandwidth (c MHz) | CC position (d, e and f MHz) |
| --- | --- | --- | --- |
| 5 | 5 | 5 | 21 < f – e < 41 |
| 5 | 5 | 10 | 16 < f – e < 41 |
| 5 | 10 | 5 | 21 < f – e < 46 |
| 10 | 5 | 5 | 16 < f – e < 46 |
| 5 | 5 | 15 | 11 < f – e < 41 |
| 5 | 10 | 10 | 16 < f – e < 46 |
| 5 | 15 | 5 | 21 < f – e < 51 |
| 10 | 5 | 10 | 11 < f – e < 46 |
| 10 | 10 | 5 | 16 < f – e < 51 |
| 5 | 5 | 20 | 6 < f – e < 41 |
| 5 | 10 | 15 | 11 < f – e < 46 |
| 5 | 15 | 10 | 16 < f – e < 51 |
| 5 | 20 | 5 | 21 < f – e < 56 |
| 10 | 5 | 15 | 6 < f – e < 46 |
| 10 | 10 | 10 | 11 < f – e < 51 |
| 10 | 15 | 5 | 16 < f – e < 56 |
| 5 | 10 | 20 | 10 < f – e < 46 |
| 5 | 15 | 15 | 15 < f – e < 51 |
| 5 | 20 | 10 | 20 < f – e < 56 |
| 10 | 5 | 20 | 5 < f – e < 46 |
| 10 | 10 | 15 | 10 < f – e < 51 |
| 10 | 15 | 10 | 15 < f – e < 56 |
| 10 | 20 | 5 | 20 < f – e < 61 |
| 5 | 15 | 20 | 15 < f – e <= 50 |
| 5 | 20 | 15 | 20 < f – e <= 55 |
| 10 | 10 | 20 | 10 < f – e <= 50 |
| 10 | 15 | 15 | 15 < f – e <= 55 |
| 10 | 20 | 10 | 20 < f – e <= 60 |
| 5 | 20 | 20 | 20 < f – e <= 50 |
| 10 | 15 | 20 | 15 < f – e <= 50 |
| 10 | 20 | 15 | 20 < f – e <= 55 |
| 10 | 20 | 20 | 20 < f – e <= 50 |

Next assume that both paired and unpaired spectra within Band 66 are used, and the 3 carriers transmitted by the CA_13A-66A-66A BS are a, b and c MHz in channel bandwidth and d, e and f MHz from the lower edges of Band 13 and  Band 66 (paired and unpaired) DL frequency bands as shown in Figure 3 below:

a MHz

b MHz

3rd CC

2nd CC

c MHz

1st CC

2110

746

d MHz

MHz

e MHz

f MHz

f1-low

f1-high

f2-low

f2-high

f3-low

f3-high

2180

Figure 3: 3 carriers transmitted by the CA_13A-66A-66A BS using both paired and unpaired spectra within Band 66

And the corresponding BS receive blocks are as shown in Figure 4 below:

a MHz

b MHz

2nd CC

1st CC

1710

777

d MHz

MHz

e MHz

f1-low

f1-high

f2-low

f2-high

Figure 4: 2 carriers received by the CA_13A-66A-66A BS using both paired and unpaired spectra within Band 66

Then the 3rd order IMD products caused in the BS by transmitting of the 3 CC can be calculated as shown in Table 5 below:

Table 5: CA_13A-66A-66A BS DL 3rd order IMD products using both paired and unpaired spectra within Band 66

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (f1-low + f2-low – f3-high) | 676 + d + e – f – c |
| (f1-high + f2-high – f3-low) | 676 + d + a + e + b – f |
| (f1-low + f3-low – f2-high) | 816 + d + f – e – b |
| (f1-high + f3-high – f2-low) | 816 + d + a + f + c – e |
| (f2-low + f3-low – f1-high) | 3544 + e + f – d – a |
| (f2-high + f3-high – f1-low) | 3544 + e + b + f + c – d |
| (f1-low + f2-low + f3-low) | 5036 + d + e + f |
| (f1-high + f2-high + f3-high) | 5036 + d + a + e + b + f + c |

Comparing the IMD frequency limits in Table 5 with the BS receive blocks in Figure 4 it can be deduced that the 3rd order IMD products caused in the BS by transmitting of the 3 CC will only fall into the BS own receive block of Band 13 under one of the transmit configurations shown in Table 6 below:

Table 6: CA_13A-66A-66A BS transmit configurations using both paired and unpaired spectra within Band 66 with 3rd IMD within Band 13 BS own receive blocks

| 1st CC channel bandwidth (a MHz) | 2nd CC channel bandwidth (b MHz) | 3rd CC channel bandwidth (c MHz) | CC position (d, e and f MHz) |
| --- | --- | --- | --- |
| 5 | 5 | 5 | 29 < e – f < 49 |
| 5 | 5 | 10 | 29 < e – f < 54 |
| 5 | 10 | 5 | 24 < e – f < 49 |
| 10 | 5 | 5 | 24 < e – f < 54 |
| 5 | 5 | 15 | 29 < e – f < 59 |
| 5 | 10 | 10 | 24 < e – f < 54 |
| 5 | 15 | 5 | 19 < e – f < 49 |
| 10 | 5 | 10 | 24 < e – f < 59 |
| 10 | 10 | 5 | 19 < e – f < 54 |
| 5 | 5 | 20 | 29 < e – f < 64 |
| 5 | 10 | 15 | 24 < e – f < 59 |
| 5 | 15 | 10 | 19 < e – f < 54 |
| 5 | 20 | 5 | 14 < e – f < 49 |
| 10 | 5 | 15 | 24 < e – f < 64 |
| 10 | 10 | 10 | 19 < e – f < 59 |
| 10 | 15 | 5 | 14 < e – f < 54 |
| 5 | 10 | 20 | 24 < e – f <= 60 |
| 5 | 15 | 15 | 19 < e – f <= 55 |
| 5 | 20 | 10 | 14 < e – f <= 50 |
| 10 | 5 | 20 | 24 < e – f <= 65 |
| 10 | 10 | 15 | 19 < e – f <= 60 |
| 10 | 15 | 10 | 14 < e – f <= 55 |
| 10 | 20 | 5 | 9 < e – f <= 50 |
| 5 | 15 | 20 | 19 < e – f <= 55 |
| 5 | 20 | 15 | 14 < e – f <= 50 |
| 10 | 10 | 20 | 19 < e – f <= 60 |
| 10 | 15 | 15 | 14 < e – f <= 55 |
| 10 | 20 | 10 | 9 < e – f <= 50 |
| 5 | 20 | 20 | 14 < e – f <= 50 |
| 10 | 15 | 20 | 14 < e – f <= 55 |
| 10 | 20 | 15 | 9 < e – f <= 50 |
| 10 | 20 | 20 | 9 < e – f <= 50 |

Next assume that the unpaired spectrum within Band 66 is used, and the 3 carriers transmitted by the CA_13A-66A-66A BS are a, b and c MHz in channel bandwidth and d, e and f MHz from the lower edges of Band 13 and unpaired Band 66 DL frequency bands as shown in Figure 5 below:

a MHz

b MHz

3rd CC

2nd CC

c MHz

1st CC

2180

746

d MHz

MHz

e MHz

f MHz

f1-low

f1-high

f2-low

f2-high

f3-low

f3-high

Figure 5: 3 carriers transmitted by the CA_13A-66A-66A BS using unpaired spectrum within Band 66

And the corresponding BS receive blocks are as shown in Figure 6 below:

a MHz

1st CC

777

d MHz

MHz

f1-low

f1-high

Figure 6: 1 carrier received by the CA_13A-66A-66A BS using unpaired spectrum within Band 66

Then the 3rd order IMD products caused in the BS by transmitting of the 3 CC can be calculated as shown in Table 7 below:

Table 7: CA_13A-66A-66A BS DL 3rd order IMD products using unpaired spectrum within Band 66

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (f1-low + f2-low – f3-high) | 746 + d + e – f – c |
| (f1-high + f2-high – f3-low) | 746 + d + a + e + b – f |
| (f1-low + f3-low – f2-high) | 746 + d + f – e – b |
| (f1-high + f3-high – f2-low) | 746 + d + a + f + c – e |
| (f2-low + f3-low – f1-high) | 3614 + e + f – d – a |
| (f2-high + f3-high – f1-low) | 3614 + e + b + f + c – d |
| (f1-low + f2-low + f3-low) | 5106 + d + e + f |
| (f1-high + f2-high + f3-high) | 5106 + d + a + e + b + f + c |

Comparing the IMD frequency limits in Table 7 with the BS receive blocks in Figure 6, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of the 3 CC will not fall into the BS own receive block of Band 13.

## 2.3	Possible Remedies for BS IMD

Note that Bands 6, 18, 19, 20, 22, 27, 28, 44 and 68 are not intended for use in the same geographical area as Bands 13 and 66. Therefore, the focus here is on the IMD falling into Bands 5, 12, 13, 14, 17, 26, 42, 43 and 46.

The possible remedies IMD caused by BS supporting CA_13A-66A-66A into the BS receive band of Bands 5, 12, 13, 14, 17, 26, 42, 43 and 46 are described below:

- Transmit Path Generated IMD: This type of IMD is generated in the oscillator or as signals pass through the amplifier stages of the BS transmit path. The BS transmit path will generate IMD which will appear at the BS equipment antenna connector (EAC) as spurious emissions. Spurious emissions appearing at the EAC and falling within the receive band of own or other BS will not be attenuated by the receive filter, and will pass into the receive path and desensitize the receiver. The 3rd IMD products of CA_13A-66A-66A DL carriers falling within the receive band of Bands 5, 12, 13, 14, 17, 26, 42, 43 and 46 can be mitigated by transmit filtering with not very sharp roll-off considering the sufficient frequency gap between Bands 13 and 66 DL and Bands 5, 12, 13, 14, 17, 26, 42, 43 and 46 UL. And if the BS can avoid the transmit configurations shown in Tables 4 and 6 above, then the 3rd IMD products will not fall into the own BS receive block of Band 13.

- Receiver Path Generated IMD: This type of IMD is generated when the BS transmit signals appearing at the EAC pass through the receive filter and are applied to the front end of the receiver chain. This type of interference can be mitigated through a combination of receive filter attenuation within the transmit band and high 2nd and 3rd order Input Intercept Point (IIP2 and IIP3) of the receive path. Since there is a sufficient frequency gap between Bands 13 and 66 DL and Bands 5, 12, 13, 14, 17, 26, 42, 43 and 46 UL, the Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 BS receive filter attenuation in the Bands 13and 66 BS transmit band should be substantial enough to limit the level of the transmit signals applied to the receiver front end. And the high IIP3 of the BS receive path would further reduce the potential interference to well below the Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 BS receiver noise floor. Also as discussed above, the 3rd IMD products will fall into the own BS receive block of Band 13 only if the BS use the transmit configurations shown in Tables 4 and 6.

- Antenna Passive Intermodulation: Even passive systems such as antennas, duplexers or filter combiners can generate IMD products via the non-linearities of dissimilar metals used in their construction, discontinuities in connectors or imperfections in the antenna system. This type of interference impacts all BS systems, but can be particularly troublesome when multiple BS operating in different frequency bands share a common antenna system. In the CA_13A-66A-66A case, the Band 13 DL signals would be combined with the Band 66 DL signals via a combiner and a diplexer and applied to a common antenna system, with the 3rd IMD products fall within Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 BS receive band, and desensitize the BS receiver; in this case Bands 13 and 66 BS transmitters should not share the same antenna with Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 BS receiver, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 BS receiver desensitization. The following alternatives can be used to avoid antenna sharing of Bands 13 and 66 BS transmitters with Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 BS receiver:

- Use separate antenna elements for the transmitter and receiver. This could be done with the higher frequency bands using state-of-the-art Adaptive Active Antenna (AAA). With the performance target of current antenna system and the extra coupling loss between the transmitter and receiver antenna elements, the PIM products should be low enough in level that they would not cause BS receiver desensitization.

- Use separate transmitter chains and antenna elements for transmitting the CA_13A-66A-66A carriers. This technique could also be used for load balancing between the multiple transmit antennas of a BS.

# 3.	Conclusions

In this paper, we have investigated the impact of harmonics and IMD products caused by LTE Advanced BS supporting CA_13A-66A-66A to the receiver of own or different BS. We have shown in addition to the frequency bands impacted by harmonics and IMD products from the constituent 2 DL CA configurations, the 3rd IMD products caused by BS supporting CA_13A-66A-66A may fall into the BS receive band of Bands 5, 12, 13, 14, 17, 26, 42, 43 or 46, and even the own BS receive block of Band 13 if certain BS transmit configurations are used, hence BS receiver desensitization may be an issue.

However, with the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 13 and 66 BS transmitters do not share the same antenna with Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 BS receiver, or the own Band 13 BS receiver if the aforementioned BS transmit configurations are used.

Therefore, in order to prevent BS receiver desensitization, it is recommended that Bands 13 and 66 BS transmitters should not share the same antenna with Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 BS receiver, or the own Band 13 BS receiver if the aforementioned BS transmit configurations are used, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 BS or the own Band 13 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

# 4.	Text Proposal

<New section>

#### 6.13.3.3 CA_13A-66A-66A

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of CA_13A-66A-66A DL carriers can be calculated as shown in Table 6.13.3.3-1 below:

Table 6.13.3.3-1: CA_13A-66A-66A DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 746 | 756 | 2110 | 2200 | 2110 | 2200 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 656 |  |  | 846 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 656 |  |  | 846 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 3464 |  |  | 3654 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 4966 |  |  | 5156 |  |  |

It can be seen from Table 6.13.3.3-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA_13A-66A-66A may fall into the BS receive band of Bands 5, 6, 12, 13, 14, 17, 18, 19, 20, 22, 26, 27, 28, 42, 43, 44, 46 and 68.

Note that the 3rd order IMD products may fall into the BS own receive block within Band 13. Assume that the paired spectrum within Band 66 is used, and the 3 carriers transmitted by the CA_13A-66A-66A BS are a, b and c MHz in channel bandwidth and d, e and f MHz from the lower edges of Band 13 and Band 66 DL frequency bands as shown in Figure 6.13.3.3-1 below:

a MHz

b MHz

3rd CC

2nd CC

c MHz

1st CC

2110

746

d MHz

MHz

e MHz

f MHz

f1-low

f1-high

f2-low

f2-high

f3-low

f3-high

Figure 6.13.3.3-1: 3 carriers transmitted by the CA_13A-66A-66A BS using paired spectrum within Band 66

And the corresponding BS receive blocks are as shown in Figure 6.13.3.3-2 below:

a MHz

b MHz

3rd CC

2nd CC

c MHz

1st CC

1710

777

d MHz

MHz

e MHz

f MHz

f1-low

f1-high

f2-low

f2-high

f3-low

f3-high

Figure 6.13.3.3-2: 3 carriers received by the CA_13A-66A-66A BS using paired spectrum within Band 66

Then the 3rd order IMD products caused in the BS by transmitting of the 3 CC can be calculated as shown in Table 6.13.3.3-2 below:

Table 6.13.3.3-2: CA_13A-66A-66A BS DL 3rd order IMD products using paired spectrum within Band 66

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (f1-low + f2-low – f3-high) | 746 + d + e – f – c |
| (f1-high + f2-high – f3-low) | 746 + d + a + e + b – f |
| (f1-low + f3-low – f2-high) | 746 + d + f – e – b |
| (f1-high + f3-high – f2-low) | 746 + d + a + f + c – e |
| (f2-low + f3-low – f1-high) | 3474 + e + f – d – a |
| (f2-high + f3-high – f1-low) | 3474 + e + b + f + c – d |
| (f1-low + f2-low + f3-low) | 4966 + d + e + f |
| (f1-high + f2-high + f3-high) | 4966 + d + a + e + b + f + c |

Comparing the IMD frequency limits in Table 6.13.3.3-2 with the BS receive blocks in Figure 6.13.3.3-2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of the 3 CC will only fall into the BS own receive block of Band 13 under one of the transmit configurations shown in Table 6.13.3.3-3 below:

Table 6.13.3.3-3: CA_13A-66A-66A BS transmit configurations using paired spectrum within Band 66 with 3rd IMD within Band 13 BS own receive blocks

| 1st CC channel bandwidth (a MHz) | 2nd CC channel bandwidth (b MHz) | 3rd CC channel bandwidth (c MHz) | CC position (d, e and f MHz) |
| --- | --- | --- | --- |
| 5 | 5 | 5 | 21 < f – e < 41 |
| 5 | 5 | 10 | 16 < f – e < 41 |
| 5 | 10 | 5 | 21 < f – e < 46 |
| 10 | 5 | 5 | 16 < f – e < 46 |
| 5 | 5 | 15 | 11 < f – e < 41 |
| 5 | 10 | 10 | 16 < f – e < 46 |
| 5 | 15 | 5 | 21 < f – e < 51 |
| 10 | 5 | 10 | 11 < f – e < 46 |
| 10 | 10 | 5 | 16 < f – e < 51 |
| 5 | 5 | 20 | 6 < f – e < 41 |
| 5 | 10 | 15 | 11 < f – e < 46 |
| 5 | 15 | 10 | 16 < f – e < 51 |
| 5 | 20 | 5 | 21 < f – e < 56 |
| 10 | 5 | 15 | 6 < f – e < 46 |
| 10 | 10 | 10 | 11 < f – e < 51 |
| 10 | 15 | 5 | 16 < f – e < 56 |
| 5 | 10 | 20 | 10 < f – e < 46 |
| 5 | 15 | 15 | 15 < f – e < 51 |
| 5 | 20 | 10 | 20 < f – e < 56 |
| 10 | 5 | 20 | 5 < f – e < 46 |
| 10 | 10 | 15 | 10 < f – e < 51 |
| 10 | 15 | 10 | 15 < f – e < 56 |
| 10 | 20 | 5 | 20 < f – e < 61 |
| 5 | 15 | 20 | 15 < f – e <= 50 |
| 5 | 20 | 15 | 20 < f – e <= 55 |
| 10 | 10 | 20 | 10 < f – e <= 50 |
| 10 | 15 | 15 | 15 < f – e <= 55 |
| 10 | 20 | 10 | 20 < f – e <= 60 |
| 5 | 20 | 20 | 20 < f – e <= 50 |
| 10 | 15 | 20 | 15 < f – e <= 50 |
| 10 | 20 | 15 | 20 < f – e <= 55 |
| 10 | 20 | 20 | 20 < f – e <= 50 |

Next assume that both paired and unpaired spectra within Band 66 are used, and the 3 carriers transmitted by the CA_13A-66A-66A BS are a, b and c MHz in channel bandwidth and d, e and f MHz from the lower edges of Band 13 and  Band 66 (paired and unpaired) DL frequency bands as shown in Figure 6.13.3.3-3 below:

a MHz

b MHz

3rd CC

2nd CC

c MHz

1st CC

2110

746

d MHz

MHz

e MHz

f MHz

f1-low

f1-high

f2-low

f2-high

f3-low

f3-high

2180

Figure 6.13.3.3-3: 3 carriers transmitted by the CA_13A-66A-66A BS using both paired and unpaired spectra within Band 66

And the corresponding BS receive blocks are as shown in Figure 6.13.3.3-4 below:

a MHz

b MHz

2nd CC

1st CC

1710

777

d MHz

MHz

e MHz

f1-low

f1-high

f2-low

f2-high

Figure 6.13.3.3-4: 2 carriers received by the CA_13A-66A-66A BS using both paired and unpaired spectra within Band 66

Then the 3rd order IMD products caused in the BS by transmitting of the 3 CC can be calculated as shown in Table 6.13.3.3-4 below:

Table 6.13.3.3-4: CA_13A-66A-66A BS DL 3rd order IMD products using both paired and unpaired spectra within Band 66

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (f1-low + f2-low – f3-high) | 676 + d + e – f – c |
| (f1-high + f2-high – f3-low) | 676 + d + a + e + b – f |
| (f1-low + f3-low – f2-high) | 816 + d + f – e – b |
| (f1-high + f3-high – f2-low) | 816 + d + a + f + c – e |
| (f2-low + f3-low – f1-high) | 3544 + e + f – d – a |
| (f2-high + f3-high – f1-low) | 3544 + e + b + f + c – d |
| (f1-low + f2-low + f3-low) | 5036 + d + e + f |
| (f1-high + f2-high + f3-high) | 5036 + d + a + e + b + f + c |

Comparing the IMD frequency limits in Table 6.13.3.3-4 with the BS receive blocks in Figure 6.13.3.3-4 it can be deduced that the 3rd order IMD products caused in the BS by transmitting of the 3 CC will only fall into the BS own receive block of Band 13 under one of the transmit configurations shown in Table 6.13.3.3-5 below:

Table 6.13.3.3-5: CA_13A-66A-66A BS transmit configurations using both paired and unpaired spectra within Band 66 with 3rd IMD within Band 13 BS own receive blocks

| 1st CC channel bandwidth (a MHz) | 2nd CC channel bandwidth (b MHz) | 3rd CC channel bandwidth (c MHz) | CC position (d, e and f MHz) |
| --- | --- | --- | --- |
| 5 | 5 | 5 | 29 < e – f < 49 |
| 5 | 5 | 10 | 29 < e – f < 54 |
| 5 | 10 | 5 | 24 < e – f < 49 |
| 10 | 5 | 5 | 24 < e – f < 54 |
| 5 | 5 | 15 | 29 < e – f < 59 |
| 5 | 10 | 10 | 24 < e – f < 54 |
| 5 | 15 | 5 | 19 < e – f < 49 |
| 10 | 5 | 10 | 24 < e – f < 59 |
| 10 | 10 | 5 | 19 < e – f < 54 |
| 5 | 5 | 20 | 29 < e – f < 64 |
| 5 | 10 | 15 | 24 < e – f < 59 |
| 5 | 15 | 10 | 19 < e – f < 54 |
| 5 | 20 | 5 | 14 < e – f < 49 |
| 10 | 5 | 15 | 24 < e – f < 64 |
| 10 | 10 | 10 | 19 < e – f < 59 |
| 10 | 15 | 5 | 14 < e – f < 54 |
| 5 | 10 | 20 | 24 < e – f <= 60 |
| 5 | 15 | 15 | 19 < e – f <= 55 |
| 5 | 20 | 10 | 14 < e – f <= 50 |
| 10 | 5 | 20 | 24 < e – f <= 65 |
| 10 | 10 | 15 | 19 < e – f <= 60 |
| 10 | 15 | 10 | 14 < e – f <= 55 |
| 10 | 20 | 5 | 9 < e – f <= 50 |
| 5 | 15 | 20 | 19 < e – f <= 55 |
| 5 | 20 | 15 | 14 < e – f <= 50 |
| 10 | 10 | 20 | 19 < e – f <= 60 |
| 10 | 15 | 15 | 14 < e – f <= 55 |
| 10 | 20 | 10 | 9 < e – f <= 50 |
| 5 | 20 | 20 | 14 < e – f <= 50 |
| 10 | 15 | 20 | 14 < e – f <= 55 |
| 10 | 20 | 15 | 9 < e – f <= 50 |
| 10 | 20 | 20 | 9 < e – f <= 50 |

Next assume that the unpaired spectrum within Band 66 is used, and the 3 carriers transmitted by the CA_13A-66A-66A BS are a, b and c MHz in channel bandwidth and d, e and f MHz from the lower edges of Band 13 and unpaired Band 66 DL frequency bands as shown in Figure 6.13.3.3-5 below:

a MHz

b MHz

3rd CC

2nd CC

c MHz

1st CC

2180

746

d MHz

MHz

e MHz

f MHz

f1-low

f1-high

f2-low

f2-high

f3-low

f3-high

Figure 6.13.3.3-5: 3 carriers transmitted by the CA_13A-66A-66A BS using unpaired spectrum within Band 66

And the corresponding BS receive blocks are as shown in Figure 6.13.3.3-6 below:

a MHz

1st CC

777

d MHz

MHz

f1-low

f1-high

Figure 6.13.3.3-6: 1 carrier received by the CA_13A-66A-66A BS using unpaired spectrum within Band 66

Then the 3rd order IMD products caused in the BS by transmitting of the 3 CC can be calculated as shown in Table 6.13.3.3-6 below:

Table 6.13.3.3-6: CA_13A-66A-66A BS DL 3rd order IMD products using unpaired spectrum within Band 66

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (f1-low + f2-low – f3-high) | 746 + d + e – f – c |
| (f1-high + f2-high – f3-low) | 746 + d + a + e + b – f |
| (f1-low + f3-low – f2-high) | 746 + d + f – e – b |
| (f1-high + f3-high – f2-low) | 746 + d + a + f + c – e |
| (f2-low + f3-low – f1-high) | 3614 + e + f – d – a |
| (f2-high + f3-high – f1-low) | 3614 + e + b + f + c – d |
| (f1-low + f2-low + f3-low) | 5106 + d + e + f |
| (f1-high + f2-high + f3-high) | 5106 + d + a + e + b + f + c |

Comparing the IMD frequency limits in Table 6.13.3.3-6 with the BS receive blocks in Figure 6.13.3.3-6, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of the 3 CC will not fall into the BS own receive block of Band 13.

Note that Bands 6, 18, 19, 20, 22, 27, 28, 44 and 68 are not intended for use in the same geographical area as Bands 13 and 66. Therefore, the focus here is on the IMD falling into Bands 5, 12, 13, 14, 17, 26, 42, 43 and 46.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 13 and 66 BS transmitters do not share the same antenna with Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 BS receiver, or the own Band 13 BS receiver if the aforementioned BS transmit configurations are used.

Therefore, in order to prevent BS receiver desensitization, it is recommended that Bands 13 and 66 BS transmitters should not share the same antenna with Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 BS receiver, or the own Band 13 BS receiver if the aforementioned BS transmit configurations are used, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 5, 12, 13, 14, 17, 26, 42, 43 or 46 BS or the own Band 13 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of section>

# References

[1]	RP-160582, “Revised WID: LTE Advanced inter-band CA Rel-14 for 3DL/1UL”, Huawei, HiSilicon.

[2]	R4-163743, “TR 36.714-03-01: 3DL/1UL inter-band CA R14 v0.2.0”, Huawei.

[3]	3GPP TS 36.104 v13.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.