3GPP TSG-RAN WG4 #79	R4-164225

Nanjing, China, 23th – 27th May 2016

Source: 	Ericsson

Title:  	Simulation assumptions and deployment scenarios for ITU-R WP5D

Agenda Item:	9.3

Document for:	Approval

# Introduction

During RAN#71, A SI to develop requirements and specifications for New Radio (NR) systems was approved [1]. In addition, TSG RAN has received an LS from ITU-R WP5D, requesting sharing parameters by February 2017 [1]  where [3] outlines a proposed work plan for ITU-R related work.

It would thus be essential to start the coexistence work very early in RAN4 for developing requirements for NR as well as handling the ITU-R related work.

In this contribution, we elaborate on some important and fundamental aspects related to deployment scenarios, spectrum considerations and some relevant parameters for coexistence studies related to NR system with respect to ITU-R WP5D request for sharing parameters [1].

# Coexistence simulations

The objectives for NR system SID, outlines quite many important RAN4 core and partly performance aspects including co-existence studies as highlighted below:

- Identify relevant RF parameters used to be used for sharing and co-existence studies

The objectives of the SI indicate extensive work in RAN4 not only capturing the aspects for the new RAT and but also the RF characteristics for bands up to 100 GHz. Covering the RF and testing aspects of beam-forming capable BS system is also an essential part of the work in RAN4.

## Spectrum consideration

As included in the LS from ITU-R WP5D [1], we can find that:

=== <<< From ITU-R WP5D LS [1]>>> ===

WP 5D is seeking the technical support and information relevant to the frequency range (24.25-86 GHz) being considered under AI 1.13:

i)	Utilizing the enclosed template in the Attachment, please provide to Working Party 5D the information on IMT-2020 system characteristics between 24.25 GHz and 86 GHz to be used in sharing and compatibility studies.

=== <<< From ITU-R WP5D LS [1]>>> ===

The above range is very wide. A number of spectrum ranges are available between 24.25GHz and 86GHz. It is not possible to perform simulations for all those spectrum ranges due to expected heavy workload, however we cannot also perform too few simulations, else we will lose these spectrum for NR systems. Regarding potential spectrum ranges that can be considered for ITU-R parameter related studies, we have explained the rationale for considering three different ranges  in our companion paper [3]. Thus, we propose to consider the following three spectrum ranges for the studies in RAN4 related to ITU-R sharing parameter requests, as listed below:

  - 30GHz

  - 45 GHz

  - 70GHz

Proposal: Consider 30GHz, 45GHz and 70GHz for RAN4 studies related to sharing parameters for ITU-R.

## Deployment scenarios

In this section, we describe our proposals deployment scenarios for both 30GHz and 70GHz.

### Deployment in 30GHz

For 30GHz, three deployment cases, i.e. indoor hotspots, urban micro and dense urban micro deployments, are available. A summary of these deployments are given below in Table 1

Table 1 Deployments in 30GHz spectrum

|  | Indoor hotspot | Urban macro | Dense urban Micro |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Micro | Manhattan |  |
| ISD | 20m | 500m | 200m (this is for corresponding macro layer) | Based on Manhattan (~50m) |  |
| Overlaid/underlaid | NA | NA | NA | NA |  |
| Grid shift | 0%, 50%,100% |  |  |  |  |
| Indoor/outdoor percentage of users | 100% indoor | 100% outdoor | [20% outdoor, 80% indoor] |  | 100% outdoor |
| Speed | 3kmph | 30kmph | 3kmph | 3kmph |  |
| BS Antenna height | 3m | 25m | 10m | 10m |  |

### Deployment in 45GHz

We only consider indoor hotspots and dense urban for 45GHz, as described in Table 2

Table 2 Deployments in 45GHz spectrum

|  | Indoor hotspot | Dense urban Micro |  |
| --- | --- | --- | --- |
|  |  | Micro | Manhattan |
| ISD | 20m | 200m (this is for corresponding macro layer) | Based on Manhattan (~50m) |
| Overlaid/underlaid | NA | NA | NA |
| Grid shift | 0%, 50%,100% |  |  |
| Indoor/outdoor percentage of users | 100% indoor | [80% indoor, 20% outdoor] | 100% outdoor |
| Speed | 3kmph | 3kmph | 3kmph |
| BS Antenna height | 3m | 10m | 10m |

### Deployment in 70GHz

We only consider indoor hotspots and dense urban for 70GHz, as described in Table 3

Table 3 Deployments in 70GHz spectrum

|  | Indoor hotspot | Dense urban Micro |  |
| --- | --- | --- | --- |
|  |  | Micro | Manhattan |
| ISD | 20m | 200m (this is for corresponding macro layer) | Based on Manhattan (~50m) |
| Overlaid/underlaid | NA | NA | NA |
| Grid shift | 0%, 50%,100% |  |  |
| Indoor/outdoor percentage of users | 100% indoor | [80% indoor, 20% outdoor] | 100% outdoor |
| Speed | 3kmph | 3kmph | 3kmph |
| BS Antenna height | 3m | 10m | 10m |

### Numbers of antennas at the BS and UE

Since beamforming will be an integral part of 5G systems, the number of antennas at the BS and UE will need to be defined before the evaluation works.

Table 4 Proposal on number of antennas at the UE and BS

|  | 30GHz | 45GHz | 70GHz |
| --- | --- | --- | --- |
| Antennas at the UE | [16] | [16] | [32] |
| Antennas at BS | [256] | [256] | [256] |

## Other parameter assumptions

Other relevant parameters are listed here. RAN1 has agreed on some parameters as in [4]. This list need to be updated based on new RAN1 input for the May meeting.

Table 5 other parameter assumptions

| Parameter | Indoor hotspot |  |  | Urban macro | Dense urban micro |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 30 GHz | 45GHz | 70GHz | 30MHz | 30GHz | 45GHz | 70GHz |  |
| System bandwidth | 200MHz | 200MHz | 200MHz | 200MHz | 200MHz | 200MHz | 200MHz |  |
| BS Tx power | TBD | TBD | TBD | TBD | TBD | TBD | TBD |  |
| UE TX power | TBD | TBD | TBD | TBD | TBD | TBD | TBD |  |
| BS antenna gain | 6dBi |  |  |  |  |  |  |  |
| UE antenna gain | Proposal: Follow the modeling of TR36.873 |  |  |  |  |  |  |  |
| BS noise figure | 8dB |  |  |  |  |  |  |  |
| UE Noise figure | [9dB] |  |  |  |  |  |  |  |
| UE receiver | MMSE-IRC as baseline |  |  |  |  |  |  |  |
| Traffic model | Consider full buffer and FTP model 1/2/3 with packet size 0.5 Mbytes (other value is not precluded). |  |  |  |  |  |  |  |

## RAN4 studies for ITU-R

Possible topics where RAN4 need to provide parameters for WP5D are listed in the LS.

=== <<< From ITU-R WP5D LS [1]>>> ===

TABLE 1

IMT-2020 technology related parameters in the frequency range 24.25-86 GHz

|  |  | IMT-2020 |  |
| --- | --- | --- | --- |
| No. | Parameter | Base station | Mobile station |
| 1 | Access technique |  |  |
| 2 | Modulation parameters |  |  |
| 3 | Channel spacing |  |  |
| 4 | Channel bandwidth (MHz) |  |  |
| 5 | Signal bandwidth (MHz) |  |  |
| 6 | Transmitter characteristics |  |  |
| 6.1 | Power dynamic range (dB) |  |  |
| 6.2 | Spectral mask |  |  |
| 6.3 | ACLR |  |  |
| 6.4 | Spurious emissions |  |  |

TABLE 1 (end)

|  |  | IMT-2020 |  |
| --- | --- | --- | --- |
| No. | Parameter | Base station | Mobile station |
| 7 | Receiver characteristics |  |  |
| 7.1 | Noise figure |  |  |
| 7.2 | Sensitivity |  |  |
| 7.3 | Blocking response |  |  |
| 7.4 | ACS |  |  |

=== <<< From ITU-R WP5D LS [1]>>> ===

Some of the parameters in the above table are depended on 3GPP works on other groups than RAN4 (e.g. access technique, Channel bandwidth, 	Signal bandwidth, etc are expected from RAN1). So, RAN4 coexistence studies should provide following parameters for different scenarios as mentioned in Table 1 and Table 3.

- Power dynamic range

- Transmitter ACLR parameter

- Receiver ACS parameters

## Timeline

The LS from ITU-R WP5D suggests two deadlines, as described below:

=== <<< From ITU-R WP5D LS [1]>>> ===

ii) WP 5D kindly asks for information as follows in order to meet the deadline: 
- Initial system characteristics by October 2016 meeting of WP 5D, additionally any views on the items included in Table 1 below 
- Final system characteristics by February 2017 meeting of WP 5D, specifically the final values to be included in Table 1.

=== <<< From ITU-R WP5D LS [1]>>> ===

Based on the above suggested timeline in the LS, we can organize the work in RAN4 is following ways:

| Output | WP5D meeting | RAN4 meetings | Suggested actions in RAN4 |
| --- | --- | --- | --- |
| Initial system characteristics | 4-12 October 2016 | RAN4#79: May 2016 RAN4#80: August 2016 | - Discuss deployment scenarios, initial system characteristics and agree on simulation parameters in RAN4#79: May and if needed also in RAN4#80: August meeting - Provide initial simulation results in RAN4#80: August 2016 meeting - Prepare the LS to ITU-R WP5D in RAN4#80: August meeting with initial system characteristics |
| Final system characteristics | Feb 2017 | RAN4#80bis:October 2016 RAN4#81: November 2016 | - Continue simulations and discussions regarding NR in 30GHz and 70GHz RAN4#80bis: October 2016 and RAN4#81: November 2016 meeting - Provide final simulation results in RAN4#81: November 2016 meeting - Prepare the 2nd LS to ITU-R WP5D in RAN4#81: November meeting with detailed system characteristics for WP5D’s considerations |

# Conclusion

In this paper, we listed the scenarios for NR SI. We have highlighted three spectrum ranges for which the coexistence studies can be performed. We have also listed some simulation parameters base on RAN1 input. Finally, we also proposed a time plan for coexistence simulations related to ITU-R response from RAN4.

Based on the discussions above, we propose the following:

Proposal-1: Consider 30GHz, 45GHz and 70GHz for RAN4 studies related to sharing parameters for ITU-R.

Proposal-2: Agree on the scenario and other system related parameters as described in Table 1 to Table 5 of Section 2 of this paper.

# References

ITU-R WP5D, “Working Party 5D LIAISON STATEMENT TO EXTERNAL ORGANIZATIONS: Characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz”, RP-160508, 3GPP TSG RAN #71, Göteborg, Sweden, March 7 - 10, 2016

R4-161728, “RF parameters for sharing studies in ITU-R WP5D”, Ericsson.

R4-164261, ” Frequency ranges for compatibility studies in ITU-R”, Ericsson

R1-163887,