3GPP TSG RAN WG4 Meeting #79		R4-164248

Nanjing, P.R. China 23 - 27 May 2016

Source: 	Ericsson

Title:	Radio measurements for congestion control for V2X

Agenda Item:	7.13.3

Document for:	Discussion

# Introduction

V2X communications is a type of communications in which a large number of communication entities with high demands for radio resources can concentrate on a limited geographical area, making the network congestion inevitable. As a result, a congestion control mechanism is needed to make the system work in such scenarios.

Additionally, (decentralized) congestion control is currently a mandatory part of the ITS technology in the EU [1] and one of the targets of the current SID/WID is to find solutions for the coexistence of LTE-based ITS with other ITS technologies. Hence, the congestion control should be studied and specified by 3GPP.

In the following sections we focus on measurements of the radio resources to support congestion control mechanisms.

# Measurements for congestion control

## Channel busy ratio (CBR)

Generally speaking, congestion control is a mechanism to adapt the transmit behaviors of a node in the network based on the level of occupancy of the communication medium, also known as congestion level or network load. As such, the congestion control algorithm requires certain measures of the channel occupancy level as input.

In case of decentralized congestion control (DCC) and centralized congestion control (CCC), each individual UE measures the channel occupancy level by itself and passes the measurements to the congestion control functionalities residing inside the UE. In any case the measurements by the individual UEs are crucial, and therefore need to be specified by RAN.

Observation 1: The channel occupancy level is crucial for the congestion control algorithms.

The most common way of obtaining channel occupancy level is by measuring the energy level in the radio resources over which the communications takes place. As a baseline, in [1][2] a measure called Channel Busy Ratio (CBR) has been defined and determined as follows:

- Np probes of the receive signal are taken uniformly distributed within the measuring interval Tm.

- For all channel probes (of lengthTp) the average signal level S is determined.

- CBR measured for the receive signal level threshold Sth is given as:

CBR(Sth)=∑ (1 ∀ probes with S>Sth)Np

In [1] the values for the above parameters were indicated as follows: Tp=8s, Tm= 100 ms, Sth= -85dBm, Np=12500. We observed that the CBR as defined above can be used in 3GPP as the channel occupancy level, provided that we have appropriate design of the method to determine S (sensing) and of parameters Np, Tm, Tp, and Sth.

Observation 2:	Because of standard harmonization reasons it is important that 3GPP defines a PC5 load measurement that is equivalent to CBR and that can be used instead of DSRC-based CBR in the DCC protocol. The CBR defined in [1] [2] can be considered as the channel occupancy level for CC in LTE-based ITS, under appropriate designs of the sensing method.

## Measurement of CBR

Note that the sensing method for determining CBR in [1][2] aligns with the TDMA nature of the ETSI ITS technology that is based on the IEEE 802.11 standard. In contrast, LTE is an OFDMA system, i.e., a UE’s transmission can use part of the system bandwidth. Another aspect of the LTE V2X is that a complete transmission consists of an SA transmission and a data transmission. Consequently, the sensing method in LTE needs to take these aspects into account.

Observation 3: 	LTE ITS has the following aspects that need to be taken into consideration when designing sensing method for CBR: 1) A transmission can use only part of the system bandwidth; 2) Separate SA and data transmissions

RAN1 #84bis has agreed on having separate SA and data (sub)pools and they are FDMed from system perspective. In order to provide meaningful information on the real system load the CBR measurement should consider only data resources. For similar reasons, SL-SS subframes should be excluded from the measurement.

According to one approach currently discussed in RAN1 [3], the measurements may be carried out as follows:

- The sensing for determining CBR for LTE ITS is performed over a measuring interval Tm=100ms, as follows:

  - The data frequency resources are divided into subbands

    - FFS the number of subbands, preferably aligned with PC5 scheduling granularity in frequency domain.

  - Each probe consists of measuring the average signal power S over a single data subband over a single subframe.

  - The probe is declared busy if S exceeds a threshold Sth

  - CBRSth= number of busy probes in Tmtotal number of probes in Tm.

In order to achieve a reliable CBR estimate, the parameter values have to be designed carefully:

- The number of samples taken in each probe should be large enough to have reliable statistics.

- The threshold Sth needs to be sufficiently larger than the effective noise power on the subband to avoid false alarm in declaring a busy probe.

Proposal 1:	RAN4 shall investigate CBR performance and determine suitable parameter value for Sth for the August meeting (RAN4#80).

# Conclusion

In this contribution we have discussed CBR measurement for V2X that would facilitate congestion control and coexistence of LTE-based ITS with other ITS technologies.

The following observations were made:

Observation 1: 	The channel occupancy level is crucial for the congestion control algorithms.

Observation 2:	Because of standard harmonization reasons it is important that 3GPP defines a PC5 load measurement that is equivalent to CBR and that can be used instead of DSRC-based CBR in the DCC protocol. The CBR defined in [1] [2] can be considered as the channel occupancy level for CC in LTE-based ITS, under appropriate designs of the sensing method.

Observation 3: 	LTE ITS has the following aspects that need to be taken into consideration when designing sensing method for CBR: 1) A transmission can use only part of the system bandwidth; 2) Separate SA and data transmissions

In order to progress on the work of coexistence of LTE-based ITS with other ITS technologies, the following proposed:

Proposal 1:	RAN4 shall investigate CBR performance and determine suitable parameter value for Sth for the August meeting (RAN4#80).

# Reference

- Draft ETSI EN 302 571 V2.0.0 (2016-03), Radiocommunications equipment operating in the 5 855 MHz to 5 925 MHz frequency band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU

- ETSI TS 102 687 (V1.1.1) (07-2011): "Intelligent Transport Systems (ITS); Decentralized Congestion Control Mechanisms for Intelligent Transport Systems operating in the 5 GHz range; Access layer part".

- R1-165268, Radio measurements for congestion control for V2X, Ericsson