3GPP TSG-WG4 Meeting #80	R4-165447

# Gothenburg SE, 22 - 26 Aug 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3734 | rev |  | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: NB-IoT Radio Link Monitoring Performance Test for Out-of-Sync in Normal Coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB-IoT_Perf |  |  |  |  |  |  | Date: |  | 2016-08-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Introduction of NB-IoT Radio Link Monitoring Performance Test for Out-of-Sync in Normal Coverage |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of NB-IoT Radio Link Monitoring Performance Test for Out-of-Sync in Normal Coverage |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test case for NB-IoT Radio Link Monitoring Performance Test for Out-of-Sync in Normal Coverage |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.x.y1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

### A.7.x.y1	NB-IoT HD-FDD Radio Link Monitoring Test for Out-of-sync in Enhanced Coverage

#### A.7.x.y1.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT HD-FDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.x.y1.1-1, A.7.x.y1.1-2 and A.7.x.y1.1-3 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.x.y1.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

At the start of the time duration T1 up to time point A, the UE should be provided with a UL grant. The UE is expected to decode the UL grant, and complete the UL transmission during T1. During the time period T2, the SNR shall be decreased in small steps from SNR1 to SNR2. During the period from time point B to time point D, the UE will be provided with UL grants repeatedly for [10] times to ensure UE decoded at least one NPDCCH successfully. The UE shall stop transmitting uplink signal no later than time point C and continues to keep its transmitter off until point D.

Table A.7.x.y1.1-1: General test parameters for NB-IoT HD-FDD out-of-sync testing for UE Category NB1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| NB-IoT RF Channel Number |  | 1 | One NB-IoT carrier frequency |
| NB-IoT Channel Bandwidth (BWchannel) | KHz | 180 |  |
| CP length |  | Enhanced |  |
| Deployment Mode |  | Standalone |  |
| NPDCCH transmission parameters Rmax |  | [8] | Other NPDCCH parameters for out-of-sync test are defined in Table 7.23.2-1 |
| DRX |  | OFF |  |
| Layer 3 filtering Note 2 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2 | ms | 0 | T310 is disabled |
| T311 timer Note 2 | ms | 100 | T311 is enabled |
| T1 | ms | [100] |  |
| T2 | ms | [10] |  |
| T3 | ms | [500] |  |
| SNR1 Note 3 | dB | [-4.7] |  |
| SNR2 Note 3 | dB | [-9.5] |  |
| SNR3 Note 3 | dB | [-3.5] |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331. Note 3: SNR levels in Figure A.7.x.y1.1-1 correspond to the signal to noise ratio over the narrowband reference signal REs. |  |  |  |

Table A.7.x.y1.1-2: Cell specific test parameters for NB-IoT HD-FDD (cell # 1) for out-of-sync radio link monitoring for UE Category NB1

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| OCNG Pattern as defined in A.3.2.3.1 Note 1 |  | NOP.1 FDD |  |  |  |  |
| NPDSCH parameters as defined in A.3.1.5.1 Note 2 |  | R.14 HD-FDD |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.6.1 Note 2 |  | R.26 HD-FDD |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |
| OCNG_RA Note 1 | dB |  |  |  |  |  |
| OCNG_RB Note 1 | dB |  |  |  |  |  |
|  | dBm/15 KHz | -98 |  |  |  |  |
| Propagation Condition | - | ETU 1Hz |  |  |  |  |
| Antenna Configuration | - | 1x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, NRSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |  |  |  |

Figure A.7.x.y1.1-1: SNR variation for out-of-sync testing

#### A.7.x.y1.2	Test Requirements

The UE behaviors in each test shall be as follows:

- At the start of the time duration T1 up to time point A, the UE should be provided with a UL grant. The UE is expected to decode the UL grant, and complete the UL transmission during T1.

- During the time period T2, the SNR shall be decreased in small steps from SNR1 to SNR2.

- During the period from time point B to time point D, the UE will be provided with UL grants repeatedly for [10] times to ensure UE decoded at least one NPDCCH successfully.

- The UE shall stop transmitting uplink signal no later than time point C and continues to keep its transmitter off until point D.

A correct event is defined as UE behaviors correctly in each of above steps. The correct events observed during repeated tests shall be at least 90%.

--- END OF CHANGE ---